3GPP TSG-RAN WG2 Meeting #120	R2-2212043

Toulouse, France, November, 2022

Agenda item:		7.2.3

Source:		Lenovo

Title:		Inclusion of neighbour cell ephemeris in system information

Document for:		Discussion

- Introduction

In this contribution we discuss on the provisioning of neighbour cell ephemeris information, especially in a broadcasting manner.

- Discussion

In the latest RAN2#119bis meeting there was a discussion on whether to support neighbour cell ephemeris information broadcasting for Rel-17 IoT NTN. Supporting companies argued that the information is necessary as that for NR NTN in neighbour cell measurement and thus in mobility management, while the opposing companies considered it as an optimization and thus would like to have this in Rel-18 since Rel-17 has frozen. During the discussion possible RRM requirements from RAN4 was also mentioned as a possible reason to provision neighbour cell ephemeris information.

For this meeting, an LS (R2-2211171_R4-2217265) has been received from RAN4, indicating that:

In LTE_NBIOT_eMTC_NTN_req WI, RAN4 is specifying RRM requirements for NB-IoT and eMTC operation over NTN functionality defined in the Rel-17 LTE_NBIOT_eMTC_NTN work item.

And also the following RAN4 agreement was indicated:

Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

Based on the information provided by RAN4, at least we can confirm that neighbour cell ephemeris information is necessary from RAN4 perspective, and the issue is whether we introduce this in Rel-17 as correction or leave it to Rel-18 IoT NTN.

Observation 1: Neighbour cell ephemeris information to fulfill RAN4 RRM requirements for IoT NTN.

Considering that this is the last meeting for Rel-17 IoT NTN, RAN2 needs to make decision on this issue.

Proposal 1: RAN2 to decide whether to include neighbour cell ephemeris in system information for Rel-17 IoT NTN.

If RAN2 decides to include neighbour cell ephemeris in system information, it is necessary to have it in a backward-compatible way. Referring to the handling of neighbour cell ephemeris in system information for NR NTN, the neighbour cell ephemeris is included in SIB19 as part of ntn-NeighCellConfig in ntn-NeighCellConfigList reusing the same element ntn-Config as the serving cell ephemeris, and further extension (e.g., to include ephemeris for more neighbour cells) is also allowed as ntn-NeighCellConfigListExt in a backward-compatible way.

For IoT NTN, the serving cell ephemeris is currently included as part of ServingSatelliteInfo in SIB31, which is similar to ntn-Config in SIB19 for NR NTN. Therefore including neighbour cell ephemeris in SIB31 can be easily implemented by reusing the elements of ServingSatelliteInfo, as a lateNonCriticalExtension for SIB31.

Observation 2: Including neighbour cell ephemeris in SIB31 can be implemented in a backward-compatible way.

Proposal 2: If RAN2 agree to include neighbour cell ephemeris in system information for Rel-17 IoT NTN, it can be implemented by an extension IE in SIB31 using the same elements of ServingSatelliteInfo.

- Conclusion

In this contribution, we discuss the provisioning of neighbour cell ephemeris in system information for IoT NTN. The following observation are given:

Observation 1: Neighbour cell ephemeris information to fulfill RAN4 RRM requirements for IoT NTN.

Observation 2: Including neighbour cell ephemeris in SIB31 can be implemented in a backward-compatible way.

And we propose:

Proposal 1: RAN2 to decide whether to include neighbour cell ephemeris in system information for Rel-17 IoT NTN.

Proposal 2: If RAN2 agree to include neighbour cell ephemeris in system information for Rel-17 IoT NTN, it can be implemented by an extension IE in SIB31 using the same elements of ServingSatelliteInfo.