3GPP TSG-RAN WG2 Meeting #120                             	R2-2212059

Toulouse, France, November 14 –18, 2022

Agenda item:	8.3.3

Source:	Samsung

Title:	Discussion on SSB/SIB-less Solutions for NES

Document for:	Discussion & Decision

# Introduction

In RAN2#119bis-e meeting, RAN2 made the following agreements:

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

Also, RAN2 email discussion [POST119bis-e][304] provided an open issue list [1]. This document discusses those open issues.

# Discussion

According to the latest TP [2], SSB-less and SIB-less solutions in multi-carrier scenarios are being discussed for NES cell to save its energy by not transmitting SIB or both SSB and SIB. In any case, those NES solutions aim to maximize the NES gain by a simple solution. It should be the direction of this study.

Observation 1. NES solution should aim to maximize the NES gain by a simple mechanism.

In both SSB-less and SIB-less solutions discussed in RAN2, UE has to anyway receive SSB and SIB on the anchor cell. From the UE point of view, there is no big difference from camping on the anchor cell. Then, UE does not need to communicate with two different cells. It just increases UE power consumption and complexity. The simplest way is to keep UE camping on the anchor cell and let the gNB redirect the UE to the NES cell when it is needed.

Proposal 1. UE camps on the anchor cell, not SSB-less/SIB-less cell

NES gain will be maximized when the gNB has a deep sleep by switching off all possibility TX/RX functionalities. If the NES cell does not transmit SSB and/or SIB, we need to consider not transmitting paging together with not transmitting SSB/SIB. The SSB-less and SIB-less solutions assume multi-carrier scenarios, there is no strong motivation that both carriers transmit the duplicate paging messages.

Observation 2. NES gain will be maximized when the NES cell does not transmit paging together with not transmitting SSB/SIB.

Proposal 2. UE monitors paging on the anchor cell.

Another source of network energy consumption would be uplink monitoring especially for UE-triggered transmission using reserved resource, for instance, PRACH monitoring. In CA scenario, it is beneficial for UE to transmit PRACH preamble on the anchor cell. If the gNB needs to redirect the UE to the NES cell due to e.g. overload of the anchor cell, gNB may re-direct the UE to an NES cell or non-anchor cell during the random access procedure. In case of random access due to paging, paging may re-direct the UE to perform the RA procedure on an NES cell or non-anchor cell.

Observation 3. NES gain will be maximized when the NES cell does not always monitor PRACH.

Proposal 3. UE in RRC_INACTIVE/IDLE initiates the random access procedure on the anchor cell.

Proposal 4. gNB may re-direct UE to an NES or non-anchor cell during the random access procedure.

Proposal 5. Paging may redirect UE to perform the random access procedure on an NES or non-anchor cell.

RAN2 is considering both SSB/SIB-less solution and SIB-less solution. The main difference is that in SSB-less solution, synchronization on the NES cell relies on the anchor cell. The other aspects would be the same, and we see no reason to have different details. In short, a common mechanism can be applied.

Proposal 6. Common mechanism is applied for both SSB/SIB-less and SIB-less solutions.

# Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. UE camps on the anchor cell, not SSB-less/SIB-less cell

Proposal 2. UE monitors paging on the anchor cell.

Proposal 3. UE in RRC_INACTIVE/IDLE initiates the random access procedure on the anchor cell.

Proposal 4. gNB may re-direct UE to an NES or non-anchor cell during the random access procedure.

Proposal 5. Paging may redirect UE to perform the random access procedure on an NES or non-anchor cell.

Proposal 6. Common mechanism is applied for both SSB/SIB-less and SIB-less solutions.

# Reference

[1] R2-2211428, Report of [POST119bis][304][NES] TP on cell selection/reselection and SSB/SIB-less (Huawei), Huawei, HiSilicon

[2] R2-2211427, TP on cell selection / reselection and SSB / SIB-less, Huawei, HiSilicon