CR3GPP TSG-RAN WG2 Meeting #120	R2-2212086

# France, 14th Nov – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On including SSB and CSI-RS measurements in RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2022-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current procedural text for capturing the neighbor cell measurements in the RLF report is as follows: 1> for each of the configured measObjectNR in which measurements are available: 2> if the SS/PBCH block-based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 4> for each neighbour cell included, include the optional fields that are available; 2> if the CSI-RS based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure; 4> for each neighbour cell included, include the optional fields that are available; Based on this procedure, the UE initially checks if the SSB based measurement results are available for a given measObject and if yes, then the UE includes the SSB based measurements ordered such that cell with highest SSB RSRP are listed first, if SSB RSRP is available, else cell with highest SSB RSRQ are listed first, if SSB RSRQ is available else cell with highest SSB SINR are listed first. Then the UE checks if the CSI-RS based measurement results are available for the same measObject and if yes, then the UE includes the CSI-RS based measurements ordered such that cell with highest CSI-RS RSRP are listed first, if CSI-RS RSRP is available, else cell with highest CSI-RS RSRQ are listed first, if CSI-RS RSRQ is available else cell with highest CSI-RS SINR are listed first. This procedural text is not correct as the UE can include only one list of neighbor cell PCIs. Based on the procedural text, the UE initially includes the list of PCIs ordered based on SSB based measurements and then the UE should include the list of PCIs ordered based CSI-RS based measurements. Consider the example below. - UE initially sets and orders measResultListNR in measResultNeighCells PCI-1,2,3,4,5,6,7,8 in the order of best measured cell based on SSB measurements. - UE then sets and orders measResultListNR in measResultNeighCells PCI-7,8,9,10,11,12,13,14 in the order of best measured cell based on CSI-RS measurements. By doing so, the measResultListNR in measResultNeighCells includes PCI-7,8,9,10,11,12,13,14 related measurements wherein only PCI-7 and 8 have both SSB based and CSI-RS based measurements and PCI-9,10,11,12,13,14 will have only CSI-RS based measurements. Thus PCI-1,2,3,4,5,6 related SSB based measurements are lost |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedural text is updated such that the UE creates the measResultListNR in measResultNeighCells based on the SSB measurements, if SSB based measurements are available. Then, the UE includes any CSI-RS based measurements for the respective PCIs included in the measResultListNR in measResultNeighCells If the UE does not have any SSB measurements available for a measObject, then the UE creates the measResultListNR in measResultNeighCells based on the CSI-RS measurements, if CSI-RS based measurements are available. Impact analysis Impacted functionality: RLF report Inter-operability: No inter-operability issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measResultListNR in measResultNeighCells is not correctly set by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	if the CSI-RS based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell(in case HO failure) or PCell (in case RLF), if available;

1>	for each of the configured measObjectNR in which measurements are available:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

4>	for each neighbour cell included, include the optional fields that are available (i.e. include the CSI-RS based measurement quantities, if available);

2>	if the CSI-RS based measurement quantities are available for the cells not yet included in measResultListNR in measResultNeighCells:

3>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure;

4>	for each neighbour cell included, include the optional fields that are available;

1>	for each of the configured EUTRA frequencies in which measurements are available;

2>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

END OF CHANGES