3GPP TSG-RAN WG2 #120-Toulouse	Tdoc R2-2212091

France, 14th – 18th November 2022

Agenda Item:	8.13.5

Source:	Ericsson

Title:	Enhancements of SON reports for NR-U

Document for:	Discussion, Decision

# 1	Introduction

The new Rel.18 WID on further enhancement of data collection for SON/MDT includes the support of NR-U in the SON/MDT framework, as one of the objectives to achieve [1].

In this paper, we discuss required enhancements that can be introduced in the SON/MDT framework to support the NR-U system in particular enhancements needed for RA-Report, RLF report and the SHR.

# 2	Discussion

In RAN2#119 it has been agreed to prioritize enhancement of the RA report and RLF report (for radio link failure and handover failure) including the RA-InformationCommon, as well as SHR.

Agreement:

1	RAN2 to prioritize (at least in the beginning of the discussion) the following scenarios for potential enhancement on existing SON signaling reports, e.g. the RA-Report/RA-Information, the RLF-Report (for RLF and HOF), the SHR.

Furthermore, in RAN2#119bis-e meeting some progress was made-

Agreements:

1	The UE will log information of multiple RA procedures related to consistent LBT failures. FFS details.

2	Introduce a new raPurpose in the RA-Report to indicate that the RA was initiated following a “consistent LBT failures” in the SpCell.

3	RAN2 agree to log kind of “the number of LBT failures” in the RA report.

LBT failure is the failure to access the channel before transmission.

The definition of “the number of LBT failures” should be clarified.

FFS: how to log the number of LBT failures in the RA report.

FFS: how to fulfil RAN3 request in logging RSSI.

In the following, we further discuss the possible enhancements in the RA-Report, RLF report as well as SHR.

### 2.1 Enhancements to the RA-Report and RA-Information

As agreed, UE will log information of the multiple RA procedures related to consistent LBT failures. From TS 38.331 we note that in the RA report (partially copied below for reference), the frequency information of the RA resources  are available in the RA-InformationCommon IE.

RA-ReportList-r16 ::= SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16

RA-Report-r16 ::=                    SEQUENCE {

cellId-r16                           CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

ra-InformationCommon-r16             RA-InformationCommon-r16                         OPTIONAL,

raPurpose-r16                        ENUMERATED {accessRelated, beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized,

schedulingRequestFailure, noPUCCHResourceAvailable, requestForOtherSI,

msg3RequestForOtherSI-r17, spare8, spare7, spare6, spare5, spare4, spare3,

spare2, spare1},

...,

[[

spCellID-r17                         CGI-Info-Logging-r16                             OPTIONAL

]]

}

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...,

}

Since, the multiple RA procedures (performed as part of LBT recovery) may belong to multiple BWPs, we propose to enhance RA report with a list of RA-InformationCommon IEs; each of which will contain information about the RA attempts at a different BWP. Network can thus identify the potential problem at BWP granularity.

A List of RA-InformationCommon is used to capture information about the multiple RA procedures performed by the UE at different BWPs.

NR-U information per RA attempt:

It was agreed to log kind of ‘the number of LBT failures’ will be logged in the RA report. In the following we provide different options for this and discuss their pros and cons.

Number of LBT failures can be logged per BWP/per RA procedure. However, that doesn’t provide any useful information to the network since network is unaware of the beams that UE tried to use for RA.

Logging number of LBT failures per BWP is very coarse information for network to perform any optimization.

It would be useful for the network to know whether the single RA attempt (i.e., preamble transmission attempt) was blocked by LBT or not. That is needed because from MAC perspective even if an RA attempt is blocked by LBT, anyhow that is counted as RACH transmission, and the corresponding RACH attempt count is stepped in MAC in the case that LBT recovery config is not configured.

However, some companies argue that this solution is complex for the UEs, so as a compromise we propose the UE logs the number of LBT failure per selected beam. This solution enables the network to identify problems at the beam level and perform optimizations along with minimal overhead on the UE. Equipped with this information, network can analyze the number of LBT failure per selected beam and figure out which beams performs better for the UE (which beams are more frequently selected by the UE and which beams where successful when LBT issue was resolved).

In the below table, a possible scenario with SSB as an example is analyzed. UE selects beam #01 but is not able to perform RA attempt due to LBT failure (i.e., UE fails to access the channel). UE selects beam #02 and performs few RA attempts; although there are no LBT issues, the RA remains unsuccessful pointing to the beam being unsuitable. However, the UE succeeds in first attempt with beam #01 when the LBT issue is not detected. In such scenario, network can take optimization decision of the beams.

| # Beam | # RA attempt (with successful channel access) | # LBT (channel access) failure |
| --- | --- | --- |
| 1 | 0 | 20 |
| 2 | 5 | 0 |
| 1 | 1 | 0 |

UE logs number of LBT failures per selected beam in the RA report.

In addition, the information of the RSSI and the EDT used by the UE at the time of LBT issue when executing a random access procedure would be highly beneficial to pinpoint whether the random-access related issue was due to a bad uplink coverage, interference or the configuration used by the UE at the time of random-access procedure. Needless to mention that each of the above issues e.g., bad coverage, interference in shared spectrum, or the configuration used by the UE requires a course of different actions by the network to enhance the random access performance..

In addition, when it comes the type of the RSSI measurements we think UE can report an average value of the measured RSSI samples that let to LBT failure and the average value of the measured RSSI samples that let to successful channel access. Such measurements enable the RAN node to understand the dynamic of the channel when it is occupied and not occupied separately. For such analysis same level of measurements for the applied EDT values are required.

UE includes the average RSSI values per selected beam for the following quantities- 
1) average RSSI for the failed channel access attempts due to LBT failures
2) average RSSI for the successful channel access attempts.

UE includes the average applied EDT value in UL applied by UE per selected beam for the following quantities-
1) average applied EDT value for the failed channel access attempts due to LBT failure.
2) average applied EDT value for the successful channel access attempt.

### 2.2 Enhancements to the RLF-Report

It is already possible for the UE in the current specification to include in the RLF-Report that the RLF cause was an “lbtFailure” due to consistent UL LBT failures experienced in all the BWPs configured with PRACH resources of the SpCell. However, it is not possible for the UE to include in the RLF-Report information on whether at the moment of RLF, UL consistent LBT failures were triggered and not cancelled at MAC layer. For example, the failure may be due to maximum number of RLC retransmissions reached or due to random access problems, but obviously the UL consistent LBT failures may have impacted it. Hence, it is useful for the network to know that, so that the network can determine how much of the issue is due to LBT problems in the unlicensed spectrum or to other reasons not strictly related to LBT operations. Another example can be the UE generating an RLF due to random access problems in a BWP that was selected after a BWP switch triggered by consistent UL LBT failures. In this case the RLF cause would be randomAccessProblems, but the UL consistent LBT failure in the first BWP also contributed to it.

Similarly, when the UE detects an HOF, that may be due, at least to some extent, to LBT problems that the UE may have encountered during the HO.

UE logs in the RLF-Report information indications that consistent LBT failure indirectly caused RLF/HOF. FFS on the details

In certain scenarios the UE may include in the RLF-Report the RA information common. Currently, this happens in case the RLF is due to random access problems and in case of HOF. However, in case of UL consistent LBT failures experienced just before the failure, the UE may have executed multiple random-access procedures in different BWPs. The information associated to all these random access procedures initiated just before the failure while UL consistent LBT failures were triggered and not cancelled, will be lost if we follow the current specification. We believe that at least some RA information associated to those failed random access procedures may be of benefit for the network.

If at the moment of RLF/HOF, the UE had consistent UL LBT failures triggered in one or more BWPs at MAC layer, the RLF-Report includes information associated to the random access procedures that were initiated due to consistent UL LBT failures just before the RLF/HOF.

Given that RAN3 sent an LS to RAN2 that UE logs the latest RSSI measurements in the RLF report, if RAN2 agree to include the RSSI measurements in the RA-InformationCommon, the RSSI measurements will be automatically captured for the RLF report as well.

UE includes the RSSI measurements in the RA-InformationCommon logged as part of RLF report. The average RSSI can be logged per selected beam level for the successful and failed channel access attempts.

However, in some scenarios like T310 expiry, UE performs RACH attempts only if it is configured with lbt-FailureRecoveryConfig by the network. If this is not configured, UE does not perform such and may end up in not logging any information RA-InformationCommon IE.

UE may not populate RA-InformationCommon IE if it is not configured with lbt-FailureRecoveryConfig.

In such scenario, we propose that UE logs RSSI and applied EDT value information separately in the RLF report.

If RA-InformationCommon is not present, UE includes the RSSI measurements and applied EDT value in the RLF report.

### 2.3 Enhancements to the SHR

For the SHR, one immediate enhancement that RAN2 could consider is whether to introduce for the NR-U system some new triggering conditions for the SHR generation. For example, the HO may be successful but during the HO the UE may have experienced some LBT problems, e.g. UL consistent LBT failures were detected. Logging this information may be beneficial for the network to optimize the HO, and avoid problems during the HO.

Introduce new SHR triggering conditions for NR-U, e.g. the number of UL LBT failure prior to successfully completion of the HO.

Similar to the RLF case, in the current specification the UE logs the RA information in the SHR, only when the SHR is triggered due to T304 timer value becoming larger than a certain threshold. We note also for this case, that the UE may trigger random access in multiple BWPs due to consistent UL LBT failures just before successfully completing the HO. The information associated to all these random access procedures initiated just before the successful HO completion, while UL consistent LBT failures were triggered and not cancelled, will be lost if we follow the current specification. We believe that at least some information associated to those failed random access procedures may be of benefit for the network.

SHR includes information associated to the random access procedures that were initiated due to such consistent UL LBT failures just before the successful HO completion.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Logging number of LBT failures per BWP is very coarse information for network to perform any optimization.

Observation 2	UE may not populate RA-InformationCommon IE if it is not configured with lbt-FailureRecoveryConfig.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A List of RA-InformationCommon is used to capture information about the multiple RA procedures performed by the UE at different BWPs.

Proposal 2	UE logs number of LBT failures per selected beam in the RA report.

Proposal 3	UE includes the average RSSI values per selected beam for the following quantities-  1) average RSSI for the failed channel access attempts due to LBT failures 2) average RSSI for the successful channel access attempts.

Proposal 4	UE includes the average applied EDT value in UL applied by UE per selected beam for the following quantities- 1) average applied EDT value for the failed channel access attempts due to LBT failure. 2) average applied EDT value for the successful channel access attempt.

Proposal 5	According to RAN3 LS, UE logs in the RLF-Report information indications that consistent LBT failure indirectly caused RLF/HOF. FFS on the details

Proposal 6	If at the moment of RLF/HOF, the UE had consistent UL LBT failures triggered in one or more BWPs at MAC layer, the RLF-Report includes information associated to the random access procedures that were initiated due to consistent UL LBT failures just before the RLF/HOF.

Proposal 7	UE includes the RSSI measurements in the RA-InformationCommon logged as part of RLF report. The average RSSI can be logged per selected beam level for the successful and failed channel access attempts.

Proposal 8	If RA-InformationCommon is not present, UE includes the RSSI measurements and applied EDT value in the RLF report.

Proposal 9	Introduce new SHR triggering conditions for NR-U, e.g. UL LBT failure prior to successfully completion of the HO.

Proposal 10	SHR includes information associated to the random access procedures that were initiated due to such consistent UL LBT failures just before the successful HO completion.

# 4	References

RP-221825, Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), 3GPP TSG RAN meeting #96, Budapest, Hungary, June 6-9, 2022.

R3-225241, LS on NR-U support for MRO, 3GPP TSG-RAN WG2 #117-e, 15 – 24 Aug 2022