3GPP TSG-RAN WG2 Meeting #120	R2-2212127

Toulouse, France, 14-18 November 2022

Source: 			Qualcomm Incorporated

Title: 				Discussion on TN-NTN mobility in RRC INACTIVE and SDT Capability

Document for: 	Decision

Agenda Item: 	6.0.1.1

# Introduction

RAN2 in their #119bis meeting discussed the issue of UE capability difference between TN and NTN modes in case of NTN-TN mobility in RRC_INACTIVE with the focus on SDT UE capability [1][2].

The discussion was postponed with the following note.

- On whether to support RNA configuration across TN and NTN cells, there is no consensus and companies are encouraged to bring this discussion to next RAN2 (also possibly RAN3) meetings to reach a consensus

# Discussion

The scenario for the discussion is as follows.

- The UE supports SDT in TN and does not support the same in NTN.

- The UE receives sdt-Config from TN cell and reselects to NTN cell broadcasting sdt-ConfigCommon.

- According to the current specification text, the UE uses SDT procedure, if applicable, in the NTN cell. This is incompatible with the UE capability.

In the offline discussion, summarized in [2], companies considered that the problem could be addressed if the network makes sure RNA configuration spans cells only from a single RAT (TN or NTN). This ensures the UE triggers RNA Update (SRB0), which is not subject to SDT, when it moves from TN cell to NTN cell and vice versa.

This however misses the fact that the mobility between TN cell and NTN cell can also result in Tracking Area change, hence NAS triggering Registration Update procedure. As can be seen in the following text in TS38.331, RNA Update is triggered only if upper layers do not send NAS message as a result of SIB1 acquisition.

##### 5.2.2.4.2 Actions upon reception of the SIB1 […] 4> forward the trackingAreaCode to upper layers; 4> forward the trackingAreaList to upper layers, if included; 4> forward the received posSIB-MappingInfo to upper layers, if included; 4> forward the PLMN identity or SNPN identity or PNI-NPN identity to upper layers; 4> if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers: 5> if the serving cell does not belong to the configured ran-NotificationAreaInfo: 6> initiate an RNA update as specified in 5.3.13.8;

In this particular scenario, NAS message for Registration Update needs to be evaluated for SDT if sdt-SRB2-Indication is configured.

Observation 1:	The restriction that RNA configuration cannot span across TN and NTN cells does not sufficiently address the issue of SDT UE capability difference between TN and NTN.

It is our view that the entire impact of UE capability differences between TN and NTN on RRC INACTIVE mobility should be investigated in more holistic manner. The fact that the UE may be configured with something it does not support due to TN-NTN mobility requires careful review of all relevant UE capabilities, i.e. not limited to SDT.

Proposal 1:	RAN2 to investigate the impact of UE capability differences between TN and NTN on RRC INACTIVE mobility in more holistic manner in release-17 NTN WI agenda [3].

# Conclusion

In this document we continued the discussion on the issue of UE capability difference between TN and NTN in case of NTN-TN mobility in RRC_INACTIVE.

Observation 1:	The restriction that RNA configuration cannot span across TN and NTN cells does not sufficiently address the issue of SDT UE capability difference between TN and NTN.

Proposal 1:	RAN2 to investigate the impact of UE capability differences between TN and NTN on RRC INACTIVE mobility in more holistic manner in release-17 NTN WI agenda [3].

# References

[1]		R2-2209925		Issues on Small Data Transmission under TN & NTN Mobility		FGI

[2]		R2-2211051		Report for [AT119bis-e][003][NR17] RRC corrections		Huawei, HiSilicon

[3]		R2-2211568		Discussion for clarification on TN NTN mobility in RRC_INACTIVE		Qualcomm Incorporated