3GPP TSG-RAN WG2 Meeting #120	R2-2212153

# Toulouse, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3679 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice-based random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently it is not clear from RAN specifications how UE derives the NSAG to use for slice-based random access. Lower layers (MAC) assumes this is provided by its higher layers (RRC), but correspondling information is missing in RRC specification. It is not clear that the slice-based random access applies only to RRC_Idle -> RRC_Connected transition. Further, not captured in RAN specification that UE might trigger a slice-based cell re-selection before the RRC Connection establislishment is initiated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At initiation of RRC connection establishment, it is added that lower layers (MAC) in the random access procedure for the RRC connection establishment apply the highest prioritized NSAG (as provided by upper layers (NAS)) that are published in SIB1, and that are associated with the slices that triggered the access. Referenses are added for the relevant specifications. Added a note to indicate that the UE may trigger a slice-based cell re-selection before the RRC connection establishment procedure initiated, to re-select to a cell that supports the most prioritized NSAG. Impact Analysis Impacted 5G architecture options: NR SA, NE-DC,NR-DC Impacted functionality: RAN slicing enhancement Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no obvious inter-operability issues. 2. If the UE is implemented according to the CR and the network is not, there is no obvious inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Slice-based random access is missing in the specification. The specification will remain unclear and deviate from UE protocol model for slice-based random access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide NSAG information (including NSAG priority) and one or more S-NSSAI(s) associated with the S-NSSAI(s) of the signalling transaction triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

2>	apply the NSAG with highest NSAG priority among the NSAGs that are published in SIB1, and that are associated with the S-NSSAI(s) of the signalling transaction triggering the access attempt, in the random access procedure (TS 38.321 [3], clause 5.1);

NOTE: 	The NSAG information provided by upper layers may trigger a slice-based cell reselection (see TS 38.304 [20]) before this procedure is initiated (see TS 23.501 [32], clause 5.3.4.3.4)

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

End of changes