3GPP TSG-RAN WG2 Meeting #120	R2-2212169

Toulouse, France, 14th – 18th November, 2022

Agenda Item:	8.2.2

Source:	Spreadtrum Communications

Title:	Discussion on potential solutions for SL positioning

Document for:	Discussion and Decision

Introduction

In RAN1 #109, RAN1 discussed the SL-PRS resource allocation and agreed the following two schemes:

Agreement With regards to the SL-PRS resource allocation, study the following two schemes: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS - Applicable regardless of the network coverage - FFS: potential mechanisms, if needed, for SL-PRS resource coordination across a number of transmitting UEs (e.g. IUC-like solutions). - Note: Other Schemes are not precluded to be studied FFS how to handle resource allocation of SL-Positioning measurement report

In RAN2 #119e and RAN2 #119-bis-e, RAN2 discussed the SL positioning and agreed the following:

RAN2#119e Agreements: Proposal 1 (modified): Confirm that for sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. FFS if partial coverage case assumes anything about which UEs are in coverage. Proposal 2: Study the architecture and signaling procedures to enable at least the following two operation scenarios: - Operation Scenario 1: PC5-only-based positioning. - Operation Scenario 2: Combination of Uu- and PC5-based positioning. RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server. FFS from RAN2 perspective if there are cases without a UE in the location server role. Proposal 4 (modified): Align with SA2/RAN1 on the terms for sidelink positioning, and introduce the following terms of UE role as the baseline for further discussion: - Target UE: UE to be positioned - Anchor UE: UE supporting positioning of target UE, e.g., by transmitting and/or receiving anchor signals for positioning, providing positioning-related information, etc., over the SL interface. FFS: clarification of the knowledge of the anchor UE. Additional roles can be considered. Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP). FFS where it is specified. The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355). Study the potential impact to LPP for support of sidelink positioning procedures between UE and LMF. FFS how much impact (if any), e.g., only to carry the new protocol, and if the PC5-only and hybrid PC5+Uu cases are the same or different. RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers. RAN2 will study the question of cast type for positioning signalling. For SL-PRS, follow RAN1 decision and consider cast type if something arises in RAN2 scope.

RAN2 #119-bis-Agreements:

Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work.

1.	Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF

2.	Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP

3.	Use of SLPP/RSPP between the UE and the LMF

Agreement:

Proposal 3 (modified): In order to enable sidelink positioning, SLPP/RSPP shall support at least the following functionalities:

1.	SL Positioning Capability Transfer

2.	SL Positioning Assistance Data exchange

3.	SL Location Information Transfer

4.	Error handling

5.	Abort

This agreement does not imply any specific signalling structure.

Agreements:

Proposal 5: Unicast/one-to-one operation is assumed as baseline for exchange of sidelink positioning signaling.

Proposal 6 (modified): RAN2 shall study applicability of at least the following positioning signaling for groupcast/broadcast (in addition to unicast), including addressing any security aspects (involving SA3 where needed). FFS the specific use case:

•	SL positioning capability transfer

•	SL positioning assistance data

•	FFS SL location information transfer

Agreements:

Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation.  FFS if groupcast/broadcast operation, if supported, would be session-based or session-less.

Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs.  RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

Agreement:

Proposal 4(14/20) (modified): RAN2 do not decide to support the role of assistant UE for now.  FFS if there is spec impact in RAN2 from the assistant UE.

In this contribution, we further discuss the sidelink positioning issues with respect to protocol, signalling procedure, SL server UE, etc., and express our opinions.

# 2 Discussion

SL positioning protocol stack

In the last meeting, RAN2 has agree that a new protocol for SL positioning between UEs will be introduced. And the functionality of new protocol is similar to LPP protocol and the following procedure can be performed by using new protocol:

- transfer capability related to SL positioning between UEs;

- transfer assistance data related to SL positioning between UEs;

- transfer location information related to SL positioning between UEs.

Observation 1: A new protocol for sidelink positioning (e.g., RSPP, SLPP) at least includes the following functionalities:

- transfer capability related to SL positioning between UEs;

- transfer assistance data related to SL positioning between UEs;

- transfer location information related to SL positioning between UEs.

And further, the new protocol stack for sidelink positioning between UEs needs to be further study. Before that, we can review the existing SL protocol stack. In sidelink, radio protocol stack for NR sidelink communication has four types as shown in Figure 1-4: 1) Control plane protocol stack for SCCH for RRC; 2) Control plane protocol stack for SCCH for PC5-S; 3) Control plane protocol stack for SBCCH; 4) User plane protocol stack for STCH. We can see that PC5-S messages are carried directly over PDCP. Referring to Section 9.1.1.4 of TS 38.331, different PC5-S messages are transmitted by PDCP using SRB0, SRB1 and SRB2 respectively. And in SL, PC5-S is still regarded as control plane signalling, but it is directly carried over PDCP, and it does not need to be encapsulated in PC5-RRC.

Figure 1: CP protocol stack for SCCH for RRC.      Figure 2: CP protocol stack for SCCH for PC5-S.

Figure 3: CP protocol stack for SBCCH.                   Figure 4: UP protocol stack for STCH.

Considering the existing protocol stack of the SL interface, there are two options to carry positioning new protocol:

- Alt 1: CP-based solution, with new protocol over PC5-S or over PC5-RRC or over PC5-D;

- Alt 2: CP-based solution, with new protocol over PDCP.

Figure 5 illustrates one possible CP-based protocol stack for alternative 1. For Alt 1, SLPP/RSPP messages are carried over PC5-S or PC5-RRC. In this option, SLPP/RSPP messages needs to be encapsulated into PC5-S or PC5-RRC. This design is similar to LPP that it is over NAS and transmission reliability and latency can be guaranteed. But alt1 increases processing complexity. And PC5-S/PC5-RRC/PC5-D also needs to be enhanced to support three cast types. And another possible CP-based protocol stack for alternative 2 in Figure 6. It is similar to the protocol stack of PC5-S over PDCP in Figure 2 and SL positioning signalling can be transmitted over PDCP layer as SRB. SLPP/RSPP does not need to be encapsulated into PC5-S or PC5-RRC. This option can reduces processing complexity and ensures the priority of SL signalling. And another option is SL signalling over PDCP as DRB. This option cannot guarantee the scheduling priority of SL signalling.

Above all, we think that Alt2 solution is more suitable to processing SLPP/RSPP messages.

Figure 5. CP stack over PC5-S/RRC for SLPP/RSPP       Figure 6. CP stack over PDCP for SLPP/RSPP

Proposal 1: CP-based protocol stack over PDCP may be more suitable for SL positioning signalling transmission between UEs.

In the last meeting, RAN2 agreed to future study the following three protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage and RAN2 will down-select during normative work.

1.	Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF

2.	Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP

3.	Use of SLPP/RSPP between the UE and the LMF

But LS from SA2 related to SL positioning is sent to RAN group [1], the protocol used between UE and LMF are mentioned. From RAN2 perspective, option2 has least spec impact and ensures that the protocol between UE and LMF, and UEs can be defined as common as possible.

Proposal 2: For protocol between UE and LMF, SLPP/RSPP can be transported as a container within LPP.

General signalling procedure

The last meeting has agreed that for SL positioning in-coverage, partial coverage and out-of-coverage scenarios shall be supported. And the general signalling procedure can be separately analysed for out-of-coverage and in-coverage scenarios.

For out-of-coverage scenarios, there is no LMF involved. SL positioning procedure occurs between UEs. The following Figure 7 illustrates a general SL positioning signalling procedure for out-of-coverage scenarios.

Figure 7: A general SL positioning procedure for out-of-coverage scenarios

1a) or 1b).	The UE A receives a Sidelink Location Request message, or the UE A initiated a SL positioning location request itself. UE A may be a target UE or anchor UE.

2.	The UE A may initiate the discovery with UE B and/or perform anchor UE (re)selection.

3.	The UE A initiates the sidelink positioning capability exchange procedure with UE B.

4.	The UE A performs the assistance data transfer procedure with UE B.

5.	The UE A transmits the SL PRS and the UE B measure the SL PRS, or the UE B transmit the SL PRS and the UE A measures the SL PRS.

6.	The UE A performs the location information transfer procedure with UE B.

7.	The UE A or UE B calculate the SL positioning estimate results.

8.  The UE B transmits location estimate results to UE A

9a) or 9b).	The UE A provides the location estimate via Sidelink Location Response.

The following Figure 8 illustrates a general SL positioning signalling procedure for in-coverage scenarios.

Figure 8: A general SL positioning procedure for in-coverage scenarios

1a) or 1b).	The LMF receives a Location Service Request from the AMF or target UE.

2.	The LMF obtains the sidelink positioning capability of the target UE from the target UE, e.g., via the LPP Request Capabilities message. Or, The LMF obtains the Uu positioning result estimate of the target UE from the target UE, e.g., via the LPP provide location information message.

3.	The LMF determines to perform the sidelink positioning, e.g., when the UE is not able to perform Uu-based positioning (not support, or not able to measure on enough TRP to perform Uu-based positioning), or, when the Uu positioning can not meet the positioning QoS requirements.

4.	The LMF may initiate a discovery procedure to find suitable anchor UEs via PC5 interface.

5.	The LMF may perform the exchange of LPP messages for sidelink positioning between the LMF and Anchor UE(s), e.g., Assistance Data Transfer and Location Information Transfer.

6.	The LMF performs the exchange of LPP messages for sidelink positioning between the LMF and the target UE, e.g., Assistance Data Transfer and Location Information Transfer.

7a) or 7b).	The LMF or target UE obtains the location information of the target UE based on the information from Step 5 or 6.

8a) or 8b).	The LMF sends Location Service Response to the AMF or target UE.

Proposal 3: UE-based and network-based SL positioning methods should be considered.

Proposal 4: Capture Figure 7 and 8 into the TR as a general SL positioning procedure for out-of-coverage and in-coverage scenarios.

Anchor UE (re)Selection

After initiate sidelink positioning, we need to find suitable anchor UEs to assist positioning target UE. In order to determine the relative or absolute location of the target UE, we need to find suitable assisted UEs or anchor UEs.

Currently SA2 has a project, namely, Study on Architecture Enhancement to support Ranging based services and sidelink positioning, to study the architecture and signalling enhancement for SL positioning. SA2 lists a key issue “Ranging/Sidelink Positioning device discovery” to study how to find suitable SL devices, and takes the discovery Model A and Model B procedure as the baseline in TR 23.700-86. Thus, in the subsequent RAN2 discussion, we can assume that model A and model B can be a baseline for SL positioning discovery procedure.

In discovery procedure, discovery message may include the UE type, positioning service code, UE capability, etc. message. And this part will be discussed in SA2. And in RAN2, we need to discuss the selection of anchor UEs.  For the sake of select the appropriate anchor UEs, besides high layer criteria in the discovery procedure, AS layer criteria should also be considered due to the uncertainty of wireless environment. Mobility, channel condition such as NLOS/LOS indication of AS layer criteria may be considered.

Proposal 5: In order to select suitable anchor UEs, AS layer criteria should be considered besides the high layer criteria of discovery procedure.

Resource Allocation

SA2 wonder whether SL positioning server UE functionalities can support more functionalities, e.g. SL Positioning/Ranging method determination, operation coordination, resource coordination and scheduling, in addition to result calculation. From our perspective, SL positioning method determination, operation coordination and resource coordination can be support by SL positioning server UE. Especially, for out-of-coverage UE, there is no network involved and SL server UE can collect the UE capabilities and positioning KPI requirements to determine the positioning methods.

For resource coordination and scheduling, in scheme1, because the positioning needs to be completed by multi-point cooperation, network can cooperate to allocate SL-PRS resources in a centralized way for multiple UEs. And SL positioning server UE can serve as a collaboration node to distribute network scheduled resources.

And for scheme2, we can reuse SL communication resource allocation coordination scheme: inter-UE-coordination. And the further enhancements for scheme2 can be performed in SL topic.

Proposal 6: In addition to location calculation, SL Positioning Server functionalities can support more functionalities, e.g. SL Positioning/Ranging method determination, operation coordination and resource coordination and scheduling.

More specifically, for scheme 1 resource allocation, the gNB may allocate resources for SL-PRS in a centralised manner. In order to obtain the relative or absolute positioning estimate for a positioning session, multiple UEs may be involved as shown in Figure 9. Purple UE may be as a target UE requiring positioning estimate. And other white UEs serve as anchor UEs to transmit or measurement SL PRS to support positioning of target UE. According to the current resource allocation scheme, the in-coverage UEs will request the serving gNB to obtain the SL-PRS resources respectively, and out-of-coverage UEs will obtain the resources through sensing by itself.

And in order to reduce signalling overhead between UE and gNB for in-coverage UEs or to avoid sensing procedure for out-of-coverage UEs, RAN2 can further study mode1 coordination of SL PRS resources scheme. In this scheme, network can cooperate to allocate SL-PRS resources in a centralized way for multiple UEs participating a SL positioning session regardless of UE within out-of-coverage or in-coverage.

Figure 9. SL positioning scenario for centralised resources allocation

Proposal 7: RAN2 to further study mode1 coordination of SL PRS resources for multiple UEs participating a SL positioning session.

SL-PRS Configuration procedure

After sidelink positioning initiation and anchor UE selection procedure, SL-PRS configuration is a critical issue. In Rel-16/Rel-17 positioning, PRS and SRS are configured by LMF/gNB through LPP/RRC signalling.

Back to the SL-PRS configuration, because SL UE can be in-coverage, partial-coverage and out-of-coverage. Thus, there are three types of configuration.

The first type is that LMF or gNB determines the SL-PRS configuration and sends the configuration to SL UE. The first type is applicable to in-coverage UE or out-of-coverage UE. For the RRC_CONNECTED UE, the configuration can be send to UE via dedicated signalling. And for RRC_IDLE/INACTIVE UE, the configuration can be send to UE via system information. And for out-of-coverage UE, the SL-PRS can also be configured to UE via pre-configuration. The pre-configuration can be allocated to UE when it is in coverage.

The second type is the combination of semi-static configuration and sensing. Specifically, some semi-static configuration parameters, such as, RS comb size, RS bandwidth, number of symbols, resource type etc., can be configured to the UE by LMF or gNB.  And the dynamic parameters can be determined by UE itself, such as obtaining the time-frequency resources through sensing. And this options can be used by in-coverage and out-of-coverage UEs. Similar to the type one, for the RRC_CONNECTED UE, the semi-static configuration can be send to UE via dedicated signalling. And for RRC_IDLE/INACTIVE UE, the semi-static configuration can be send to UE via system information. And for out-of-coverage UE, the semi-static configuration can also be send to UE via pre-configuration.

The third type is that the SL-PRS configuration is determined by UE itself, that is, the specific time-frequency resources configuration can be determined via mode2 sensing. And other SL-PRS configuration is determined by UE. This type may only applied for specific scenarios, such as for out-of-coverage UE and without pre-configuration parameters.

Proposal 8: Consider following SL-PRS configuration schemes:

- Alt1: SL-PRS is configured by gNB/LMF via high layer signalling;

- Alt2: SL-PRS semi-static parameters are configured by gNB/LMF and dynamic parameters are determined by UE;

- Alt3: SL-PRS is determined by UE.

Combination SL and Uu Positioning

Sidelink interface have many roles when positioning a target UE. Simplicity scheme is standalone sidelink positioning. That is SL positioning can independently determine a location of target UE, especially for out-of coverage UEs. Figure 10 illustrates standalone SL positioning. In this case, UE can obtain relative location information with respect to another UE, or obtain absolute location relative to a UEs with known locations via SL positioning measurements.

Figure 10. PC5-only based positioning

For Combination sidelink and Uu positioning, specific scenarios are shown in figure 11-13. Specifically， Figure 11 is SL positioning to enhance Uu positioning. Figure 12 is combined SL and Uu positioning to enable positioning. And Figure 13 is that relative position is obtained by SL positioning, and the absolute position is obtained by combining with Uu positioning.

Figure 11. SL positioning to enhance Uu positioning   Figure 12. Combined SL and Uu positioning to enable positioning

Figure 13. SL communication to offload Uu control signalling

- SL positioning to enhance Uu positioning performance.

Figure 11 shows the specific scenario. SL positioning can be utilized to enhance the performance of Uu positioning. When the UE needs ultra-high positioning accuracy, or there are harsh NLOS conditions of Uu link, target UE (purple UE) can initiate hybrid positioning. It can be seen from the figure that normal Uu positioning may be able to determine the location of the target UE, but the accuracy may not be enough. Thus, turning on hybrid positioning can improve positioning accuracy.

In hybrid positioning, anchor UE can be RSU or other UE with known/fixed location. And the location of anchor UE can be determined by Uu-based positioning. In this scenario, LMF calculation location (network-based) or UE calculation location (UE-based) can be supported.

- Combined SL and Uu positioning to enable positioning function

Figure 12 shows the specific scenario. In this scenario, the wireless condition is deteriorated seriously and LOS path is rare. The positioning function cannot be achieved only by Uu interface positioning. Thus, SL positioning provides additional path to achieve positioning function. SL anchor UE also need have known or fixed locations. The location of SL anchor UE can obtain by Uu positioning.

- SL communication to offload Uu control signalling

In this scheme, SL positioning is controlled by gNB/LMF via Uu signalling. Normally, Uu signalling should be transmitted by Uu link between anchor UEs and gNB. However, the anchor UE may not be in-coverage of network, and in order to reduce the Uu signalling overhead, SL link can be used to offload Uu control signalling toward SL positioning. In this scenario, the target UE (purple UE) is similar to the relay UE to relay the Uu control signalling between gNB and anchor UEs (blue UE).

From the above analysis, the three scenario have benefits to positioning. Thus we think RAN2 should to support hybrid positioning.

Observation 2： Combination SL and Uu positioning can facilitate the positioning accuracy.

Proposal 9: Figure 11, 12 and 13 may be used as subsequent research scenarios for Combination SL and Uu positioning.

For above Figure 11 and 12 scenarios, we can see that SL positioning is used as a supplement to improve the accuracy of Uu positioning or enable the positioning function. And SL positioning may be initiated when the radio condition or Uu positioning are worse. And further which entity will decide to initiate SL positioning, e.g., LMF or serving gNB or target UE? Thus we think we should further study this issue:

Proposal 10: RAN2 to further study a trigger procedure to enable combination of Uu and PC5-based positioning.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 positioning sidelink positioning techniques. And the proposals are given as follows:

Observation 1: A new protocol for sidelink positioning (e.g., RSPP, SLPP) at least includes the following functionalities:

- transfer capability related to SL positioning between UEs;

- transfer assistance data related to SL positioning between UEs;

- transfer location information related to SL positioning between UEs.

Proposal 1: CP-based protocol stack over PDCP may be more suitable for SL positioning signalling transmission between UEs.

Proposal 2: For protocol between UE and LMF, SLPP/RSPP can be transported as a container within LPP.

Proposal 3: UE-based and network-based SL positioning methods should be considered.

Proposal 4: Capture Figure 7 and 8 into the TR as a general SL positioning procedure for out-of-coverage and in-coverage scenarios.

Proposal 5: In order to select suitable anchor UEs, AS layer criteria should be considered besides the high layer criteria of discovery procedure.

Proposal 6: In addition to location calculation, SL Positioning Server functionalities can support more functionalities, e.g. SL Positioning/Ranging method determination, operation coordination and resource coordination and scheduling.

Proposal 7: RAN2 to further study mode1 coordination of SL PRS resources for multiple UEs participating a SL positioning session.

Proposal 8: Consider following SL-PRS configuration schemes:

- Alt1: SL-PRS is configured by gNB/LMF via high layer signalling;

- Alt2: SL-PRS semi-static parameters are configured by gNB/LMF and dynamic parameters are determined by UE;

- Alt3: SL-PRS is determined by UE.

Observation 2： Combination SL and Uu positioning can facilitate the positioning accuracy.

Proposal 9: Figure 11, 12 and 13 may be used as subsequent research scenarios for Combination SL and Uu positioning.

Proposal 10: RAN2 to further study a trigger procedure to enable combination of Uu and PC5-based positioning.

References

[1]. R2-2211139, LS on RAN dependency for Ranging/Sidelink Positioning, 3GPP SA WG2 #S2-153e.