3GPP TSG RAN WG2 Meeting #120                                                           R2-2212186

# Toulouse, France, 14th-18th November 2022

Agenda item:	8.18.2

Source:	Intel Corporation

Title:	MT-SDT mechanism

Document for:	Discussion and decision

# Introduction

This document address how to specify mobile terminated triggered SDT (MT-SDT) with impacts foreseen in RAN2, RAN3 as captured in WID [1]. A background on related LTE feature is provided, i.e., Mobile Terminated Early Data Transmission (MT-EDT) mechanism. As this is the first meeting, areas for RAN2 discussion and to get inputs from RAN3 are identified considering the identified objectives in the agreed WID [1]:

- MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response;

- MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

Note: Data transmission in DL within paging message is not in scope of this WI.

# Discussion

## [Background] LTE MT-EDT

MT-EDT mechanism are defined in LTE for Control Plane CIoT EPS Optimisation (CP CIoT EPS Optim.) and for MT-EDT for User Plane CIoT EPS Optimisation (UP CIoT EPS Optim.). CP CIoT kind of solutions are not applicable to NR, therefore the focus is on understanding the background details on MT-EDT solution defined for UP CIoT.

LTE feature of MT-EDT for User Plane CIoT EPS Optimisation targets similar requirements as Rel-18 MT SDT in NR/5GC.

### MT-EDT triggering (with UP CIoT EPS Optim.)

Figure 1 summarizes how MT-EDT is triggered with UP CIoT EPS Optim. as captured in TS 36.300 [2]:

Figure 1. MT-EDT for User Plane CIoT EPS Optimisation. as captured in TS 36.300 [2]

Some points to highlight on the MT-EDT triggering part (for User Plane CIoT EPS Optim.):

- eNB receives from MME “DL data size information” as part of S1-AP PAGING message.

- If the data can fit in one single downlink transmission, eNB includes mt-EDT indication in the Paging message for the UE. Note: LTE EDT feature only supported transmission of 1 UL and 1 DL transmission (which is different than for Rel-17 SDT and the new target Rel-18 SDT. The ASN.1 associated with this mt-EDT indication in Paging message is shown below as captured in TS 38.331 [3]:

Paging ::=					SEQUENCE {

pagingRecordList			PagingRecordList			OPTIONAL,	-- Need ON

systemInfoModification		ENUMERATED {true}			OPTIONAL,	-- Need ON

etws-Indication				ENUMERATED {true}			OPTIONAL,	-- Need ON

nonCriticalExtension		Paging-v890-IEs				OPTIONAL

}

<*** TEXT OMITTED ***>

Paging-v1530-IEs ::=			SEQUENCE {

accessType					ENUMERATED {non3GPP}		OPTIONAL,	-- Need ON

nonCriticalExtension		Paging-v1610-IEs			OPTIONAL

}

Paging-v1610-IEs ::=			SEQUENCE {

pagingRecordList-v1610		PagingRecordList-v1610		OPTIONAL,	-- Need ON

uac-ParamModification-r16	ENUMERATED {true}			OPTIONAL,	-- Need ON

nonCriticalExtension		Paging-v1700-IEs			OPTIONAL

}

PagingRecordList ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord

PagingRecordList-v1610 ::=	SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v1610

PagingRecord-v1610 ::=		SEQUENCE {

accessType-r16				ENUMERATED {non3GPP}	OPTIONAL,		-- Need ON

mt-EDT-r16					ENUMERATED {true}		OPTIONAL		-- Need ON

}

For MT-EDT for User Plane CIoT EPS Optim., when eNB determines that MT-EDT can be initiated, eNB sends an mt-EDT indication to a UE in the specific paging record in the Paging message.

### UL/DL EDT data after MT-EDT initiation with UP CIoT EPS Optim.)

Some points to highlight on the EDT data exchanged after UE detects a MT-EDT indication (for User Plane CIoT EPS Optim.):

- UE selects a random access preamble not configured for EDT and sends RRCConnectionResumeRequest message with the resume cause mt-EDT and without user data.

RRCConnectionResumeRequest-r13-IEs ::=		SEQUENCE {

resumeIdentity-r13								CHOICE {

resumeID-r13									ResumeIdentity-r13,

truncatedResumeID-r13							BIT STRING (SIZE (24))

},

shortResumeMAC-I-r13							BIT STRING (SIZE (16)),

resumeCause-r13									ResumeCause,

spare											BIT STRING (SIZE (1))

}

<*** TEXT OMITTED ***>

ResumeCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280,

mt-EDT-v1610

}

- The MME could include the Pending Data Indication in the S1AP UE Context Resume Response message to notify the eNB of further data traffic in excess of that initially signalled. Based on that the eNB may use this indication to decide whether to release the UE

When UE detect MT-EDT data for User Plane CIoT EPS Optimisation, UE initiates resume with non-EDT RA preambles and including a mt-EDT resume cause value.

## Functional operation target by MT-SDT feature

Firstly, RAN2 needs to align on the desirable approach to discuss possible solutions that could enable MT-SDT operation for RA-SDT and CG-SDT. From plenary discussion and considering the agreed WID [1], our understanding is that majority preferred enabling a common solution for both mechanisms (i.e., differences are only discussed, if/when required) and Rel-17 SDT solution is re-used as a baseline for Rel-18 MT-SDT after the MT-SDT paging trigger is detected. Moreover, it is good to confirm the understanding that a UE configured with MT-SDT should also be configured with RA-SDT or CG-SDT.

- RA-SDT and CG SDT solutions specified in Rel-17 is re-used as a baseline for Rel-18 MT-SDT after the MT-SDT paging trigger is detected.

- A common MT-SDT solution is target for subsequent RA-SDT and CG-SDT. FFS if any different handling is required for any step.

- A UE configured with MT-SDT needs to also be configured with RA-SDT and/or CG-SDT.

The following functionality is required for MT-SDT feature to work:

- DL data belongs to a radio bearer (RB) that is configured to RA-SDT and/or CG-SDT

- gNB informs the UE when MT-SDT traffic is detected for the given UE via paging.

- UE is monitoring paging for a MT-SDT indication, when applicable (e.g. when defined in specification or configured by the network depending on how the feature is defined)

- UE initiates resume for SDT upon detecting a MT-SDT indication for the given UE.

- If a UE and network supports MT-SDT, the following new functionality is supported to enable MT-SDT:

  - DL data can trigger MT-SDT paging when belongs to a RB that is configured with RA-SDT and/or CG-SDT. Suggest using the term “MT-SDT traffic” to refer to any DL SDT traffic carried over a RB configured for SDT.

  - gNB informs the UE when MT-SDT traffic is detected for the given UE via paging.

  - UE will monitor a MT-SDT indication sent via paging (e.g. when defined in specification or configured by the network depending on how the feature is specified).

  - UE initiates resume for SDT upon detecting a MT-SDT indication for the given UE.

## Further impacts foreseen by MT-SDT feature

### MT-SDT triggering impacts on network

When the anchor gNB (i.e., the gNB where the UE Inactive AS Context is stored) receives MT-SDT traffic, the gNB would need to notify the UE and/or other gNBs in UE’s configured RNA about this MT-SDT traffic. In addition to the functionality already identified above, the following impacts are also foreseen:

- Anchor gNB needs to detect when MT-SDT traffic is available in DL. For LTE, this was handled by MME and a threshold of the maximum size supposed was defined considering UE’s capability as for MT EDT, UE could only exchange one transmission in DL. For NR, this is different as multiple] data transmissions are allowed via SDT. I.e., if MT-SDT traffic is large to be able to send it all at once to UE, it can be segmented while still send it in a given SDT session. Therefore, the decision on whether anchor gNB can trigger or not MT-SDT can be left up to gNB implementation for NR.

- Anchor gNB informs other gNBs in the UE’s configured RNA when MT-SDT traffic is detected for that given UE to trigger a Paging message with MT-SDT indication. This would only apply for RA-SDT. Xn impact is foreseen (e.g., a new Paging cause) as different gNBs puts together the final RRC Paging message.

- It is left up to gNB implementation when/how to detect/trigger that MT-SDT traffic is available.

- Anchor gNB informs other gNBs in the UE’s configured RNA when MT-SDT traffic is detected for that given UE to trigger a Page for MT-SDT. Inform RAN3 on potential impact to Xn interface from this. Note: this impact would only apply to MT-SDT with RA-SDT (i.e., not to CG-SDT).

### Ongoing UL/DL SDT traffic after MT-SDT initiation

The assumption is that the MT-SDT traffic belongs to one of the RBs configured for SDT operation as explained in Proposal 4. The potential impacts when initiating SDT after UE detects MT-SDT being available are:

- How UE performs initial access upon detecting MT-SDT notification from gNB.

- Whether/how gNB is aware that this UE is initiating resume for MT-SDT (i.e., without any UL SDT traffic).

- When initiating SDT after UE detects MT-SDT being available for the given UE, RAN2 needs to discuss:

  - How UE performs initial access upon detecting MT-SDT notification from gNB.

  - Whether/how gNB is aware that this UE is initiating resume for MT-SDT (i.e., without any UL SDT traffic).

Further analysis is performed here below for the possible ways that UE may initiate resume for SDT considering these last points 6) and 7):

- When UE initiates RA-SDT, possible ways on how gNB can differentiate that this UE is accessing for MT-SDT operation:

- UE uses non-SDT RA proc. (2 step or 4 step) and normal resume procedure (i.e,. not using SDT RACH resource). When performing normal resume procedure, UE could apply it same as legacy (e.g., legacy MT resume cause) or alternatively a new MT-SDT resume indication (e.g. as a new MT-SDT cause value as in LTE-EDT) could be used to easy gNB implementation when determine that UE is initiating RA-SDT due to MT-SDT.

- UE uses SDT RA proc. to initiate RA-SDT procedure. This would imply a waste of UL resources as UE does not have UL SDT traffic, at least for the 1st UL SDT transmission. The advantage is that the gNB would know directly that RA-SDT is initiated. Moreover, the anchor gNB could potentially know by itself that this UE is initiating RA-SDT for the MT SDT traffic as this gNB is the one that triggered the corresponding MT-SDT paging and its corresponding DL SDT traffic is stored in its UE’s buffers. If this is not preferable, it is also possible to consider whether a new MT-SDT indication is provided by UE as it is explained in previous option (A.1).

- When UE initiates with CG-SDT, possible ways on how gNB can differentiate that this UE is accessing for MT-SDT operation:

- UE initiates CG-SDT in the configured CG-SDT resource. However, there would not be UL SDT traffic which might lead to a potential waste in those UL resources that are already scheduled for the SDT operation of a given UE. For this option, one potential problem that will require discussion is whether there is any issue on using CG-SDT resources to initiate CG-SDT triggered by MT-SDT when the MT-SDT traffic might belong to a DRB that is not configured for CG-SDT configuration (i.e., a DRB only configured for RA-SDT).

- Same as previous option (B.1) but UE uses normal resume to initiate the SDT operation using CG-SDT resource. The potential advantage is that UE is initiating normal resume due to MT-SDT via CG-SDT resources and gNB later “transitions” (i.e., keeps) the UE in SDT operation (i.e. UE is not moved into RRC_CONNECTED while MT SDT is sent and potentially any subsequent UL/DL SDT traffic).

In summary, further discussion is needed for RAN2 to evaluate the foreseen impacts and the preferred approach.

- Further impacts for RAN2 to evaluate considering that UE may initiate RA-SDT or CG-SDT in response to MT-SDT:

    - If UE initiates RA-SDT, which RA procedure is initiated (e.g. RA-SDT vs non-RA-SDT) is selected to initiate RA-SDT due to MT-SDT.

    - If UE initiates CG-SDT, whether CG-SDT resources are used to initiate CG-SDT due to MT-SDT.

# Conclusion

The observations captured are the following:

Observation 1.	LTE feature of MT-EDT for User Plane CIoT EPS Optimisation targets similar requirements as Rel-18 MT SDT in NR/5GC.

Observation 2.	For MT-EDT for User Plane CIoT EPS Optim., when eNB determines that MT-EDT can be initiated, eNB sends an mt-EDT indication to a UE in the specific paging record in the Paging message.

Observation 3.	When UE detect MT-EDT data for User Plane CIoT EPS Optimisation, UE initiates resume with non-EDT RA preambles and including a mt-EDT resume cause value.

The proposals captured are the following:

Functional operation target by MT-SDT feature

Proposal 1.	RA-SDT and CG SDT solutions specified in Rel-17 is re-used as a baseline for Rel-18 MT-SDT after the MT-SDT paging trigger is detected.

Proposal 2.	A common MT-SDT solution is target for subsequent RA-SDT and CG-SDT. FFS if any different handling is required for any step.

Proposal 3.	A UE configured with MT-SDT needs to also be configured with RA-SDT and/or CG-SDT.

Proposal 4.	If a UE and network supports MT-SDT, the following new functionality is supported to enable MT-SDT:

Proposal 4.1.	DL data can trigger MT-SDT paging when belongs to a RB that is configured with RA-SDT and/or CG-SDT. Suggest using the term “MT-SDT traffic” to refer to any DL SDT traffic carried over a RB configured for SDT.

Proposal 4.2.	gNB informs the UE when MT-SDT traffic is detected for the given UE via paging.

Proposal 4.3.	UE will monitor a MT-SDT indication sent via paging (e.g. when defined in specification or configured by the network depending on how the feature is specified).

Proposal 4.4.	UE initiates resume for SDT upon detecting a MT-SDT indication for the given UE.

MT-SDT triggering impacts on network

Proposal 5.	It is left up to gNB implementation when/how to detect/trigger that MT-SDT traffic is available.

Proposal 6.	Anchor gNB informs other gNBs in the UE’s configured RNA when MT-SDT traffic is detected for that given UE to trigger a Page for MT-SDT. Inform RAN3 on potential impact to Xn interface from this. Note: this impact would only apply to MT-SDT with RA-SDT (i.e. not to CG-SDT).

Proposal 7.	When initiating SDT after UE detects MT-SDT being available for the given UE, RAN2 needs to discuss:

Proposal 7.1.	How UE performs initial access upon detecting MT-SDT notification from gNB.

Proposal 7.2.	Whether/how gNB is aware that this UE is initiating resume for MT-SDT (i.e. without any UL SDT traffic).

Ongoing UL/DL SDT traffic after MT-SDT initiation

Proposal 8.	Further impacts for RAN2 to evaluate considering that UE may initiate RA-SDT or CG-SDT in response to MT-SDT:

Proposal 8.1.1.	If UE initiates RA-SDT, which RA procedure is initiated (e.g. RA-SDT vs non-RA-SDT) is selected to initiate RA-SDT due to MT-SDT.

Proposal 8.1.2.	If UE initiates CG-SDT, whether CG-SDT resources are used to initiate CG-SDT due to MT-SDT.

# Reference

- RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, WID, Dec. 2021.

- TS 36.300, Overall description, LTE, V17.2.0 (2022-10).

- TS 36.331, Radio Resource Control (RRC) LTE, V17.2.0 (2022-10).