3GPP TSG-RAN WG2 Meeting #120					R2-2212209

Toulouse, France, November 2022

Agenda Item:	8.11.2

Source:	AT&T, FirstNet

Title:	Service expectations for Multicast Sessions in RRC_INACTIVE

Document for:	Discussion/Approval

# Introduction

In the discussion regarding the RAN2 reply to [1], it became clear that there are differing views regarding service expectations for Multicast reception in RRC_INACTIVE state. One view observes that the absence of H-ARQ disadvantages Flows in RRC_INACTIVE state, suggesting that the corresponding performance expectations should be lower. This view also notes that seamless/lossless mobility is not required in RRC_INACTIVE state. A second view observes that the mapping of MBS QoS Flows are to QoS Profiles is independent of RRC state, so no difference in performance should be expected.

In this contribution, we examine these views and propose an agreement for aligning the two views expressed herein.

# Multicast performance in RRC_INACTIVE state

The question in [1] prompting this discussion was

Q1: SA2 would also like to understand:

- If there are significant differences in the quality and reliability of the reception of MBS data between UEs in RRC Connected state and UEs in RRC Inactive state

…

This question was first answered in the affirmative, noting 1) the absence of H-ARQ; and 2) the statement that seamless/lossless mobility is not required for UEs in RRC_INACTIVE state.

Hybrid ARQ (H-ARQ) is one of many well-accepted techniques for improving the reliability of a data channel in an efficient manner. The improvement is often noted as existing at the expense of increased latency. Many Multicast use cases are latency-tolerant, so the resulting improvement in reliability and efficiency are generally viewed as positive enhancements. However, the degree of enhancement is dependent on the specific use case and should therefore be considered on a case-by-case basis.

H-ARQ feedback for Multicast was introduced in Rel-17. The introduction was justified as having obvious benefits with minor specification impact [2]. Note that Multicast is supported in EUTRA without support for H-ARQ. Although not specified, it may be assumed that EUTRA implementations employed mechanisms other than H-ARQ to ensure reliable delivery of Multicast service (e.g., less aggressive MCS).

Observation 1: H-ARQ is a desirable but not essential mechanism for providing Multicast Service

We further note that the Multicast channel is coded using G-RNTI or G-CS-RNTI. The RNTI specific to the Multicast Session is provisioned by RRC and is available to the UE in RRC_CONNECTED state and remains valid for the Multicast Session after the UE transitions to RRC_INACTIVE state. If UEs in RRC_CONNECTED transmit HARQ-NACK feedback, the subsequent retransmissions will be coded by the same RNTI as the original transmissions. Therefore, an incremental retransmission benefit will be available to all UEs within a coverage area, including both UEs in RRC_CONNECTED and RRC_INACTIVE states.

Observation 2: An incremental H-ARQ benefit may exist for all UEs within a coverage area, including both UEs in RRC_CONNECTED and RRC_INACTIVE states.

Any benefit that H-ARQ would provide would have to be weighed against other service expectations. As an example, a Mission Critical use case may require a large number of UEs to receive a Multicast session within a coverage area, the number being larger than the number of UEs that a gNB would be able to maintain in RRC_CONNECTED state. In such a use case, the incremental benefit of H-ARQ could be traded for other, possibly less spectrally efficient means for maintaining channel reliability if the end result is that more UEs would be able to reliably receive the Multicast session.

Observation 3: Mechanisms other than H-ARQ can be employed to achieve reliability targets for Multicast Sessions.

Regarding the mobility exclusion cited earlier (seamless/lossless mobility is not required for UEs in RRC_INACTIVE), we understand that this exclusion is necessary due to the possibility that a mobility event may require a new UE configuration, and that transfer is most efficiently performed in RRC_CONNECTED state. While we believe that this reconfiguration could potentially be performed using group communication, we acknowledge that performing the reconfiguration using group resources may be an inefficient use of these resources and that it may be preferrable to require a UE to transition to RRC_CONNECTED state to perform the needed mobility functions. We leave the best mechanism for this state transition as FFS.

Observation 4: Mobility events can be performed efficiently by returning RRC_INACTIVE UEs to RRC_CONNECTED state.

Based on our observations, we offer the following proposals:

Proposal 1: RAN2 will begin a discussion of service expectations for UEs in RRC_INACTIVE state

Proposal 2: Service expectations during mobility events are not considered for UEs in RRC_INACTIVE state

# Conclusion

This contribution offers the following observations and proposals:

Observation 1: H-ARQ is a desirable but not essential mechanism for providing Multicast Service

Observation 2: An incremental H-ARQ benefit may exist for all UEs within a coverage area, including both UEs in RRC_CONNECTED and RRC_INACTIVE states.

Observation 3: Mechanisms other than H-ARQ can be employed to achieve reliability targets for Multicast Sessions.

Observation 4: Mobility events can be performed efficiently by returning RRC_INACTIVE UEs to RRC_CONNECTED state.

Proposal 1: RAN2 will begin a discussion of service expectations for UEs in RRC_INACTIVE state

Proposal 2: Service expectations during mobility events are not considered for UEs in RRC_INACTIVE state

# Reference

[1] R2-2209356, LS on FS_5MBS_Ph2 progress (S2-2207470; contact: Huawei)

[2] R1-2007341, Summary#3 on NR Multicast and Broadcast Services, Moderator (CMCC)