3GPP TSG-RAN WG2 #120 	R2-2212238

Toulouse, France, 14th –18th November 2022

Agenda Item:	8.6.3.1 Enhancements for neighbor cell measurements

Source:	NEC

Title:	Enhancements for neighbor cell measurements

Document for:	Discussion, Decision

# Introduction

In this contribution, we share our view on Neighbour cell measurement enhancement aspects

# Discussion

## Neighbour cell measurement enhancement for eMTC

The objective of IoT NTN (Non-Terrestrial Networks) enhancements is copied as below,

4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

The first sub-bullet is about enhancement on Neighbour cell measurement, even though eMTC is also mentioned, it is clear this is about neighbour cell measurement trigger before RLF.

Regarding RLF, only in case of NB-IOT, there is predicable RLF e.g., at the time when serving cell stop to serve the coverage area or when UE is moving too far away from cell centre, because there is no support of HO, RLF will happen. Hence network can configure time or location-based trigger for neighbour cell measurement before RLF. Differently from NB-IOT case, HO /CHO is supported for eMTC, then RLF for eMTC only covers unpredictable radio link issues, hence there is no way network could configure neighbour cell measurement trigger before RLF, instead we use the cell stop time and distance condition for CHO configuration. So in our understanding that triggering neighbour cell measurement before RLF is intended for NB-IoT but not for eMTC.

Objective of Support of neighbour cell measurements and corresponding measurements triggering before RLF is intended for NB-IoT but not for eMTC

- Some companies say eMTC UE should starts neighbour cell measurement at T-service or when its distance to serving cell above a threshold, but this is covered by eMTC CHO enhancement.

- Observation 2 triggering neighbour cell measurement when serving cell is about to swich off or when UE is moving away from serving cell for eMTC is covered by CHO/measurement configuration enhancement

- Another proposal is to enhance S_measure , which is configured to allow UE having very good RSRP not to measure neighbour cells. however, enhancement on S-measure is not even agreed or discussed in NR NTN, we do not think it is fundamentally important, since for NTN cell,  it seems better for UE always measure neighbouring cell as CHO/measurement configuration.

- Observation 3 there is no need to enhance S_ measure for eMTC over NTN as same for NR NTN

No need to support neighbour cell measurements and corresponding measurement triggering before RLF for eMTC over NTN

triggering neighbour cell measurement when serving cell is about to swich off or when UE is moving away from serving cell for eMTC should be discussed in the  CHO/measurement configuration enhancement topic

## Neighbour cell measurements before RLF for NB-IOT

    - RSRP-based trigger

From 36.331, the configuration of RSRP-based neighbor cell measurement before RLF for NB-IoT is copied as below:

-- ASN1START SystemInformationBlockType3-NB-r13 ::= SEQUENCE { cellReselectionInfoCommon-r13 SEQUENCE { //the unrelated part is skipped// ]], [[ connMeasConfig-r17 ConnMeasConfig-NB-r17 OPTIONAL, -- Need OR t-Service-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR ]] } //the unrelated part is skipped// ConnMeasConfig-NB-r17 ::= SEQUENCE { s-MeasureIntra-r17 NRSRP-Range-NB-r14, s-MeasureInter-r17 NRSRP-Range-NB-r14 OPTIONAL, -- Need OP neighCellMeasCriteria-r17 SEQUENCE { s-MeasureDeltaP-r17 ENUMERATED {dB6, dB9, dB12, dB15}, t-MeasureDeltaP-r17 ENUMERATED {s15, s30, s45, s60} } OPTIONAL -- Need OR } -- ASN1STOP

- It seems common understanding that RSRP-based measurement trigger will not be used in NTN scenario, connMeasConfig-r17 above is optional IE anyway, so it could be left to NW whether to configure it:

for NB-IoT, it is up to NW whether to configure Rel-17 NRSRP-based trigger for neighboring measurement in RRC-connected mode.

    - Time-based trigger

- In RAN2#119e meeting, we made following agreement

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- Time-based trigger has been agreed for both earth-moving and (quasi)earth fixed cell. however, we notice there are two different time-based triggers during discussion:

- First one is the fixed/cell specific time-based trigger, which is linked to T-service. the event of serving cell stop covering current area can happen not only for quasi earth-fixed cell, but also for earth moving cell. because according to TR38.821, feeder link switch is to switch the feeder link (SRI) between different NTN GWs toward the same satellite. This may be due to e.g. maintenance, traffic offloading, or (for LEO) due to the satellite moving out of visibility with respect to the current NTN GW. These reasons for feeder link switch are valid not only for quasi- earth fixed cell but also for earth moving cell. moreover, T-Service-r17 is already included in system information and can be informed to UE. there is no need of extra signalling. UE behaviour can be further specified.

For NB-IOT, T-Service could be reused by UE to trigger neighbor cell measurement (i.e., no need of extra signaling), UE behavior can be clarified in spec implementation stage

For NB-IOT, triggering neighbor cell measurement based on T-service is applicable to both quasi-earth fixed cell and earth-moving cell scenarios

- Second one is the UE-specific time-based trigger. This is only for earth moving cell, where different UEs will move out of the serving cell coverage at different time. it might be possible for UE to calculate when it will be out of coverage, however, it seems more straight forward to use location-based trigger in this case

For NB-IOT, UE-specific time -based trigger of neighbour measurement is not introduced

- For NB-IOT, UE should keep the connection to serving cell as long as possible, hence, as long as serving cell is available, there is no need to trigger RLF, we see no reason to use the neighbour cell time information (start time) to trigger measurement before RLF

For NB-IOT, no need to use the start time of neighbor cell for neighbor cell measurement enhancement

    - Location-based trigger

- As we discuss in previous section, location-based trigger would be useful especially for earth moving cell, but it is also useful for earth-fixed cell to cover the case where UE moves out of the coverage.

for NB-IOT, agree to introduce location-based trigger of connected mode measurement for both earth fixed and earth moving cell

    - Combination of trigger types

- In our understanding, RSRP-based trigger might be not used, and time-based and location-based triggers are for different use cases, then can be used independently.

All trigger types (time-based, location-based and legacy RSRP-based) for neighbour cell measurement before RLF are used independently

# Summary

Regarding the objective “Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel 17 (TN) NB-IoT, eMTC as a baseline”, we discuss some details and propose:

For eMTC over NTN:

- Objective of Support of neighbour cell measurements and corresponding measurements triggering before RLF is intended for NB-IoT but not for eMTC

- Observation 2 triggering neighbour cell measurement when serving cell is about to swich off or when UE is moving away from serving cell for eMTC is covered by CHO/measurement configuration enhancement

- Observation 3 there is no need to enhance S_ measure for eMTC over NTN as same for NR NTN

- No need to support neighbour cell measurements and corresponding measurement triggering before RLF for eMTC over NTN

triggering neighbour cell measurement when serving cell is about to swich off or when UE is moving away from serving cell for eMTC should be discussed in the CHO/measurement configuration enhancement topic

For NB-IOT over NTN:

for NB-IoT, it is up to NW whether to configure Rel-17 NRSRP-based trigger for neighboring measurement in RRC-connected mode.

For NB-IOT, T-Service could be reused by UE to trigger neighbor cell measurement (i.e., no need of extra signaling), UE behavior can be clarified in spec implementation stage

For NB-IOT, triggering neighbor cell measurement based on T-service is applicable to both quasi-earth fixed cell and earth-moving cell scenarios

For NB-IOT, UE-specific time-based trigger of neighbour measurement is not introduced

For NB-IOT, no need to use the start time of neighbor cell for neighbor cell measurement enhancement

for NB-IOT, agree to introduce location-based trigger of connected mode measurement for both earth fixed and earth moving cell

All trigger types (time-based, location-based and legacy RSRP-based) for neighbour cell measurement before RLF are used independently