3GPP TSG-RAN WG2 Meeting #120	R2-2212267

Toulouse, France, 14th – 18th of November 2022

Agenda item:	8.8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	SA2 Status for Uncrewed Aerial Vehicles in Rel-18

WID/SID:	NR_UAV - Release 18

Document for:	Information

# 1	Introduction

In December 2021 RAN#94 has approved Rel-18 work on NR UAVs, see [1]. Among the other, the following objective was listed, pending the progress in SA2:

3. Study and specify, if needed, enhancements for UAV identification broadcast [RAN2, SA2]. Applicable to both LTE and NR Note: This study should consider existing techniques based on Uu or non 3GPP technologies, or unlicensed band as the baseline. Note: This description is a placeholder for a more detailed objective to be drafted once SA2 will have concluded their study on the architectural aspects.

As SA2 has reached the conclusions regarding the key issues (KIs) they have identified during the study, we provide those conclusions in RAN2. Those shall be considered for potential WID updates at RAN#98 (December 2022).

# 2	Key Issues and Conclusions Made by SA2

SA2 has analysed the following three KIs in their study [2]:

- Key Issue #1 - Transport C2 communication over PC5 interface

- Key Issue #2 - Support of Broadcast Remote ID

- Key Issue #3 - Support of Detect and Avoid Mechanism in 3GPP system

As can be seen above, KI#2 is the one that matches the objective from [1].

Observation 1: SA2’s Key Issue #2 is the one that matches Objective 3 from NR UAV WID in RP-213600.

Nevertheless, below we provide the conclusions for each of the KIs listed above.

For Key Issue #1 the following has been concluded [2]:

The UAV engaged in C2 communication over PC5 may or may not be capable of Uu communication with the network. Only Unicast mode C2 communication over PC5 is supported in this release. For C2 communication over PC5, the UAV-C may be pre-paired or dynamically paired. Two types of authorization are supported for C2 communication over PC5: - Authorization based on provisioned policy in UAV similar to clause 5.1.3 of TS 23.304 [8]. - Authorization based on C2 communication authorization procedure defined in TS 23.256 [5] if the UAV is capable of Uu communication with the network. In this case the UAV UE performs C2 communication authorization following the procedure define in TS 23.256 [5], before initiating UAV to UAV-C direct communication over PC5. The Unicast mode 5G Prose Direct Communication procedure as defined in clause 6.4.3 of TS 23.304 [8] is used as the baseline for establishing C2 communication over PC5, with the following enhancements and adaptations: - Prose Service info is replaced with "C2 Communication Service". - C2 Communication Service identifier may be pre-configured or derived from the CAA-level UAV ID. Both UE-oriented and service-oriented unicast link establishment are supported as defined in clause 6.4.3.1 of TS 23.304 [8].

For Key Issue #2 the following has been concluded [2]:

It is proposed to progress to normative phase the following: - a mechanism based on PC5 leveraging U2X (UAV-to-everything) as described by the architectural enhancements identified in solution #5, applicable to both UAV UE that registers to the MNO network, and to UAVs that operate out of coverage - for scenarios that UAV UEs in coverage and register to the network, and to UAV-C that are 3GPP UEs, a mechanism using MBS location dependent broadcast service, based on solution #3. It is assumed that security of the U2X solution will be addressed by SA WG3. NOTE 1: It is expected that SA WG3 will perform a security analysis of solution #5. It is assumed that a combination of U2X and network-based solutions can be deployed. NOTE 2: It is expected that RAN will define necessary enhancements to support PC5 for NR as described in solution #5.

For Key Issue #3 the following has been concluded [2]:

It is proposed to progress to normative phase the following: - a mechanism for DAA leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution #5, with the following principles: - The detection and resolution of collisions is locally performed between UAVs using direct UAV to UAV communication over PC5. - The USS can (optionally) be informed of the collision situation. NOTE 1: The UAV may inform the USS of the collision situation over application layer communication. No normative work in SA WG2 is expected to enable this. - Both unicast and broadcast mode direct communication over PC5 is supported for DAA. - Additionally, may support network-assisted (ground based) DAA solution #7. It is applicable for a specific area, such as a stadium or arena where drones are used. NOTE 2: The content of "local policies for DAA" in Solution #7 and how it is communicated between UAV, UAV-C and AAM is out of 3GPP scope. NOTE 3: Whether the UAV UEs served by different PLMNs and with subscriptions to different PLMNs is supported depends on the Rel-18 NR sidelink RAN study conclusion. It is assumed that security of the U2X solution will be addressed by SA WG3.

As can be seen above, for KI#2, the SA2’s expectation is that the UAV UE supports PC5 for the purpose of broadcasting remote ID.

Observation 2: SA2 expects the UAV UE supports PC5 for the purpose of remote ID broadcasting.

The rapporteur has submitted this document just for information, while a detailed discussion on the NR UAV scope definition is expected to occur at RAN#98.

Proposal 1: If necessary, Rel-18 NR UAV WID is updated at RAN#98 (December 2022), considering the SA2 conclusions available in TR 23.700-58.

# 3	Conclusion

This document shares the recent conclusions reached by SA2 on NR UAV support. The following observations and proposals have been made:

Observation 1: SA2’s Key Issue #2 is the one that matches Objective 3 from NR UAV WID in RP-213600.

Observation 2: SA2 expects the UAV UE supports PC5 for the purpose of remote ID broadcasting.

Proposal 1: If necessary, Rel-18 NR UAV WID is updated at RAN#98 (December 2022), considering the SA2 conclusions available in TR 23.700-58.

# References

- RP-213600 New WID on NR support for UAV (Uncrewed Aerial Vehicles) 3GPP TSG RAN Meeting #94e Electronic Meeting, Dec. 6 - 17, 2021

- 3GPP TR 23.700-58 Study of further architecture enhancements for uncrewed aerial systems and urban air mobility, v1.1.0, November 2022