3GPP TSG-RAN2 Meeting #120	R2-2212302

Toulouse, France, 14-18 November 2022	Revision of R2-2210710

Agenda Item:	5.1.1 Stage 2 and Organisational

Source: 	Ericsson, Ericsson, Nokia (Rapporteur)

Title:  	Enhancements of Public Warning System

Document for:	Discussion and Decision

# Introduction

In this contribution enhancements of Public Warning System (ePWS) are discussed.

# Background

ePWS

In Rel-16 enhancements of Public Warning System (ePWS) were introduced which enable [1-8]:

- Language-independent content (e.g. pictogram mapped to a disaster)

- Notifications to UEs with no user interface or with a user interface that is incapable of displaying text (e.g. IoT device)

A separate Message Identifier range (4401-4422) has been defined for ePWS warnings, i.e. see non-ETWS (CMAS) and ETWS warning messages with ePWS functionality in 23.041.

38.300

In section 16.4 “Public Warning System” a high level description of PWS is provided. For example it is clarified that KPAS and EU-Alert use the same AS mechanism as CMAS:

NR connected to 5GC provides support for public warning systems (PWS) through means of system information broadcast capability. NR is responsible for scheduling and broadcasting of the warning messages as well as for paging the UE to provide indication that the warning message is being broadcast:

-	Earthquake and Tsunami Warning System: ETWS is a public warning system developed to meet the regulatory requirements for warning notifications related to earthquake and/or tsunami events (see TS 22.168 [14]). ETWS warning notifications can either be a primary notification (short notification) or secondary notification (providing detailed information).

-	Commercial Mobile Alert System: CMAS is a public warning system developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [15]).

Different SIBs are defined for ETWS primary notification, ETWS secondary notification and CMAS notification. Paging is used to inform UEs about ETWS indication and CMAS indication (see clause 9.2.5). UE monitors ETWS/CMAS indication in its own paging occasion for RRC_IDLE and for RRC_INACTIVE while no SDT procedure (see clause 18.0) is ongoing. UE monitors ETWS/CMAS indication in any paging occasion for RRC Connected and during the SDT procedure in RRC_INACTIVE. Paging indicating ETWS/CMAS notification triggers acquisition of system information (without delaying until the next modification period).

KPAS and EU-Alert are public warning systems developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [15]). KPAS and EU-Alert uses the same AS mechanisms as CMAS. Therefore, the NR procedures defined for CMAS equally apply for KPAS and EU-Alert.

# Discussion

An ETWS warning message with ePWS functionality is either mapped to SIB6 or SIB7. And a CMAS warning message with ePWS functionality is mapped to SIB8. Furthermore the same monitoring, re-assembly and forwarding rules apply for warning messages with ePWS functionality:

Observation: The ETWS/CMAS AS mechanisms are re-used for ETWS/CMAS warning messages with ePWS functionality.

Similar as has been done for EU-Alert/KPAS it would be good to:

Proposal 1: Clarify in 38.300 that ETWS/CMAS warning messages with ePWS functionality use the same AS mechanisms as ETWS/CMAS.

Proposal 2: RAN2 to discuss for which release to introduce this clarification.

A TP for 38.300 is provided in the Annex for further discussion.

# Summary

RAN2 is kindly asked to discuss enhancements of Public Warning System (ePWS):

Observation: The ETWS/CMAS AS mechanisms are re-used for ETWS/CMAS warning messages with ePWS functionality.

Proposal 1: Clarify in 38.300 that ETWS/CMAS warning messages with ePWS functionality use the same AS mechanisms as ETWS/CMAS.

Proposal 2: RAN2 to discuss for which release to introduce this clarification.

# References

- S1-180584, Requirements for 3GPP system supporting ePWS-UEs, SA1, CR 22.268

- S1-180585, Definition of ePWS-UE and abbreviation of ePWS, SA1, CR 22.268

- S1-180586, Background of ePWS-UE requirements, SA1, CR 22.268

- S1-180587, Requirements for ePWS-UE with different or no user interface or with different roles, SA1, CR 22.268

- S1-180588, Requirements for the improvement of understanding the PWS message, SA1, CR 22.268

- S1-180589, Addition of NOTE in legacy requirements for the improvement of understanding the PWS message, SA1, CR 22.268

- C1-199011, Addition of the support of ePWS functionality via E-UTRAN and NG-RAN, CR 23.041

- C1-199012, Support of language-independent content mapped to a disaster in a warning message, CR 23.041

# Annex: TP 38.300

# 2	References

<TEXT OMITTED>

[x]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

<TEXT OMITTED>

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ePWS	enhancements of Public Warning System

ETWS	Earthquake and Tsunami Warning System

<TEXT OMITTED>

PWS	Public Warning System

<TEXT OMITTED>

## 16.4	Public Warning System

NR connected to 5GC provides support for public warning systems (PWS) through means of system information broadcast capability. NR is responsible for scheduling and broadcasting of the warning messages as well as for paging the UE to provide indication that the warning message is being broadcast:

-	Earthquake and Tsunami Warning System: ETWS is a public warning system developed to meet the regulatory requirements for warning notifications related to earthquake and/or tsunami events (see TS 22.168 [14]). ETWS warning notifications can either be a primary notification (short notification) or secondary notification (providing detailed information).

-	Commercial Mobile Alert System: CMAS is a public warning system developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [15]).

Different SIBs are defined for ETWS primary notification, ETWS secondary notification and CMAS notification. Paging is used to inform UEs about ETWS indication and CMAS indication (see clause 9.2.5). UE monitors ETWS/CMAS indication in its own paging occasion for RRC_IDLE and for RRC_INACTIVE while no SDT procedure (see clause 18.0) is ongoing. UE monitors ETWS/CMAS indication in any paging occasion for RRC Connected and during the SDT procedure in RRC_INACTIVE. Paging indicating ETWS/CMAS notification triggers acquisition of system information (without delaying until the next modification period).

Enhancements of public warning system (ePWS) enable broadcast of language-independent content (e.g. pictogram mapped to a disaster) and notifications to UEs with no user interface or with a user interface that is incapable of displaying text (e.g. IoT device), see clause 9 in TS 22.268 [15] and TS 23.041 [x]. ETWS/CMAS notifications with ePWS functionality use the same AS mechanisms as ETWS/CMAS.

KPAS and EU-Alert are public warning systems developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [15]). KPAS and EU-Alert uses the same AS mechanisms as CMAS. Therefore, the NR procedures defined for CMAS equally apply for KPAS and EU-Alert.