3GPP RAN WG2 Meeting #120	R2-2212340

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.8.2

Source:	InterDigital

Title:	Flight path reporting for UAV

Document for:	Discussion, Decision

# Introduction

A new work item on NR support for aerial UEs (UAVs) was agree for Rel-18. Since aerial UEs are supported via LTE as from Rel-15, work done in LTE is a starting point for this objective. Although NR-specific enhancements can be considered if needed, overall the LTE and NR solutions should be harmonized as much as possible.

The following objective has been provided regarding measurement reporting for aerial UEs in NR:

- Specify the following enhancements on measurement reports [RAN2]:

  - UE-triggered measurement report based on configured height thresholds

  - Reporting of height, location and speed in measurement report

  - Flight path reporting

  - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

RAN2#119e [6] has captured the following agreements relevant to flight path reporting for aerial UAVs:

- As in LTE, flight path plan reporting will be introduced.  Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report.  FFS if timestamp is mandatory or optional for NR.  FFS if further enhancements are needed

And the following relevant agreements are captured in RAN2#119bis-e [7]:

- The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE.  FFS on flight path details (waypoints and what is time information).

- Allow the flight path to be updated.  FFS on the details.

- FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting

This document continues the discussion on flight path reporting for UAVs, specifically the content and reporting procedure.

# Flight path reporting for UAV

## Flight path report content

The basic content of a flight path report agreed in NR is a location list of waypoints (3D location information) and optional timestamp information (if configured by the network and available at the UE). Details on waypoints, what is time information, and what information to report is currently listed for further study.

Waypoint definition

In LTE, a waypoint describes planned location(s) for the UE along the flight path. The locations are reported via the LocationInfo IE, which allows the UE to provide location coordinates corresponding to a 3D location in space as defined by TS 36.355.

The LocationInfo IE is also supported in NR, along with the ability to transmit 3D location coordinates (e.g. via CommonLocationInfo -> locationCoordinate) defined by the updated TS 37.355. Considering LTE and NR should be harmonized as much as possible (as per the WID) and a similar framework is already defined in NR to report a 3D location in space, to minimize specification effort it is proposed the same framework is used.

Proposal 1:	A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

Timestamp definition

An additional benefit of describing waypoints via locationCoordinate in NR is that there is already an accompanying locationTimestamp defined, which provides the UTC time when the corresponding location estimate is valid. Considering this form is similar to the LTE definition of timestamp information (i.e. YYMMDDhhmmss) this seems a suitable way to describe time information with minimal specification effort.

Proposal 2:	A timestamp provides the UTC time when the associated waypoint is valid.

Distribution of waypoints

It has been discussed whether restrictions should be placed on how a UE can report a waypoint, for example, how waypoints are to be distributed in space. During flight a UE may remain in a place for some time. Assuming LTE definition of timestamps is adopted for NR, a UE cannot explicitly indicate a duration of how long it will remain in a specific location.

However, allowing a UE to report the same waypoint location but with different timestamp can imply the UE will remain in a location for an extended duration. Even without timestamp information, a UE reporting multiple identical waypoints can be inferred as ‘the UE will remain in one place for a while’. Such information is useful, and it is proposed no additional restrictions are adopted.

Proposal 3:	No requirements are placed on spatial distribution of waypoints.

## Flight path reporting procedure

Based on agreements from RAN2#119bis-e, details on initial flight path reporting and how to subsequently update a flight path report are currently for further study and to be addressed in RAN2#120.

Indication of flight path availability

The first step of the flight path reporting procedure is to notify the network flight path information is available. In LTE, a UE indicates flight path information is available via the RRCConnectionReconfigurationComplete, RRCConnectionReestablishmentComplete, RRCConnectionResumeComplete, or RRCConnectionSetupComplete messages. This allows the network to know immediately after connection establishment whether flight path information is available, enabling subsequent flight path report configuration and request. As a baseline, a similar mechanism can be confirmed for NR.

Proposal 4:	A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message

Flight path reporting procedure

E-UTRAN can request a UE to report flight path information via flightPathInfoReq in the UEInformationRequest message. Upon request to report UE flight path information, the UE will include flightPathInfoReport in the UEInformationResponseMessage including all available waypoints up to the configured maximum. Such information is useful to the network for e.g. collision avoidance, resource provisioning, and UE configuration.

Use of the UE information request/response procedure means that flightpath reporting is under NW control, which is useful to avoid frequent flightpath reporting and interference. A similar procedure may be used as a baseline in NR.

Proposal 5:	Flight path reporting uses the UE Information request/response procedure as baseline.

Updating a previously reported flight path

If a UE does need to deviate from the planned flightpath due to collision avoidance, there may be need to notify the network. It was agreed in RAN2#119bis-e that a flight path can be updated, which is a new enhancement for NR.

One way to report updated flight path information is to simply allow the UE to autonomously report an updated flight path. However, since the initial flight path reporting relies on NW request, a new signalling procedure would need to be defined considering e.g. potential security impacts of exposing current/future UE locations, resource utilization, and interference.

Alternatively, a UE may simply indicate a previously reported flight path needs to be updated, and the NW can decide whether/when to request an updated flight path report. This would follow a similar reporting procedure as the initial flight path report, minimizing specification effort and potential interference issues. Details regarding the indication can be further discussed by RAN2.

Proposal 6:	UE does not autonomously report an updated flight path. The UE indicates the previously reported flight path is outdated, and the network can request an updated flight path via the UE Information Request procedure.

# Conclusion

In this contribution the following proposals are made concerning flight path reporting for aerial UEs in NR:

Proposal 1:	A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

Proposal 2:	A timestamp provides the UTC time when the associated waypoint is valid.

Proposal 3:	No requirements are placed on spatial distribution of waypoints.

Proposal 4:	A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message

Proposal 5:	Flight path reporting uses the UE Information request/response procedure as baseline.

Proposal 6:	UE does not autonomously report an updated flight path. The UE indicates the previously reported flight path is outdated, and the network can request an updated flight path via the UE Information Request procedure.

# References

RP-21360 – WI on NR support for UAV

3GPP TS 36.300 – Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Stage 2 v17.1.0

3GPP TS 36.311 – EUTRA Radio Resource Control (RRC) protocol specification v17.1.0

3GPP TS 38.331: Radio Resource Control (RRC) protocol specification v17.1.0

3GPP TS 36.331 – LTE positioning Protocol (LPP) v17.1.0

R2-2209301 – RAN2#119e Meeting Report

Draft_R2_119bis-e_meeting_report_v1