3GPP TSG-RAN WG2 #120	R2-2212358

Toulouse, FR, November 14th – 18th, 2022

Agenda Item:	8.2.3

Source:	Ericsson

Title:	Text proposal and Signaling for Integrity Computation at LMF

Document for:	Discussion, Decision

# 1	Introduction

The following objective was agreed for the study of integrity support for RAT dependent positioning:

- Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

Integrity for RAT independent positioning has been supported by 3GPP since release 17, specifically for GNSS / RTK.  In this paper, we give our view on the possible LPP signaling for LMF-mode RAT dependent positioning.

# 2	Discussion

In RAN2#119bis-e, the following was agreed:

Agreements:

Proposal 1-2. RAN2 study the usage of DNU flag for the RAT-dependent positioning integrity (assuming RAN1 agree to leave it to RAN2) and conclude on whether to indicate the DNU presence in the integrity principle equation.

Proposal 4. RAN2 will study the both UE-based and LMF-based integrity for RAT-dependent cases.

Proposal 7 (modified). RAN2 agree that R17 UE-based integrity mode signaling can be used as baseline with the following aspects:

-	UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure

-	LMF sends the assistance data for integrity calculation to UE for integrity of UE-based mode

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode

-	UE sends integrity result to LMF using LPP location information Transfer message

Agreement:

LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

In RAN#110b-e, the following was agreed:

Agreement - Timing measurement error can be modeled as Normal distribution. - Note: The timing measurement is applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurement - Note: it is assumed that the timing measurement error is associated with the first path - Note: it is up to RAN2 how to use the identified distribution Agreement - In the agreement on the distribution of the timing measurement error, it is assumed that the timing measurement error contains TEG related TX/RX timing error if the TEG related information is provided. - Note: The timing measurement is applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurement - Note: it is assumed that the timing measurement error is associated with the first path - Note: no more discussion on TEG related TX/RX timing error as an independent error source from timing measurement error Agreement - Study to determine whether SFN initialization time is an independent error source for the following positioning methods and integrity mode - UL-TDOA with LMF-based positioning integrity mode - UE-assisted DL-TDOA with LMF-based positioning integrity mode - FFS: Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Note: Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

In RAN#110, the following was agreed:

Agreement

•	For LMF-based positioning integrity mode, at least the followings are error sources for timing related measurements :

o	RSTD measurement is an error source for DL-TDOA

o	RTOA measurement is an error source for UL-TDOA

o	UE Rx-Tx time difference measurement is an error source for Multi-RTT

o	gNB Rx-Tx time difference measurement is an error source for Multi-RTT

•	FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range)

•	Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

RAN1 agreed to reuse the positioning integrity modes from the RAT independent positioning:

UE-based positioning integrity mode: Positioning integrity result is derived by the UE.

LMF-based positioning integrity mode: Positioning integrity result is derived by the LMF.

In RAN2#119bis-e meeting, RAN2 agreed to study the both UE-based and LMF-based integrity for RAT-dependent cases, while only signalling for UE-based integrity mode were discussed and agreed. The signalling for LMF-based integrity should also be discussed.

For LMF-based positioning integrity mode, RSTD measurement is an error source for DL-TDOA and UE Rx-Tx time difference measurement is an error source for Multi-RTT. In RAN2, while waiting for conclusion about error source from RAN1, we may also discuss required UE capability and signalling for UE to indicate the mean and standard deviation of the error sources.

For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects:

- UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

- LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

- UE performs positioning measurements and computes the positioning measurement error (same as legacy)

- UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

- LMF computes the Integrity.

For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

- UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

- LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

- UE performs positioning measurements and computes the error (same as legacy)

- UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

- LMF computes the Integrity.

#### LMF requests UE to send error source model statistics

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source and such request can be sent in the RequestLocationInformation for each RAT positioning method.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

[[

nr-UE-RSTD-Error-Request-r18				ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested } 		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-UE-RSTD-Error-Request This field indicates whether the target device is requested to report error source statistics of the RSTD measurements.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1) } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

[[

nr-UE-RxTxTimeDiff-Error-Request-r18 ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Request This field indicates whether the target device is requested to report error source statistics of the UE Rx-Tx time difference measurements.

For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

#### UE send error source model statistics

For LMF-based positioning integrity mode, RSTD measurement is an error source for DL-TDOA and UE Rx-Tx time difference measurement is an error source for Multi-RTT. RAN1 has the agreement that timing measurement error can be modeled as Normal distribution. UE may send the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

#### –	nr-Timing-Error-Source

The IE nr-Timing-Error-Source characterizes the mean and standard deviation of a normal distribution.

-- ASN1START

nr-Timing-Error-Source-r18 ::= SEQUENCE {

mean 							INTEGER(FFS)

Standard-deviation      		INTEGER(FFS)

}}

-- ASN1STOP

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-RSTD-Error-Source-r18			nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

[[

nr-RSTD-ResultDiff-Error-Source-r18 nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-RSTD-Error-Source This field specifies the mean and standard deviation of the nr-RSTD for RAT-dependent positioning integrity.

nr-RSTD-ResultDiff-Error-Source This field specifies the mean and standard deviation of the nr-RSTD-ResultDiff for RAT-dependent positioning integrity.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiff-Error-Source-r18	nr-Timing-Error-Source-r18					OPTIONAL,

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16 				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

[[

nr-UE-RxTxTimeDiffAdditional-Error-Source-r18	nr-Timing-Error-Source-r18	OPTIONAL,

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-UE-RxTxTimeDiff-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiff for RAT-dependent positioning integrity.

nr-UE-RxTxTimeDiffAdditional-Error-Source This field specifies the mean and standard deviation of the nr-UE-RxTxTimeDiffAdditional for RAT-dependent positioning integrity.

For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects and agree to the text proposal as in Annex:

	UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

	LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

	UE performs positioning measurements and computes the error (same as legacy)

	UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

	LMF computes the Integrity.

Proposal 2	For LMF-based positioning integrity mode, LMF requests UE to send error source statistics of error source in the RequestLocationInformation for each RAT positioning method.

Proposal 3	For LMF-based positioning integrity mode, UE sends the error source statistics in the SignalMeasurementInformation message for the corresponding positioning method.

# References

RP-210903 , “Revised WID on NR Positioning Enhancements“, March 16 - 26, 2021

# Text proposal

For LMF-based integrity for RAT-dependent positioning, the R17 UE-assisted integrity mode signaling can be used as baseline with the following aspects:

- UE sends capability info to LMF on integrity for UE-Assisted mode using LPP capability transfer procedure

- LMF provides the Assistance Data for Positioning (same as legacy) and request for Integrity error sources

- UE performs positioning measurements and computes the positioning measurement error (same as legacy)

- UE generates error sources for the requested measurements using mean and standard deviation and provides to the LMF using LPP

- LMF computes the Integrity.