3GPP TSG-RAN WG2 #120	Tdoc R2-2212393

Toulouse, November 14 –18, 2022

Agenda Item:	8.3.5

Source:	Ericsson

Title:	Group handover for NW energy savings

Document for:	Discussion, Decision

# 1	Introduction

In RAN WG2 Meeting #119bis-e, the following scenarios were discussed:

=>	Scenario 1: UEs are HO’ed due to switch of SOURCE cell to NES mode is considered for further study.  FFS whether any enhancements is needed.

=>	FFS Scenario 2: UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation

=>	As a first priority, discussion on RAN2 group handover are confined to the CHO framework

This contribution discusses the FFSs captured above.

# 2	Discussion

It has been discussed that UEs may need to be handed over from a source cell to a target cell due to NW energy savings. For instance, the source cell may be switched off when in a low load scenario, which would require to handover UEs to other cells. It was raised that such handover could be performed via group signaling so that it can be performed in a faster manner while also reducing the signaling required to perform the handover of UEs.

However, for NW energy savings purposes, we do not expect that such handover of UEs would be performed frequently and dynamically. For instance, the NW should switch off a cell when it expects that it can sleep for a considerable amount of time on such cell. Hence, it should not be so time critical to handover UEs faster to another cell. Similarly, it would not be expected to handover UEs back to the cell in a time critical manner, otherwise the cell should not have been switched off.

For NW energy saving purposes, a more dynamic handover is not required.

Furthermore, if a low load scenario is translated to few UEs in RRC_CONNECTED in the cell, it should not be a burden for the NW to handover such UEs, which would not justify the group handover signaling. Even in the case where a low load scenario is translated to many UEs with small traffic, it should not be an issue to handover such UEs via dedicated manner to switch off the cell, since it should be expected that such cell can sleep for a long time.

Signaling load should be an issue to handover UEs for NW energy saving purposes.

Therefore, we think that there is no need for extra efforts that the specification of a group handover solution for NW energy saving purposes would impose. Instead, the existing dedicated signaling mechanisms already specified for legacy handover can be used in combination with NW energy saving purposes. It should also be noted that if there are legacy UEs in the cell then anyway those would have to receive the legacy handover.

If there are legacy UEs in the cell, the legacy handover procedure should anyway be used.

When handing over UEs due to switch of SOURCE cell to NES mode, the legacy signaling for handover can be used, no essential issue is identified.

Similarly, for scenario 2 captured as FFS in section 1 (i.e. UEs are HO’ed due to source link degradation, where TARGET cell is selected based on its mode of operation), the legacy signaling should also be sufficient, it should be up to the NW whether to reconfigure the UE in such cases, a reconfiguration should not be frequent given that it is not expected that cells would be switched on-off in such a dynamic manner. Hence we see no need to capture such scenario.

No RAN2 impact on the UE is expected for the scenario where handover is performed due to source link degradation, where target cell is selected based on its mode of operation.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For NW energy saving purposes, a more dynamic handover is not required.

Observation 2	Signaling load should be low to handover a small group of UEs. For a larger group of UEs, it may not be desirable to handover them for NW energy saving purposes.

Based on the discussion in the previous sections we propose the following:

Proposal 1	When handing over UEs due to switch of SOURCE cell to NES mode, the legacy signaling for handover can be used, no essential issue is identified.

Proposal 2	No RAN2 impact on the UE is expected for the scenario where handover is performed due to source link degradation, where target cell is selected based on its mode of operation.