3GPP TSG-RAN WG2 #120	Tdoc R2-2212418

Toulouse, France, 2022-11-14 - 2022-11-18

Agenda Item:	8.19

Source:	Ericsson

Title:	Analysis of alternatives for sending time synchronization status

Document for:	Discussion, Decision

# 1	Introduction

SA2 has sent the LS in S2-2209876. It asks RAN2 to provide feedback on the methods listed in Annex A of TR 23.700-25.

# 2	Discussion

SA2 mentions "time synchronization status information" but it is not clear to us exactly what "time synchronization status information" is or how large it is expected to be. We suggest asking SA2 to clarify this.

Ask SA2 to elaborate on what "time synchronization status information" is and the expected size.

Alternatives 3 and 4 (and Alternative 1b) are based on broadcasting "time synchronization status information". While it is unclear exactly what "time synchronization status information" and what it represents, we can conclude that these broadcast-based solutions cannot consider any status of UEs, only status of the network. Further, RAN cannot know which UEs are receiving the information and cannot know which services are used (or subscribed) by those UEs, hence the network cannot tailor the information for the receiving UEs.

In contrast to CONNECTED-mode based solutions: with broadcast-based solutions the "time synchronization status information" can only consider status of the network, not status of the UEs. Further, the network cannot tailor the information to particular (groups of) UEs.

Alternative 4, compared to Alternative 3, allows the operator to provide the time synchronization status information only to a certain (group(s) of) users, e.g. based on subscription. In case multiple tiers of users should be supported and the different tiers should receive different information, system information overhead would grow.

Encryption of the "time synchronization status information" allows the operator to provide the information to only certain (groups of) users.

We understand that Alternatives 1a and 2 brings UEs to CONNECTED-mode which enables to achieve the required accuracy or to determine cases where the required accuracy can no longer be achieved. Solution based on CONNECTED-mode not only provides better accuracy but can also distinguish different service levels and avoids broadcasting possibly security-sensitive information.

Timing information provided in CONNECTED-mode:

- can be more accurate than timing info provided in IDLE/INACTIVE-mode.

- can distinguish different service levels

- avoids broadcasting possible security-sensitive information

However, we see a risk with solutions like 1a and 2 since they might force many UEs to enter RRC CONNECTED mode simultaneously and cause overload of the RAN. And these solutions may cause UEs to enter CONNECTED after cell reselections to acquire the timing information.

Solutions which makes many UEs transition from IDLE/INACTIVE to CONNECTED simultaneously may cause overload in the RAN.

Based on the above, we think RAN2 should suggest SA2 to investigate a solution based on RRC Connected mode where high accuracy and service assurance is required and an IDLE/INACTIVE mode solution for other cases (if needed). But SA2 should avoid solutions that trigger many UEs to simultaneously transition from IDLE to CONNECTED.

Suggest to SA2 to:

- investigate a CONNECTED-based solution where high accuracy/service assurance is required and an IDLE/INACTIVE-based solution for other cases (if needed)

- avoid solutions that trigger many UEs to go from IDLE to CONNECTED at the same time.

A draft LS-text is found in the annex.

Reply SA2 based on the text in the Annex.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In contrast to CONNECTED-mode based solutions: with broadcast-based solutions the "time synchronization status information" can only consider status of the network, not status of the UEs. Further, the network cannot tailor the information to particular (groups of) UEs.

Observation 2	Encryption of the "time synchronization status information" allows the operator to provide the information to only certain (groups of) users.

Observation 3	Timing information provided in CONNECTED-mode:

-	can be more accurate than timing info provided in IDLE/INACTIVE-mode.

-	can distinguish different service levels

-	avoids broadcasting possible security-sensitive information

Observation 4	Solutions which makes many UEs transition from IDLE/INACTIVE to CONNECTED simultaneously may cause overload in the RAN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Ask SA2 to elaborate on what "time synchronization status information" is and the expected size.

Proposal 2	Suggest to SA2 to:

-	investigate a CONNECTED-based solution where high accuracy/service assurance is required and an IDLE/INACTIVE-based solution for other cases (if needed)

-	avoid solutions that trigger many UEs to go from IDLE to CONNECTED at the same time.

Proposal 3	Reply SA2 based on the text in the Annex.

# Annex

RAN2 thanks SA2 for the LS in S2-2209876. RAN2 has analysed the methods listed in Annex A of 23.700-25 from a RAN2 perspective. It is not clear to RAN2 exactly what "time synchronization status information" is. RAN2 wants SA2 to clarify this. Q1: What is "time synchronization status information"? Some of the solutions rely on broadcasting this information and hence overhead is something which should be considered, hence RAN2 wants SA2 to clarify expected size of "time synchronization status information". Q2: What is expected size of the "time synchronization status information"? RAN2 understands that Alternatives 1a and 2 brings UEs to RRC CONNECTED to achieve the required accuracy or to determine cases where the required accuracy can no longer be achieved. Alternatives 3 and 4 (and Alternative 1b), on the other hand are based on broadcasting "time synchronization status information". RAN2 wants to higlight that broadcast-based solutions cannot consider any status of UEs, only status of the network. Further, RAN cannot know which UEs are receiving the information and cannot know which services are used by those UEs, hence it cannot tailor the information for the receiving UEs. Alternative 4, compared to Alternative 3, allows the operator to provide the time synchronization status information only to a certain (group(s) of) users , e.g. based on subscription. In case there should be multiple tiers of users and the different tiers should receive different information, there would be multiple (encrypted) versions of "time synchronization status information", and hence sytem information overhead would grow. RAN2 generally agrees that a solution based on RRC CONNECTED (in contrast to solutions based on IDLE/INACTIVE) provides better accuracy (e.g. by propagation delay compensation), can distinguish different service levels and avoids broadcasting possibly security-sensitive information. However, RAN2 sees risks in solutions like 1a and 2 that might force many UEs to enter RRC CONNECTED mode simultaneously since this could cause overload of the RAN. And solutions relying on UEs entering RRC CONNECTED may cause UEs to enter CONNECTED after cell reselections. Hence, RAN2 suggests SA2 to investigate a solution based on RRC Connected mode where high accuracy and service assurance is required and an IDLE/INACTIVE mode solution for other cases (if needed). But SA2 should avoid solutions that trigger many UEs to simultaneously transition from IDLE to CONNECTED. Action to SA2: RAN2 asks SA2 to take the above information in to account and answer the questions.