3GPP TSG-RAN WG2 Meeting #120	R2-2212449

Toulouse, France, November 14 - 18, 2022

Agenda item:	8.7.4.2

Source:	Samsung

Title:	Discussion on NR NTN Handover Enhancement

WID/SID:	NR_NTN_enh - Release 18

Document for:	Discussion and Decision

# Introduction

In RAN2#119bis-e meeting, NR NTN handover enhancements were discussed regarding common signaling for HO command, RACH-less HO, group HO, etc. [1]. The following agreement were made.

Agreements:

- RAN2 can further consider whether some information in the handover command that can be common to all UEs, can be delivered to UEs in common signalling and if there is real benefit (in terms of signalling overhead reduction) in this

- Send an LS to RAN1 (cc RAN4) listing the scenarios (intra-satellite, inter-satellite with same or different feeder links) and check with RAN1 in which scenarios RACH-less is possible (with no indication of RAN2 preference)

- Continue the discussion (in future meeting) on group HO / “UE specific pre-configuration of the target cell + group HO” indication in the next meeting, also on the possible real benefits

- RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well)

- RAN2 continues the discussion (e.g. at RAN2#120) on the solution with keeping the same PCI after switching of the satellites. Clarify at least the following:

•	RAN1 impact

•	The need to perform UL beam switching and/or RA

•	Applicability to hard or soft satellite switching

In this document, we discuss the HO enhancements on common signaling and CHO.

# Discussion

## Common signalling for target cell configuration

In RAN2#119bis-e, common signaling for HO command was discussed for the purpose of reducing overhead. To see how much overhead can be reduced, we consider the current RRC reconfiguration message for HO command.

SpCellConfig

ServeCellIndex

ReconfigurationWithSync

…

CellGroupConfig

CellGroupId

RLC-BearerConfig

MAC-CellGroupConfig

PhysicalCellGroupConfig

SpCellConfig

SCellConfig

…

RRCReconfiguration

DU config

CellGroupConfig

CU config

RadioBearerConfig

MeasConfig

ReconfigurationWithSync

ServingCellConfigCommon

…

TS 38.331 specifies “The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.”

The ServingCellConfigCommon, including DownlinkConfigCommon, UplinkConfigCommon, etc., is the cell specific configuration which is common to UEs since it contains information that can typically acquire from system information.

Observation 1: The ServingCellConfigCommon in HO command is cell-specific configuration and is common to UEs.

Proposal 1: Common signaling for HO command includes at least ServingCellConfigCommon.

Furthermore, the target cell’s common configuration can be valid for a certain duration. Only in the valid duration the UE can apply the common configuration for the HO; otherwise, the UE cannot assume the common configuration is valid.

Proposal 2: A valid duration is configured for the target cell’s common configuration.

## CHO enhancements

### CHO for earth-moving cell

In Rel-17 NTN, time-based and location-based CHO are supported. For location-based CHO, condEventD1 is introduced with execution conditions on the distance between UE and reference locations. With fixed reference locations, the location-based event may work well for quasi-fixed cells. However, for earth-moving cells, without knowing the candidate cell is an earth-moving cell, the UE will keep evaluating the distance to the fixed reference location configured in condEventD1, while the candidate cell may already move away.

Observation 2: For earth-moving candidate cell in CHO, the UE does not know the candidate cell is an earth-moving cell and can result in wrong distance evaluation if using the fixed reference location in condEventD1.

For location-based CHO, if the candidate cell is an earth-moving cell, the NW needs to indicate the candidate cell is an earth-moving cell by providing additional information. This can be similar to the reference location and distance threshold for cell reselection with earth-moving cell.

Proposal 3: For earth-moving candidate cell in CHO, the NW should provide information for UE to estimate the movement of the reference location.

### Store CHO configuration

In Rel-18 Mobility Enhancement WI, subsequent conditional PSCell addition/change (CPAC) is discussed. For NR NTN HO enhancement, we see a similar motivation. In CHO, UE has to release the CHO configurations when completing random access towards the target cell. Hence the UE doesn’t have a chance to perform subsequent CHO without prior CHO reconfiguration and re-initialization from the network. This will increase the delay for the cell change and increase the signaling overhead. Especially in NTN, for earth-moving cell, the UE connects to the serving cell for as short as 6-7 seconds (e.g., 7.56 km/s) [2], so frequent HO is required. Therefore, enabling subsequent CHO without reconfiguration and re-initialization on the CHO preparation from the network can result in a reduction of signaling overhead and interrupting time for frequent HO.

Observation 3: The UE releases CHO configuration after random access to the target cell.

Observation 4: In NTN, signaling overhead for frequent CHO configuration can be reduced.

In RAN2#119bis-e, it has been agreed that the next serving cells can be predicted by the network at lease for earth moving cell. To reduce interruption time and reduce the burden of frequency CHO configuration, the network can configure CHO candidate cells for subsequent CHO at one time and indicate UE to store CHO configuration for some candidate cells after one successful CHO instead of release all. The candidate cells can include the predicted next serving cells.

Observation 5: CHO configuration overhead can be reduced if the UE stores CHO configuration for candidate cells.

Proposal 4: The NW can configure the UE to store candidate cell configuration to enable subsequent CHO.

## Service continuity

In Rel-17 NTN, the serving cell service stop time, i.e., t-Service, is broadcasted in SIB19. In Rel-17, the UE in idle/inactive mode starts measurement before t-Service to enable cell reselection to the incoming new serving cell before t-Service. t-Service basically indicates the service ending time for the geo-location that currently served. Note that t-Service can also be utilized by connected UEs. If the UE in connected mode does not perform successful handover before t-Service, it is most likely disconnected from the serving cell after t-Service. However, the UE still delays RLF declaration according to legacy RLF detection, which will delay RRC connection resume/reestablishment. Thus it is proposed to consider fast RLF detection with the consideration of t-Service.

Observation 6: t-Service indicates the service ending time for the geo-location that currently served by the serving cell, which can also be utilized by connected UEs.

Observation 7: With the consideration of t-Service, the UE in RRC connected can detect RLF faster and it makes the following RRC connection resume/reestablishment faster.

Proposal 5: The UE in connected mode can use t-Service for fast RLF detection and recovery.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Observation 1: The ServingCellConfigCommon in HO command is cell-specific configuration and is common to UEs.

Proposal 1: Common signaling for HO command includes at least ServingCellConfigCommon.

Proposal 2: A valid duration is configured for the target cell’s common configuration.

Observation 2: For earth-moving candidate cell in CHO, the UE does not know the candidate cell is an earth-moving cell and can result in wrong distance evaluation if using the fixed reference location in condEventD1.

Proposal 3: For earth-moving candidate cell in CHO, the NW should provide information for UE to estimate the movement of the reference location.

Observation 3: The UE releases CHO configuration after random access to the target cell.

Observation 4: In NTN, signaling overhead for frequent CHO configuration can be reduced.

Observation 5: CHO configuration overhead can be reduced if the UE stores CHO configuration for candidate cells.

Proposal 4: The NW can configure the UE to store candidate cell configuration to enable subsequent CHO.

Observation 6: t-Service indicates the service ending time for the geo-location that currently served by the serving cell, which can also be utilized by connected UEs.

Observation 7: With the consideration of t-Service, the UE in RRC connected can detect RLF faster and it makes the following RRC connection resume/reestablishment faster.

Proposal 5: The UE in connected mode can use t-Service for fast RLF detection and recovery.

# Reference

[1] R2-2210801 Report from Break-out session on NR-NTN and IoT-NTN

[2] 3GPP TR 38.821 V16.0.0 Solutions for NR to support non-terrestrial networks (NTN)