3GPP TSG-RAN WG2 Meeting #120	R2-2212456

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.14.4

Source:	Nokia, Nokia Shanghai Bell

Title:	QMC support in NR-DC

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 kicked off discussions on a design for QoE handling in DC deployments. The agreements made in RAN2#119bis-e that context established set of the initial principles for the functionality, as follows:

Observation: Rel-18 QoE configuration may be created by MN or SN.

Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination.

1: In NR-DC scenario, both signalling-based and management-based QoE measurement collection shall be supported.

RAN2 assumes that there is a unique ID for QoE configurations across MN and SN. This can be accomplished by MN-SN coordination (e.g. similar as was done with measIds for NR-DC)

Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

This contribution continues development of the configuration and reporting principles to support QoE Measurements Collection in DC.

# 2	Discussion

## 2.1	QoE measurement configuration

The potential need for different QoE configuration origins (from MN or SN) is brought by making the QoE measurement data collection supported in Dual Connectivity.  In that context, NW-centric discussions on the extensions required for QoE measurement collection framework in DC has developed. In particular, RAN3 agreed that for Rel-18, only MN node will configure the UE for Signalling based QoE.

Observation 1: Only MN node will configure the UE for Signalling based QoE.

Within RAN2 scope, RRC signalling enabling the support of the legacy QoE measurement configuration does not explicitly distinguish Signalling vs. Management based QoE configuration. However, within the developed framework for DC, it becomes clear that the only use case to potentially consider SN triggered configuration remains limited to Management-based QoE. RAN3 currently investigates the potential coordination between MN and SN, to decide a way forward.

Observation 2: SN triggered configuration remains applicable only for Management based QoE and is pending RAN3 progress.

Given the understanding of the framework developed by RAN3, we note its worth also considering different types of QoE configuration within RAN. There are two sub-types of QMC configuration:

- Container based QoE,

- RAN Visible QoE.

The WI objectives (RP-221803) targets different approaches for the two configurations: For container based QoE, the first WI objective guides the following:

- QoE measurements are not performed separately for each leg (Note 1).

QoE measurements are performed at the UE application layer. To not perform them separately, from MN and SN, the coordination between the MN and the SN would need to aim at providing a single QoE configuration (as conveyed in the RRC measConfigAppLayerContainer IE) applicable for a given service, area scope and slice scope. A demanding requirement would then be how the network could choose between different XML files or merge the measConfigAppLayerContainers provided via OAM to the MN and SN respectively in case of conflict. As these containers can be transparent to the network, a coordination would need to remain implementation specific. For that reason, we believe the straightforward standard solution is to limit further the only MN-triggered configuration procedure to the container-based  QoE option:

Proposal 1: For container based QoE, only MN provides the QoE configuration to the UE over SRB1 (for both Signalling and Management-based QoE configuration).

In standalone scenario, only SRB1 is used to configure QoE configuration to the UE. In fact, for NR-DC, SRB3 is a mean to be used for a radio measurement configuration and reporting. Given the RAN2 agreement that either SRB1 or SRB3 can be used for providing SN configuration to the UE, we think the follow up observation is:

## 2.2	QoE measurement reporting

When it comes to QoE reporting, the RRC message for QoE reporting (MeasurementReportAppLayer) is possible to be transmitted with a lower priority than other SRBs used for regular radio measurements reporting. In Rel-17, for standalone deployment it is supported over SRB4. If SRB4 is not configured, the QoE Reports can’t be transferred to the Network. Dual Connectivity enables to overcome this shortcoming and facilitate the reporting continuity over the SN leg.

For Rel-18, RANs agreed:

Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

For SRB selection for SN reporting, the agreement exemplifies four options:

- Option 1: reuse SRB3

- Option 2: SRB4 (the same message can be used for transfer to the MN and the SN)

- Option 3: new SRB5 (corresponding to SRB4 in MN)

- Option 4: Split SRB4

Since the SRBs handling between the UE and the NW priority of SRB for QoE reporting should be lower than other SRB, we think SRB3 may not be suitable for QoE reporting. Both Option 2 and Option 3 are feasible, thought  Option 2 seems to risk error-prone handling with handling two nodes by the same SRB. Option 4 have most impact to specification. and it has the implication that the SRB PDCP is may be in MN which means the QoE report should be delivered to the MN PDCP first and then forwarded onwards. Therefore, we prefer having a simple solution and agree to use new SRB5 for reporting QoE o SN:

Proposal 3: In DC, SRB4 is a default choice to transfer the QoE reports (i.e., to the MN-triggered configurations).

Proposal 4: SRB5 can be used to transfer the QoE reports to the SN (e.g., due to MN overload).

It’s worth noting that RAN3 agreement allows changing the reporting leg during the ongoing QoE session:

- QoE reports can be transmitted to either MN or SN and the reporting leg (MCG or SCG) can be changed during the application session.

Changing the reporting leg might be reasonable if the MN gets overloaded. This is helpful for UL load balance and at the same time for ensuring reporting continuity. However, detailed conditions require further studies and better understanding operators’ requirements:

Proposal 5: Handling leg selection for QoE reporting is FFS.

For the message selection, from specification point of view, we believe a straightforward solution is to reuse MeasReportAppLayer message:

Proposal 6: The UE sends the QoE reports to the SN with MeasReportAppLayer message.

## 2.3	QoE Pause

To deal with RAN overload, Rel-17 introduced QoE Pause/Resume mechanism. If the networks is overloaded it can use ‘pauseReporting’ RRC indication to temporarily stop QoE reports from being sent from the UE to the network. Handling of the suspended reports relies on buffering in the UE Access Stratum, until the gNB resumes the QoE reports.

In DC, the temporary suspension of the QoE reporting may take different meaning. After configuring the UE to suspend the QoE report transmission, the UE does not know whether the suspension concerns any reporting or to MN only.

Observation 4: ‘pauseReporting’ RRC indication from MN does not clarify whether the UE can transfer the QoE reports to the SN.

Since there will be possibility to report to the SN, and Rel-18 aims to support reporting continuity, the received pause indication could trigger different actions, which are currently unspecified.

Proposal 5: Rel-17 Pause mechanism should be adopted to support NR-DC.

Proposal 6: FFS what pauseReporting implies in DC (e.g., if MN sends Pause to suspend reporting to both: MN and SN, or to suspend reporting to MN only).

# 3	Conclusion

In this document the following proposals have been made:

Proposal 1: For container based QoE, only MN provides the QoE configuration to the UE over SRB1 (for both: Signalling and Management-based QoE configuration).

Proposal 3: In DC, SRB4 is a default choice to transfer the QoE reports (i.e., to the MN-triggered configurations).

Proposal 4: SRB5 can be used to transfer the QoE reports to the SN (e.g., due to MN overload).

Proposal 5: Handling leg selection for QoE reporting is FFS.

Proposal 6: The UE sends the QoE reports to the SN with MeasReportAppLayer message.

Proposal 7: Rel-17 Pause mechanism should be adopted to support NR-DC.

Proposal 8: FFS what pauseReporting implies in DC (e.g., if MN sends Pause to suspend reporting to both: MN and SN, or to suspend reporting to MN only).