3GPP TSG-RAN WG2 #120	R2-2212461

Toulouse, France, 14 – 18 November 2022

Agenda Item:	6.2.1 Stage-2 corrections

Source:	Ericsson

Title:	Discussion on target SN without SCG

Document for:	Discussion

# 1	Introduction

In RAN2#119bis there was a discussion on whether there can be an SN without an SCG. This was discussed as part of the report in R2-22010810. From the chairman’s notes:

Discussion on P3

Proposal 3: Discuss whether there can be a target SN without SCG in CHO with SN procedure:

− Yes, i.e. as the legacy HO with SN procedure;

− No, i.e. there must be an SCG in CHO.

-	Huawei thinks there is no time to discuss this now. Lenovo raised this issue and thinks if we do not resolve it, the answer is ”no”. ZTE thinks this may be related to the CR and wonders how to proceed.

-	ZTE wonders if RAN3 input is not required any more. No strong opinion and thinks RAN3 could be informed.

Postponed. Can be discussed based on contributions in the next meeting (if time allows and use case can be made clear). CRs should be progressed based on “No” for now, we will decide in the next meeting.

# 2	Discussion

## 2.1	Target SN without SCG

in the legacy eNB/gNB to Master Node change and Inter-Master Node handover with/without Secondary Node change procedure, there can be a target SN without an SCG (e.g. the SN is only configured with SN terminated MCG bearers). The same use cases can occur for conditional handover as for handover. The UE being configured with SN terminated MCG bearers is independent on whether the UE is executing handover or conditional handover. Therefore, support for target SN without SCG needs to be captured also for conditional handover. Renaming of the procedure in 37.340 to “CHO with SN” should also be considered.

There can be an SN without an SCG, e.g. if the SN is only configured with SN terminated MCG bearers.

Update the procedure for “CHO with SCG” to “CHO with SN” taking into account that there can be a target SN without an SCG.

# Conclusion

RAN2 is kindly asked to discuss the following observation and proposal:

Observation 1	There can be an SN without an SCG, e.g. if the SN is only configured with SN terminated MCG bearers.

Proposal 1	Update the procedure for “CHO with SCG” to “CHO with SN” taking into account that there can be a target SN without an SCG.

# References

R2-2210810, Report of [AT119bis-e][201][DCCA] Stage-2 Corrections to DCCA (ZTE)