3GPP TSG-RAN WG2 #120	R2-2212495

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.16.3

Source:	Ericsson

Title:	Use cases aspect for AIML for NR air interface

Document for:	Discussion, Decision

# 1	Introduction

This is the second meeting on which RAN2 discusses the “AI/ML for NR Air Interface” SI (see the approved SID in RP-213599, and the revised version in RP-221348). However, last meeting RAN2 exclusively focused on general and organizational aspects.

The SI will focus on analysing enhancements to the following use cases:

- CSI feedback,

- beam management, and

- positioning accuracy.

For these use cases, the SI aims to evaluate the performance benefits of using AI/ML over existing frameworks, while additionally, assessing potential specification impact and complexity.

In this contribution we:

- focus on some general use-case-specific considerations,

- we provide a set of RAN1’s agreements, for each use case, concerning RAN2 work (especially focusing on the aspects that RAN2 already agreed to study, i.e., data collection, model transfer, model monitoring, model selection/(de)activation/switching, fallback)

  - Note: as captured by the Chair during RAN2#119-bis, we postpone discussions on model training.

- we start to develop on a data collection framework

  - Note: even if within RAN2 scope, it is our understanding that to discuss model transfer aspects further RAN1 input is needed.

# 2	Discussion

## 2.1	General considerations

Regarding the use cases, the SID describes the following:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

From the first set of bullet points (yellow highlight) we understand that it is solely within RAN1 scope to define the use cases and respective sub use cases for the SI framework. Therefore, new (sub) use cases should not be addressed in RAN2.

On the above, having different set of use cases discussed in the different WGs increases the complexity and could potentially hinder the SI progress.

It is not in RAN2 scope to alter the set of use cases.

Having different sets of use cases studied in RAN1 and RAN2 hinders the SI progress.

Do not discuss in RAN2 alternative use cases beyond those agreed by RAN1 and included in the SID.

As for the second main bullet point above (blue highlight), it is also our understanding that the list of (sub) use cases can vary according to RAN1 discussions until the RAN Plenary meeting in December 2022 (RP#98).

A similar conclusion can be drawn from the highlighted text below concerning RAN2’s work:

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

On this same matter, it is important to highlight that for each use case, the different sub use cases consider different models, different settings and different scenarios. Hence, any RAN2 analysis would be subject to these considerations.
And indeed, this has already happened in RAN1, where sub use cases have been left aside and will not be included in the list.

RAN2’s sub case analysis and respective solutions are subject to the list agreed by RAN1, i.e., only analysing a general/main use case from a RAN2 point of view might not be beneficial towards the SI progress.

Moreover, considering the use-case-specific status in RAN1, it is possible to observe that most of the discussion has focused on evaluations of the AI/ML-based solutions for each of these. The progress is thus insufficient for RAN2 to take on any detailed use-case-specific discussions.

Therefore, considering the above, we believe that RAN2 should postpone detailed discussion on these matters for this (sub) Agenda Item.

RAN2 to postpone detailed discussions on use cases or representative sub use cases until this list is settled after RAN#98, and after receiving direct input from RAN1.

It is important to highlight, that current RAN2 work plan [5] takes the above proposal into account for RAN2’s timeline.

RAN2’s Work Plan considers what is captured in Proposal 2 towards the SI timeline.

Below we include a brief overview of the status of concerning agreements and conclusions drawn in RAN1, that may impact RAN2 for each use case.

## 2.2	Observation from RAN1 agreements

For readability purposes, RAN1 agreements concerning RAN2 have been included in the Annex (at the very end of this document). From these, the first thing one could argue for, is that the exercise of taking current RAN1 agreements and transforming them into RAN2-related proposals is not straightforward. Specially given the fact that we see no point in studying the same points in RAN1 and RAN2 simultaneously.

Having that into consideration and the fact that RAN2 is still waiting for RAN1 input, makes it difficult for RAN2 to progress with the use cases study. However, we believe that it is at least possible to grasp, that RAN2 could primarily start by focusing on the data collection aspects for different purposes (e.g., model monitoring/inference/training/etc..).

RAN2 can start studying data collection aspects for the different use cases.

## 2.3	Data collection aspects

Data collection may be needed for model training, model inference, model monitoring, model selection, model update, etc.

However, it is important to highlight that latency requirements for NWs to obtain data can vary depending on the use that will be made of this information (e.g., the latency requirement could eventually be relaxed for model training with respect to model inference).

The above could, for example, affect when having to decide whether RRC-based solutions for this purpose are suitable. However, while having this into consideration, as a straightforward option, RAN2 can start by analysing RRC-based solutions for this purpose.

Let us articulate further on the advantages of considering such approach.

RRC messages can effectively be used to report large amounts of measurement data to the NW with reduced signalling overhead and radio resource consumption.

In this regard, for the different use cases, it could be beneficial to have UEs logging their measurements together with the meta information for several measurement occasions. Specially when the latency requirements are low.

RRC allows the NW to configure a UE to log and report data.

An RRC-based data collection solution enables the NW to efficiently collect large amounts of measurement data.

Additionally, and when considering a general data collection framework for different use cases. We consider that an RRC-based approach could eventually fulfil many of the criteria. In particular, an RRC-based solution could enable the design of a general data collecting framework.

For this purpose, RAN2 should start by consider the following aspects:

- Data types

- Configuration of measurement occasions

- Measurement occasion or logging interval

- Data collection or logging duration

- Data logging triggers

- Report types and data reporting triggers.

However, even when considering the above, we still see the need to receive RAN1 input on this matter to have a better perspective of what is needed to be studied in RAN2.

An RRC-based solution can enable the design of a general data collecting framework.

For the design of a general framework, the following aspects could be considered: data types, configuration of measurement occasions, measurement or logging interval, data collection or logging duration, data collection event type, reporting aspects.

According to RAN1 input, RAN2 to study the feasibility of having a general RRC-based data collection framework for different use cases.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is not in RAN2 scope to alter the set of use cases.

Observation 2	Having different sets of use cases studied in RAN1 and RAN2 hinders the SI progress.

Observation 3	RAN2’s sub case analysis and respective solutions are subject to the list agreed by RAN1, i.e., only analysing a general/main use case from a RAN2 point of view might not be beneficial towards the SI progress.

Observation 4	RAN2’s Work Plan considers what is captured in Proposal 2 towards the SI timeline.

Observation 5	RAN2 can start studying data collection aspects for the different use cases.

Observation 6	RRC allows the NW to configure a UE to log and report data.

Observation 7	An RRC-based data collection solution enables the NW to efficiently collect large amounts of measurement data.

Observation 8	An RRC-based solution can enable the design of a general data collecting framework.

Observation 9	For the design of a general framework, the following aspects could be considered: data types, configuration of measurement occasions, measurement or logging interval, data collection or logging duration, data collection event type, reporting aspects.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not discuss in RAN2 alternative use cases beyond those agreed by RAN1 and included in the SID.

Proposal 2	RAN2 to postpone detailed discussions on use cases or representative sub use cases until this list is settled after RAN#98, and after receiving direct input from RAN1.

Proposal 3	According to RAN1 input, RAN2 to study the feasibility of having a general RRC-based data collection framework for different use cases.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

R1-2205695, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)	“, Ad-hoc Chair (CMCC), RAN1#109-e, May 2022

R1-2208145, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#110, Toulouse, August 2022

R2-2210677, “RAN2 Work Plan for Rel-18 SI on AI/ML for NR air interface”, Ericsson, Qualcomm Inc., RAN2#119bis-e, Oct. 2022

R1-2210690, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, CMCC, RAN1#110bis-e, Oct. 2022

# Annex A: RAN1 agreements concerning RAN2

## A.1	For CSI feedback enhancement

During RAN1#109-e the following was agreed:

Agreement Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

During RAN1#110-e the following was agreed:

## Agreement In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets.

During RAN1#110bis-e the following was agreed:

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring. Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

## A.2	For Beam Management enhancement

During RAN1#109-e the following was agreed:

Agreement For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

On the above, the following was also agreed for spatial-domain DL beam prediction (BM-Case 1):

Agreement For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

While for temporal-domain DL beam prediction (BM-Case 1) it was also agreed to:

Agreement Regarding the sub use case BM-Case2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input: - The value of K is up to companies Agreement Regarding the sub use case BM-Case2, AI/ML model output should be F predictions for F future time instances, where each prediction is for each time instance. - At least F = 1 - The other value(s) of F is up to companies Agreement For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study: - Alt.1: AI/ML inference at NW side - Alt.2: AI/ML inference at UE side

During RAN1#110 the following was agreed:

Agreement For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signaling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded

During RAN1#110bis-e the following was agreed:

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Working Assumption For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following L1 beam reporting enhancement for AI/ML model inference - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring: - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects - Beam measurement and report for model monitoring - Note: This may or may not have specification impact.

## A.3	For Positioning accuracy enhancement

During RAN1#109-e the following was agreed:

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

During RAN1#110-e the following was agreed:

Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded ##### Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded

During RAN1#110bis-e the following was agreed:

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data