3GPP TSG-RAN WG2 Meeting #120	R2-2212510

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	DRX related enhancement for LPHAP

WID/SID:	FS_NR_Pos_Enh2-Core - Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN#94e, a new study item on expanded and improved NR positioning was approved [1]. For LPHAP, the following objective was included:

- Improved accuracy, integrity, and power efficiency: - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In RAN1#110bis-e meeting, the following conclusions are made for LPHAP [2]:

Conclusion - From evaluations for a LPHAP device, RAN1 concludes that the existing Rel-17 positioning for UEs in RRC_INACTIVE state cannot satisfy the target battery life required by LPHAP use case 6 in the majority of the evaluation scenarios that were examined. - Based on the evaluations, enhancements to meet the target battery life in Rel-18 are necessary.

Based on RAN1 conclusion, it is necessary to study the enhancements to meet the target battery life in LPHAP in Rel-18. In the contribution, we will present our views on DRX related enhancement for UE power saving in LPHAP.

# 2 	DRX related enhancement

## 2.1	Configuration alignment between PRS and DRX

In the legacy release, DRX was introduced to save the power resources and increase the battery lifetime of UE. For positioning in RRC_INACTIVE or RRC_IDLE, UE needs to periodically wake up to monitor the paging message during DRX OnDuration. If such paging message is not intended for the UE, it goes back to sleep for power saving. However, far from now the operation of DRX in RRC_INACTIVE/RRC_IDLE and positioning procedure are performed independently from each other, which may result in misalignment between the PRS configuration and the DRX configuration. That can further result in unnecessary power consumption as the UE needs to power on and off its receiver several times to perform both paging related and positioning related operations, e.g., PRS measurement, which is against power saving intention of LPHAP in RRC_ INACTIVE or RRC_ IDLE.

Observation 1: Independent operation of DRX configuration and positioning procedure in RRC_ INACTIVE /RRC_ IDLE will result in misalignment between the PRS reception and thus bring unnecessary UE power consumption for LPHAP

In our views, it would be beneficial if the LMF was somehow aware of the DRX configuration of the UE. Based on the knowledge of the UE’s DRX configuration, the LMF can optimize the PRS configuration of the UE so that the PRS measurement can be aligned with the already configured DRX active time, which can reduce the transition power of the UE to receive PRS when it wakes up from sleep.

Proposal 1: It’s beneficial if the LMF was somehow aware of the DRX configuration of the UE to align the PRS configuration with the configured DRX active time.

## 2.2	Positioning measurement and DRX

In our view, we shall support PRS measurement in the vicinity of paging monitoring for power saving. However, sometimes it is difficult to ensure each PRS measurement aligns with DRX active time. For example, DRX cycle may be different in different cells. Therefore, the cells that provide DRX configuration can be not only the serving cell, but also neighbouring cells around the UE in the same RNA area. After cell reselection, the PRS configuration of the UE may be not aligned with the DRX configuration of the reselected cell. To avoid re-configuration of PRS for UE after cell reselection, if the positioning accuracy requirement can be satisfied, the LMF can trigger the UE to avoid PRS measurements on some occasions which are not in line with the UE’s DRX active time in the reselected cell for power saving.  Furthermore, if the UE is in low power status or there are enough measurements available to estimate UE position, the LMF can trigger the UE to selectively avoid some positioning measurements in one or more configured DRX cycles or a certain window for UE power saving when the UE monitors paging message during the DRX active time

Proposal 2:  Selectively avoiding positioning measurement on some occasions when the PRS and DRX configuration are not aligned or when certain criteria are met is beneficial to reduce UE power consumption.

Similar enhancement can be also applied for SRS transmission in UL positioning.

## 2.2	Text proposal for RRC_INACTIVE procedure for LPHAP

Based on the discussion above, we propose the following text proposal for the TR 38.859 for the RRC_INACTIVE procedure for LPHAP.

Proposal 3: Adopt the following text proposal for DRX-related enhancement for LPHAP.

==================================BEGIN OF TP====================================== ### 6.4.X DRX-related enhancement for Low Power High Accuracy Positioning At present, the operation of DRX in RRC_INACTIVE/RRC_IDLE and positioning procedure are performed independently from each other. Misalignment between the PRS configuration and the DRX configuration may result in unnecessary power consumption of UE for LPHAP. It would be beneficial if the LMF was somehow aware of the DRX configuration of the UE so as to align the PRS measurement or SRS transmission with the configured DRX active time. With the knowledge of the UE’s DRX configuration, the LMF can optimize the PRS/SRS configuration or/and corresponding measurement/transmission of the UE, e.g., selectively avoid positioning measurement/transmission on some occasions which is not aligned with the UE’s DRX active time, to reduce UE power consumption in LPHAP. ===============================NEXT TP======================================== ## 7.X Conclusions for Low Power High Accuracy Positioning For LPHAP, it would be beneficial if the LMF was somehow aware of the DRX configuration of the UE. With the knowledge of the UE’s DRX configuration, the LMF can optimize the PRS/SRS configuration or/and the corresponding measurement/transmission of the UE for power saving in LPHAP.

# 3	Conclusion

This document has made the following observation:

Observation 1: Independent operation of DRX configuration and positioning procedure in RRC_ INACTIVE /RRC_ IDLE will result in misalignment between the PRS reception and thus bring unnecessary UE power consumption for LPHAP

And proposed the following:

Proposal 1: It’s beneficial if the LMF was somehow aware of the DRX configuration of the UE to align the PRS configuration with the configured DRX active time.

Proposal 2:  Selectively avoiding positioning measurement on some occasions when the PRS and DRX configuration are not aligned or when certain criteria are met is beneficial to reduce UE power consumption.

Proposal 3: Adopt the text proposal for DRX-related enhancement for LPHAP proposed in R2-2212510.

# 4.	References

- RP-213561, new SID on Study on expanded and improved NR positioning

- RAN1 #110bis-e Chair’s Notes