3GPP TSG-RAN WG2 Meeting #120	R2-2212608

Toulouse, France, November 13-18, 2022

Agenda Item:	8.5.2.1

Source: 	Meta

Title:  Discussion on Uplink XR-Awareness for XR services

Document for:	Discussion and Decision

# Introduction

In RAN2#119-e, RAN2 had initial agreements on XR-awareness during RAN2#119-e to identify the areas of interest to be considered during the related SI phase as shown below:

- RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions. - RAN2 will study PDU Set based parameters and PDU Set related information handling in Network and UE - RAN2 to adopt the current SA2 definition of PDU Set as an application media unit as working assumption, subjected to further guidance from SA2 and SA4 - XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts. - RAN2 can study e.g. periodicity, arrival time, jitter and frame-size variations for XR awareness to enable power savings and capacity enhancements. Can study also how often such parameters change (i.e. how dynamic they are). - RAN2 can consider how PDU sets can be mapped to DRBs (FFS if SA2 discussion on PDU set mapping to QoS (sub-)flows impacts this)

In RAN2#119-bis-e, RAN2 the following XR QoS related information to RAN for power saving was discussed with observation and agreements captured as follows :

- Observation 1. There are strong support among companies that the following set of information on DL/UL traffic is useful to RAN for UE power savings: - Traffic parameters (e.g. periodicity, start offset, etc) are useful to RAN in configuring DRX and can be semi-statically signalled to RAN. - Jitter statistics (e.g. range) are useful to RAN, e.g. in configuring DRX on duration, and can be semi-statically signalled to RAN. - Boundary indication (e.g. start and/or end of a PDU Set or a Data Burst) is useful to RAN, e.g. in timely termination of DRX active time. It can be dynamically signaled to RAN. - Information for identifying a PDU Set (e.g. sequence number) is useful to RAN and can be dynamically signalled to RAN. - Explicit indications and/or conditions for RAN to decide whether to deliver or discard a media unit is useful to RAN, e.g. to avoid unnecessary re-/transmissions and thus save UE power. - For traffic flows not based on PDU Sets, their periodicity, start offset and range of jitters are useful information to RAN, e.g. in DRX configuration. - Observation 2. SA2 have agreed that periodicity of DL/UL QoS flows and jitter information associated with each periodicity are provided to NG-RAN at PDU Session Establishment/Modification. - Observation 3. SA2 have agreed that for downlink traffic, end of data burst can be optionally signalled in the header of the last PDU of the Data Burst. - Observation 4. SA2 have agreed that for downlink traffic, PDU Set related information such as PDU Set Identifier, start/end indication of a PDU Set or number of PDUs within a PDU Set are signalled via user plane for PDU Set based handling. - Observation 5. SA2 have agreed that on downlink, whether all PDUs are needed for the usage of PDU Set by application layer is part of PDU Set QoS parameters provided to RAN. - Observation 6. SA2 have agreed that uplink PDU Set handling should be studied and led by RAN WGs. Agreements Þ 1. RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings. Þ 2. RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN.

Additionally RAN2 has also agreed on

On the PDU Sets and Data Bursts Þ 1: From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Þ Semi-static information (from CN to RAN): At least PSER and PSDB. Þ Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

In this contribution, based on SA2 agreement that UL PDU Set handling should be studied and led by RAN WGs. We provide some further discussion XR QoS related information that can be provided to RAN.

# Discussion

## Background information

In 5GS, (see TS 23.501, fig 5.7.1.5-1), UL/DL traffic classification is based on PDR (Packet Detection Rules for DL, UL traffic filter for UL). Normally, it uses 5 tuples to classify the traffic (Source IP, Destination IP, Source port, Destination port, Protocol ID), i.e., to map an IP flow with a class of QoS service.

From 3GPP TR 23.700-60 V0.3.0, the definitions of PDU Sets and Data Burst are provided as follows:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the application layer can still recover parts all or of the information unit, when some PDUs are missing. Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time. NOTE: A Data Burst can be composed by one or multiple PDU Sets.

It can be observed that both PDU Set and Data Burst are application layer characterization of XR traffic. Hence, the same concept can be applied to UL XR traffic as well.

Observation 1:

Both PDU Set and Data Burst are application layer characterization of XR traffic. Hence, the same concept can be applied to UL XR traffic as well.

Proposal 1:

Reuse PDU Set and Data Burst concept for UL XR traffic.

## Traffic Parameters and PDU Set Related Information

In S2-2209939, the following conclusions made by SA2 regarding traffic periodicity are captured:

The following information, to be provided to the NG-RAN at PDU Session Establishment/Modification via an NGAP Message, is taken as baseline for normative work: - Periodicity for UL and DL traffic of the QoS Flow.

Hence, the traffic characterization including periodicity  from DL should be kept for the uplink. However, as indicated in the examples,  generally there would be a mixed in

Proposal 2:

RAN2 to confirm traffic parameters (e.g. periodicity, start offset, etc) for the UL traffic of the QoS flow are provided to NG-RAN at PDU Session Establishment/Modification.

However, note that in additional to typical video streams, XR applications may also consists of other information such as pose and control, e.g. in Rel. 17, TR 38.838,  Study on XR (Extended Reality) evaluations for NR , the following example has been used for evaluation

In Model 2, two streams are considered. - Stream 1 for pose/control - Traffic model/requirement for stream 1 follows clause 5.2. - Stream 2 aggregating scene, video, data, and audio - Follows the statistical parameters shown in Table 5.2-1.

Observation 2:

Typical UL traffic consists of various types of information including pose, control and video which can be periodic or non-periodic.

Proposal 3:

RAN2 to discuss to differentiate and separate the UL traffic to periodic and non-periodic traffic into different QoS flows.

In S2-2209939, the following conclusions made by SA2 regarding jitter information is captured:

Traffic jitter information associated with each periodicity. Editor's Note: How 5GC derives the above information is FFS.

There is no explicit indication on whether the traffic jitter information is related to UL or DL. However, given that SA2 expects 5GC to derive the jitter information. It can be interpreted that the decision does not include the UL UE jitter. Note that the UL jitter is generally based on UE application layer implementation, e.g. hardware or software implementation or scene complexities, while at the same time, the jitter information may be helpful for delay sensitive schedulers.

Observation 3:

There is no clear indication on whether UL jitter information should be provided in SA2 agreements.

Proposal 4:

RAN2 to confirm that UL jitter information can be optionally provided by UE

On the PDU Set/Data Burst related info, such as PDU set SN for UL traffic, PDU Set Identifier,  End of Data Burst. As mentioned previously, SA2 has agreed that “whether and how to support the PDU Set handling for Uplink will be studied and led by RAN.”  While the information could be provided by the UE, the information can add  considerable overhead to the uplink transmission in the Uu interface.  Hence, RAN2 should carefully consider the impact and use cases and whether the information can be implicitly indicated to gNB by UE.

Notice that some of the information related PDU Set/Data Burst information could be important for providing information for PDU Set handling and delay information for BSR reporting for the UL traffic, e.g. End of Data Burst/PDU Set End indication can help PDU Set integrated handling and provide the indication for BSR reporting or gNB scheduling.  Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication) could also be provided help RAN making discard decisions. Note some of the PDU Set Related information has already been agreed as:

On the PDU Sets and Data Bursts Þ 1: From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Þ Semi-static information (from CN to RAN): At least PSER and PSDB. Þ Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

Proposal 5:

- RAN2 to agree providing following UL PDU Set/Data Burst Related information at least including:

  - Start PDU and End PDU of the PDU Set

  - End of Data Burst indication

  - Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication

## XR-Awareness Based User Assistance Information Framework

While the PDU Set framework provides the DL assistance information on XR-awareness to RAN, additional UL XR-awareness information, such as frame rate, assistance information to the discard timer setting, can also be provided from the UE based on the current User Assistance Information framework.

Observation 4:

XR-aware information for UL service data flow can also be provided from the UE based on the existing user assistance information framework.

Proposal 6:

RAN2 to study providing XR-awareness from the UE based on user assistance information framework

# Conclusion

Observation 1:

Both PDU Set and Data Burst are application layer characterization of XR traffic. Hence, the same concept can be applied to UL XR traffic as well.

Observation 2:

Typical UL traffic consists of various types of information including pose, control and video which can be periodic or non-periodic.

Observation 3:

There is no clear indication on whether UL jitter information should be provided in SA2 agreements.

Observation 4:

XR-aware information for UL service data flow can also be provided from the UE based on the existing user assistance information framework.

Proposal 1:

Reuse PDU Set and Data Burst concept for UL XR traffic.

Proposal 2:

RAN2 to confirm traffic parameters (e.g. periodicity, start offset, etc) for the UL traffic of the QoS flow are provided to NG-RAN at PDU Session Establishment/Modification.

Proposal 3:

RAN2 to discuss to differentiate and separate the UL traffic to periodic and non-periodic traffic into different QoS flows.

Proposal 4:

RAN2 to confirm optionally providing UL jitter information should be provided by UE

Proposal 5:

- RAN2 to agree providing following UL PDU Set/Data Burst Related information at least including:

  - Start PDU and End PDU of the PDU Set

  - End of Data Burst indication

  - Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication)

Proposal 6:

RAN2 to study providing XR-awareness from the UE based on user assistance information framework.

.

# References

R2-2210825  	Information to RAN for UE power savings based on SA2 progress       	Qualcomm Incorporated discussion

TR 38.838,  Study on XR (Extended Reality) evaluations for NR