# 3GPP TSG-RAN WG2 Meeting #120	   R2-2212626

# Toulouse, France, Nov 14th - 18th, 2022

# Agenda Item:	8.13.5

# Source:	CMCC

# Title:	SON enhancement for NR-U

# Document for:	Discussion and Decision

1	Introduction

In RAN2#119bis, we discussed SON/MDT for NR-U and the following agreements are made.

Agreements:

1	The UE will log information of multiple RA procedures related to consistent LBT failures. FFS details.

In this contribution we provide our opinions about the details of multiple RA procedures and some other SON/MDT enhancements for NR-U.

2	Discussion

When using unlicensed spectrum, LBT needs to be performed before transmission. In the previous releases, LBT-FailureRecoveryConfig is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access.

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers:

2>	start or restart the lbt-FailureDetectionTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	trigger consistent LBT failure for the active UL BWP in this Serving Cell;

3>	if this Serving Cell is the SpCell:

4>	if consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in this Serving Cell:

5>	indicate consistent LBT failure to upper layers.

4>	else:

5>	stop any ongoing Random Access procedure in this Serving Cell;

5>	switch the active UL BWP to a UL BWP, on same carrier in this Serving Cell, configured with PRACH occasion and for which consistent LBT failure has not been triggered;

5>	initiate a Random Access Procedure.

For RA procedure, before a RA procedure is successfully performed in the active BWP, some consistent LBT failure from other BWPs may occur in this serving cell at first. However, in current procedure, only the RA report of the successfully performed RA procedure will be reported to the network, the reports of consistent LBT failure from other BWPs will not be recorded and reported to the network.

Based on this, in RAN2#119bis, we have the agreement about the UE will log information of multiple RA procedures related to consistent LBT failures and details should be further discussed. As the serving cell of the multiple RA reports are the same, it seems that ra-InformationCommon in RA report is an appropriate IE to be extended for multiple RA reports.

Proposal 1: For multiple RA procedures caused by consistence LBT failure, RA-Report is extended with a list of ra-InformationCommon for UE to record multiple RA procedures at different BWPs in an RA report.

If a RA procedure is failed due to the consistent LBT failure in the active BWP, UE will switch the active UL BWP to a UL BWP, on same carrier in this Serving Cell, configured with PRACH occasion and for which consistent LBT failure has not been triggered and initiate a RA procedure. But the network has no idea that this RA procedure is triggered due to the consistent LBT failure.

Proposal 2: Add a new RA purpose for consistent LBT failure.

For RA procedure, the MAC entity shall, for each Random Access Preamble:

1>	if LBT failure indication is received from lower layers for this Random Access Preamble transmission:

2>	if lbt-FailureRecoveryConfig is configured:

3>	perform the Random Access Resource selection procedure.

2>	else:

3>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

3>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

4>	if the Random Access Preamble is transmitted on the SpCell:

5>	indicate a Random Access problem to upper layers;

5>	if this Random Access procedure was triggered for SI request:

6>	consider the Random Access procedure unsuccessfully completed.

4>	else if the Random Access Preamble is transmitted on an SCell:

5>	consider the Random Access procedure unsuccessfully completed.

3>	if the Random Access procedure is not completed:

4>	perform the Random Access Resource selection procedure.

If the LBT-FailureRecoveryConfig is configured, the lbt-FailureInstanceMaxCount can be set to different values, only consistent uplink LBT failures indication may not represent all the LBT information, the number of consistent uplink LBT failures happened for each UL BWP is useful for NR-U BWP configuration. If the LBT-FailureRecoveryConfig is not configured, the PREAMBLE_TRANSMISSION_COUNTER is incremented by 1 for each LBT failure or RA Response reception not successful, then there may exist the scenario about the LBT failure have the impact on the RA failure, even if the random access problem is not directly caused by the LBT failure.

Proposal 3: Suggest to study the LBT failure have impacts on the RA failure or RLF case.

When the LBT-FailureRecoveryConfig is configured, if the number of LBT failure is less than lbt-FailureInstanceMaxCount, the network may not know the LBT failure, and the procedure successfully complete. For a random access procedure, for each Random Access Preamble, if LBT failure indication is received from lower layers for this Random Access Preamble transmission and lbt-FailureRecoveryConfig is configured, the lbt-FailureDetectionTimer starts and increment LBT_COUNTER by 1. If the number of LBT failure for preamble is less than lbt-FailureInstanceMaxCount, network may not know there are some LBT failure occurs as the LBT failures are not included in the RA report and RLF is not triggered by consistent uplink LBT failure.

It is the same as succcessful handover, if the LBT_COUNTER do not reach the lbt-FailureInstanceMaxCount, the handover is successful completed even when the LBT_COUNTER = lbt-FailureInstanceMaxCount - 1, the network may not notice that the procedure is near failure as the LBT related information is not reported to the network for the successful procedure.

Observation 1: Even if the RA/HO procedure is successfully completed, the network may not know whether this RA procedure experiences any LBT failure or not.

Proposal 4: The LBT information (e.g., the number of LBT failures) is added in the RACH report and RLF report.

Besides, for the same unlicensed spectrum, many operators may deploy NR-U on the same unlicensed spectrum. The LBT failure information is one of the key information for operator to analyze the channel quality for the unlicensed spectrum. So even if the RLF is not triggered, reporting LBT info in measurement reporting through immediate MDT is also beneficial. Instead of reporting the consistent uplink LBT failures information until RLF occurs, including the LBT failure information in immediate MDT report and report it as soon as the measurement report is available may be another approach for optimization.

Proposal 5: The LBT information can be added in measurement reporting for immediate MDT.

3	Conclusion

Proposal 1: For multiple RA procedures caused by consistence LBT failure, RA-Report is extended with a list of ra-InformationCommon for UE to record multiple RA procedures at different BWPs in an RA report.

Proposal 2: Add a new RA purpose for consistent LBT failure.

Proposal 3: Suggest to study the LBT failure have impacts on the RA failure or RLF case.

Observation 1: Even if the RA/HO procedure is successfully completed, the network may not know whether this RA procedure experiences any LBT failure or not.

Proposal 4: The LBT information (e.g. the number of LBT failures) is added in the RACH report and RLF report.

Proposal 5: The LBT information can be added in measurement reporting for immediate MDT.

4	Reference

[1] 	RP-221825, “New WID on Further Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC”, CMCC