3GPP TSG RAN WG2 #120	R2-2212636

Toulouse, FR, 11th – 14th November, 2022

Agenda item:	8.5.4.1

Source:	CMCC

Title:	Enhancement on BSR for XR-specific capacity improvement

Document for:	Decision, Discussion

1	Introduction

In RAN2 119bis-e, some consensuses on feedback enhancement for XR have been reached, some of which are shown as follows,

1: Introduce new BS table(s) to reduce the quantization errors (e.g., for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

Delay information consists of at least “remaining time”.

2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g., via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g., what’s contained, how the triggering is done) can be discussed in the WI phase.

If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

In this contribution, some potential enhancements on BSR for XR-specific capacity improvement will be discussed.

2	Discussion

2.1 Overview

As discussed in the last meeting [1], BSR enhancement is a potential approach for XR-specific capacity enhancement.

One noticeable shortcoming of current BSR is the quantization errors caused by the inappropriate granularity. Obviously, using a longer buffer size field can reduce quantization errors, but as to the quantization error caused by overrange, long buffer size should be not applicable.

Overrange usually happens when actual buffer size is larger that can be indicated in the BSR table which can be more common in Short and Short Truncated BSR since the maximum buffer size is the relatively small (150KiB).

Therefore, sorely relying on longer buffer size would not be helpful enough to reduce the quantization caused by overrange. More BSR table can be introduced to eliminate the quantization errors.

The other also noticeable issue is that the size of generated packets in the given time is a random variable which follow a certain probability distribution, therefore, the BSR table should naturally reflect the cumulative distribution function of the corresponding random variable.

A most significant advantage of BSR is that it can provide information to the scheduler of gNB in a more real-time way. Other uplink signaling, such as RRC message, should send a scheduling request (SR) for a BSR first, and then send the BSR to request UL grant for the uplink signaling itself. BSR also has its unique advantage in terms of scheduler optimization and resource allocation since BSR itself is a MAC signaling and the resource allocation of MAC scheduler largely rely on the information provided by BSR in uplink.

However, there exists a contradictory on BSR enhancement, if the BSR carries limited information as the current mechanism, the optimization would be nearly impossible for implementation. But if BSR carries too much assistance information, the overhead caused by assistance information will impact system capacity. BSR with more assistance information and new trigger and reporting mechanism is necessary

2.2 BS table

As noted in the overview above, overrange and inappropriate granularity can cause quantization errors. In implantation, improper quantization function will also cause noticeable quantization error. A common example of that is to use linear quantization function to quantify non-linear signal, and vice versa.

Observation 1: Quantization errors in BSR are mostly caused by inappropriate granularity, overrange and improper quantization function.

The inappropriate granularity can be resolved by using more bits for Buffer size field in BSR, but the errors caused by overrange can only be solved by modifying the maximum buffer size in BS table. Overrange is more common in Short and Short Truncated BSR since the maximum buffer size in Short and Short Truncated BS table is only 150 KB, which can be too small for high-rate traffic.

Proposal 1: RAN2 is kindly asked to confirm that new BS(s) table should consider quantization errors caused by buffer size overrange and improper quantization function in high data rate traffic.

Theoretically, a proper quantization function should be the cumulative density function (CDF) followed by the random variable for the corresponding signal to be quantized, which can further reduce the quantization error. A common argument for this conclusion is that the A-law and μ-law proposed in ITU G.711 PCM for quantifying voice signal are based on the cumulative density functions for certain Gaussian distributions.

Therefore, it is necessary to study the characteristics of uplink data and its corresponding random variable in XR service. As previously discussed, XR traffic data compose of multiple types of data, e.g., video, audio, controller input, signaling and other application layer data. Similarly, the multi-modal data defined in [2] is also to describe the input data from different kinds of devices/sensors or the output data to different kinds of destinations (e.g., one or more UEs) required for the same service or application.

Specifically, in VR traffic, the audio data is relatively fixed in size, which means it can be describe as an offset for the expectation of the corresponding random variable. Signaling and other application layer data account for a small proportion of VR traffic compared with video frame, not to mention that signaling and other application layer data may be mapped to different LCH in transmission.

It’s quite clear that video frame is the main component in VR DL traffic, but the main component of XR UL traffic is not very equivocal. The reason for this kind of ambiguity is the wide difference among VR, AR and MR UL traffic. The main component for VR UL traffic can be the head tracking information or pose control information, while that of AR/MR UL traffic can be video frame, since AR/MR application usually requires information on reality and it is generally captured by UE.

Observation 2: The main component in XR UL traffic can be determined by service type, e.g., VR, AR, MR, or cloud gaming.

Observation 3: The statistical characteristics of main component in XR UL traffic should be captured in BS table to reduce quantization error caused by improper quantization function, e.g., max-min, expectation, and CDF.

For the purpose to eliminate the quantization error, it is necessary to discuss main component in XR UL traffic and the characteristics of main comment. As illustrated, main component in XR UL traffic can be signaling, head tracking information, uplink video/audio, or other not mentioned data.

Proposal 2: RAN2 is kindly asked to discuss whether new BS table should reflect the cumulative density function of main component in XR UL traffic.

The candidate distributions can be truncated Gaussian (RAN1’s assumption), Pearson, Pareto, and other distributions, as research suggests [3].

2.2 BSR format

The BSR format determines the content to be reported to gNB and the length of each content field. Current BSR consist of LCG field and Buffer size field. Besides, granularity is mainly impacted by BSR format.

Since it’s an FFS on whether to reflect delay information in BSR, and the BSR format may be discussed in WI stage, it might be too early to discuss the details on BSR format. However, there exits some issues can be discussed in current stage, one of which is the length of buffer size filed for appropriate granularity.

Because any data struct that is not 8-bit aligned requires bit-field operation for modern x86/arm processer, and LCG field for single LCG buffer size reporting is at least 3 bits, the minimal size of single LCG BSR with fine granularity is at least16 bits.

Considering too many bits for buffer size will increase overhead, and capacity improvement are not obviously due to the absence of further simulation, 8-bit buffer size can be effective enough for finer granularity in R18 from our perspective.

Proposal 3: RAN2 is kindly asked to confirm that 8-bit buffer size field is sufficient for an appropriate granularity for XR service in R18.

A potential advantage for 8-bit buffer size filed is that it is the same with that of Long and Long Truncated BSR format, which may be easier for implementation both in gNB and UE.

2.3 Assitance information in BSR

There do exits some arguments on the introduction of delay information in BSR. However, for our perspective, to capture assistance information including delay is beneficial to improve the system capacity.

The most important reason is that BSR itself is an MAC layer signaling, therefore it can deliver information in a more real-time way to the MAC scheduler for scheduling and LCP optimization. Reconfiguration of PDSB requires RRC signaling interaction, and more physical radio resource and time when compared with BSR. Except the interaction overhead, whether PDSB can update timely enough is also in doubt.

The other reason is that BSR with assistance information can provide information with per-PDU granularity, which cannot be achieved by PSDB. A PDU set may consist of several video frames (GoP option) or several slices (I or P-slice option) from the same video frame, per PDU information such as PDU importance and PDU index can be helpful.

Observation 4: BSR can transfer assistance information to scheduler in a more real-time way with per-PDU granularity.

Proposal 4: RAN2 is kindly asked to confirm that introducing assistance information to BSR is necessary for XR-specific capacity improvement.

Other candidate assistance information can be index within PDU set, PDU (set) importance for priority optimization, volume of burst data, desired minimal TB size in the next UL grant and other potential assistance information.

2.4 Reporting mechanism for enhanced BSR

The current BSR enhancement mainly aims at reduction of quantization errors and provision of assistance information. And it seems that the length of BSR MAC CE will increase due to the longer Buffer size field and the potential assistance information field.

The enhanced BSR with finer granularity and assistance information, if approved, will increase overhead, since it will be transmitted with MAC SDU and other MAC CE. Therefore, the enhanced BSR should be only trigged or transmitted on demand, e.g., the actual buffer size has significant difference with legacy BS table, or delay budget is going to be exhausted, of which the details can be discussed later.

Proposal 5: RAN2 is kindly asked to confirm that BSR with assistance information and/or less quantization errors should be only sent on demand to reduce overhead.

The discussion on reporting and triggering mechanism also need to be discussed from our perspective.

3	Summary

This contribution discusses potential enhancements on BSR for XR, the following are the observations and proposals that may be taken into consideration.

Quantization error aspect:

Observation 1: Quantization errors in BSR are mostly caused by inappropriate granularity, overrange and improper quantization function.

Proposal 1: RAN2 is kindly asked to confirm that new BS(s) table should consider quantization errors caused by buffer size overrange and improper quantization function in high data rate traffic.

Observation 2: The main component in XR UL traffic can be determined by service type, e.g., VR, AR, MR, or cloud gaming.

Observation 3: The statistical characteristics of main component in XR UL traffic should be captured in BS table to reduce quantization error caused by improper quantization function, e.g., max-min, expectation, and CDF.

Proposal 2: RAN2 is kindly asked to discuss whether new BS table should reflect the cumulative density function of main component in XR UL traffic.

BSR format aspect:

Proposal 3: RAN2 is kindly asked to confirm that 8-bit buffer size field is sufficient for an appropriate granularity for XR service in R18.

Assistance information aspect:

Observation 4: BSR can transfer assistance information to scheduler in a more real-time way with per-PDU granularity.

Proposal 4: RAN2 is kindly asked to confirm that introducing assistance information to BSR is necessary for XR-specific capacity improvement.

Overhead reduction aspect:

Proposal 5: RAN2 is kindly asked to confirm that BSR with assistance information and/or less quantization errors should be only sent on demand to reduce overhead.

References

- R2-2210802, Report from session on LTE legacy, 71 GHz, DCCA, Multi-SIM, RAN slicing, QoE and XR, Vice Chairman (Nokia)

- TR 22.847, Study on supporting tactile and multi-modality communication service, V18.2.0, 3GPP

- M. Lecci, M. Drago, A. Zanella and M. Zorzi, "An Open Framework for Analyzing and Modeling XR Network Traffic," in IEEE Access, vol. 9, pp. 129782-129795, 2021, doi: 10.1109/ACCESS.2021.3113162.