3GPP TSG-RAN WG2 Meeting #120                                        R2-2212682

Toulouse, France, Nov 14th – 18th , 2022

Source:         ZTE Corporation

Title:            Discussion on RedCap positioning

Agenda item:    8.2.5

Document for:	Discussion and Decision

- Introduction

The Rel-18 SID [1] for positioning is achieved as follows:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we discuss some technical enhancements about RedCap positioning from RAN2 perspective.

- Discussion

  - Frequency hopping

In RAN1#110-bis meeting[2], the following agreements about resource frequency hopping have been made:

- Agreement - Study the potential enhancement of the UL SRS for positioning to enable Tx frequency hopping, including but not limited to partial overlapping between hops, hopping bandwidth, time gap between frequency hopping. - Agreement - Study the potential enhancement of the DL PRS to enable Tx or Rx frequency hopping, including but not limited to impact on processing capability, hopping bandwidth in the positioning frequency layer, time gap between frequency hopping, measurement period, partial overlapping between hops.

In RAN2#119-bis meeting[3], the following agreement is made:

Agreement: RAN2 wait for RAN1 conclusion on the signalling design for assistance data for frequency hopping.

RAN1 has agreed to study the frequency hopping of UL SRS and DL PRS. The MIMO SRS frequency hopping has widely been applied to achieve power boosting and full frequency channel information (agreed in LTE, Rel-8 and NR Rel-15). But, the frequency hopping there, is a hopping within single BWP. Similar to the idea above, a frequency hopping of a reference signal for positioning can extend the bandwidth of the reference signal for positioning. By concatenating multiple hops of reference signal for positioning, the effective bandwidth of reference signal for positioning can be enlarged. It should be noted that, the channel condition between each hop should not change much. In a typical scenario (e.g., low mobility and LOS links), this condition might be fulfilled.

To be specific, there are several candidate solutions of reference signal resource hopping, as listed as follows:

- Frequency hopping within one resource

- In this way, the frequency domain resource of one reference signal is divided into several parts, each part corresponds to a frequency hop, and several hops are shown in different symbols. In other words, the three hopping parts refer to different symbols of one RS resource.

- Figure 1. Example of frequency hopping within one resource

- Frequency hopping within one resource set

- In this way, assuming that one reference signal resource set contains several reference signal resources, then different reference signal resources in the resource set may have different frequency range and correspond to different hops.

- Figure 2. Example of frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

- In this way, the several reference signal resources belonging to different CCs/BWPs/PFLs/resource sets can be associated to compose frequency hopping. One example is, different reference signal resources with the same ID in different CCs/BWPs/PFLs/resource sets correspond to different hops.

- Figure 3. Example of frequency hopping between different CCs

- Proposal 1: For RedCap UE, support RAN2 to further study the following candidate solutions of RS resource frequency hopping:

- Frequency hopping within one resource

- Frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

  - Power saving

- In RAN2#119-bis meeting, there are views to enhance the power saving of RedCap positioning. One way is to extend the eDRX, this is already specified in the WID[4] of RedCap WI, we should wait/follow their study:

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement

- Another proposal was to align reference signal configuration and DRX configuration (especially in RRC_CONNECTED). This kind of power saving enhancements is similar with LPHAP enhancements, we should avoid duplicate/parallel discussion in these two agendas. Therefore, power saving enhancement for RedCap positioning should be treated as low priority.

- Proposal 2: Power saving enhancement for RedCap positioning should be treated as low priority.

- Conclusion

- In this contribution, the following proposal is presented:

- Proposal 1: For RedCap UE, support RAN2 to further study the following candidate solutions of RS resource frequency hopping:

- Frequency hopping within one resource

- Frequency hopping within one resource set

- Frequency hopping between different CCs/BWPs/PFLs/resource sets

- Proposal 2: Power saving enhancement for RedCap positioning should be treated as low priority.

- References

- RP-213588, Revised SID on Study on expanded and improved NR positioning.

- Report of 3GPP TSG RAN WG1 meeting #110-bis-e, 2022-10.

- Report of 3GPP TSG RAN WG2 meeting #119-bis-e, 2022-10.

- RP-222675, New WID on enhanced support of reduced capability NR devices, 2022-09.