3GPP TSG-RAN2 Meeting #120	                                          R2-2212746

Nov 14th – 18th, 2022 Toulouse FR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3731 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the MUSIM Gap Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | - If an periodic MUSIM gap configuration associated with the musim-GapId indicated by the MUSIM-Gap is already setup, the periodic MUSIM gap configuration shall be released before setup the new periodic Gap with the same Gap ID - When a periodic gap is setup, it shall be associated to the corresponding Gap ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the below description to the periodic Gap setup: - If an periodic MUSIM gap configuration associated with the musim-GapId indicated by the MUSIM-Gap is already setup, release the periodic MUSIM gap configuration; - Associate the periodic MUSIM gap with the musim-GapId indicated by the MUSIM-Gap Impact analysis Impacted 5G architecture options: Standalone NR Impacted functionality: MUSIM Gap Configuration Inter-operability: If the UE is implemented according to the CR and the network is not, there is no interoperability problem. If the Network is implemented according to the CR and the UE is not, the old Gap would not be released and leads to mismatch between UE and network, which may lead to data lost. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The old Gap would not be released and leads to mismatch between UE and network, which may lead to data lost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of change

#### 5.3.5.9a	MUSIM gap configuration

/******************************Omit the Unchanged Part**************************************/

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3> if an periodic MUSIM gap configuration associated with the musim-GapId indicated by the MUSIM-Gap is already setup, release the periodic MUSIM gap configuration;

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

3> associate the periodic MUSIM gap with the musim-GapId indicated by the MUSIM-Gap

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

1>	else if musim-GapConfig is set to release:

2>	release the MUSIM gap configuration.

# End of change