3GPP TSG-RAN2 Meeting #120	R2-2212747

Nov 14th – 18th, 2022 Toulouse FR

Source: 			ZTE Corporation, Sanechips

Title: 	Further Consideration on the Intra-band ENDC Capability

Agenda item:	6.0.2.1

Document for: 	Discussion and Decision

- Introduction

In the current UE capability, the intraBandENDC-Support was introduced with per BC level to indicate whether the UE support contiguous, non-contiguous or both as below.

| Definitions for parameters | Per |
| --- | --- |
| intraBandENDC-Support Enum{non-contiguous, both} Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. | BC |

However, as discussed in [1], there are some cases (e.g. case 3/4) that the UE would support different contiguous capability for the DL and UL.

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will has two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

In the plenary meeting, RAN2 and RAN4 were asked to confirm the validity of the case 3 and case 4.In the last RAN2 meeting, it has been agreed that

Agreements: RAN2 concludes that the discussed cases are not currently supported by signalling and new signalling is needed. Case validity is up to RAN4, and if RAN4 concludes they are valid, RAN2 can then attempt to find a signalling solution. RAN4 can also develop a preference as to what release should be applicable.

Meanwhile, in RAN4, the below agreements were achieved:

Agreement: - Keep b48+n48 in the RAN4 specification as it is - RAN4 seeks the solution to make such band combination workable. Depending on the solution, RAN4 decide how to handle this band combination in the specification. - It is agreed that Case 3 for b41+n41 to be removed

In this paper, we share our further consideration on this issue.

- Discussion

In this chapter, we first clarify the meaning when the UE set the intraBandENDC-Support to both, then share our views on DL/UL separation, at last we discuss whether some additional information is needed.

2.1 Legacy Signaling Clarification

According the discussion in the previous meeting, the key issue is how to indicate “intraBandENDC-Support” for the above Case 3 and 4. However, before any further discussion, we’d like to clarify the meaning when the UE set the intraBandENDC-Support to both first.

For the case with both contiguous and non-contiguous ENDC (e.g. DC_48A-(n)48AA), there is a note in RAN4 spec  as below (Note 5 in Table 5.3B.1.3-2 38101-3 see Annex)

NOTE 5: The UE supporting these configurations mixed intra-band continuous and non-continuous ENDC with at most two sub-blocks containing more than two component carriers indicates ‘both’ by IE intraBandENDC-Support.

So according to RAN4 spec, the “Both” means there are both contiguous and non-contiguous part in one BC. E.g.  DC_48A-(n)48AA3.

Observation 1: According to RAN4’s spec, the “Both” means there are both contiguous and non-contiguous ENDC in one BC.

However, from RAN2 aspect, the “both” can also be used to indicate that the UE supports both contiguous and non-contiguous BC. For example, if the UE supports both DC_(n)48AA5 and DC_48A_n48A3 (with the same other per BC capabilities and the same featrueSetCombination), the UE may only report one BC and set the intraBandENDC-Support to both.

E.g. BC with bandEntry[0] = 48A, bandEntry [1] = n48A and intraBandENDC-Support.= both.

| 1 | DC_(n)48AA5 |
| --- | --- |
| 2 | DC_48A_n48A3 |

Observation 2: In RAN2, the “Both” can be used to indicate both contiguous BC and non-contiguous BC to save UE capability signaling.

Proposal 1: For the intraBandENDC-Support, RAN2 to clarify the meaning of “Both” first:

- Option 1: “Both” means there are both contiguous and non-contiguous ENDC in one BC.

- Option 2: “Both” means both contiguous BC and non-contiguous BC with the same band Entries are supported.

For these two options, we can further take DC_48A-(n)48AA3 /DC_48A-48A_n48A3as example, the UE would report 3 band entries:

- BandEntrylist[0] =48A;

- BandEntrylist[1] =48A;

- BandEntrylist[2] =n48A

The UE would set the intraBandENDC-Support as below for the option 1/2 respectively.

Table 1: intraBandENDC-Support for the Option 1 and Option 2

| Index | Supported BC | Signaling with Option 1 | Signaling with Option 2 |
| --- | --- | --- | --- |
| 1 | Only support DC_48A-(n)48AA3 | intraBandENDC-Support = Both | intraBandENDC-Support = Contiguous |
| 2 | Only support DC_48A-48A_n48A3 | intraBandENDC-Support = non-Contiguous | intraBandENDC-Support = non-Contiguous |
| 3 | Support both DC_48A-(n)48AA3 and DC_48A-48A_n48A3 | Report with 2 BCs BC1: intraBandENDC-Support = Both BC2:intraBandENDC-Support = non-Contiguous | intraBandENDC-Support = Both |

So from RAN2 perspective, the option 2 is more effective, anyway, it also depends on the current UE implementation and if the option 2 selected, it would also have some impact to the above note 5 in RAN4 spec.

Proposal 2: Option 2 (i.e.“Both” means both contiguous BC and non-contiguous BC with the same band Entries are supported) is more effective from signaling overhead aspect.

Proposal 2a: If option 2 was selected, send an LS to RAN4 to indicate RAN2’s decision.

2.2 Signaling for DL and UL

In this sub-chapter, we’d discuss the DL/UL separation. According to the RAN4’s agreement, the BC with band 48 would be kept. For the band 48, the below case 1~7a in Table 2 were included in the latest RAN4 spec. With the legacy signaling, as shown in the Table 2, the case 1a/7/7a can’t be supported by the current signaling for that it supports different contiguous capabilities for the DL and UL.

Table-2 UE capability signaling for different EN-DC configurations

| Case | EN-DC configuration | Uplink EN-DC Configuration | Legacy can work? |
| --- | --- | --- | --- |
| 1 | DC_(n)48DA5 | DC_(n)48AA6 | Yes(Contiguous) |
| 1a | DC_(n)48DA5 | DC_48A_n48A6 | No |
| 2 | DC_(n)48AA5 | DC_(n)48AA6 | Yes(Contiguous) |
| 3 | DC_48A_n48A3 | DC_48A_n48A5 | Yes(Non-Contiguous) |
| 4 | DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes(Non-Contiguous) |
| 5 | DC_48C_n48A3 | DC_48A_n48A5 | Yes(Non-Contiguous) |
| 6 | DC_48D_n48A3 | DC_48A_n48A5 | Yes(Non-Contiguous) |
| 7 | DC_48A-(n)48AA3 | DC_(n)48AA5 | No |
| 7a | DC_48A-(n)48AA3 | DC_48A_n48A5 | No |

In [1], it suggested to introduce new capabilities to indicate the contiguous capability for intra-band EN-DC DL and UL separately. Take the above cases as an example:

Table 2:Intra-band EN-DC with New Capability

| Case | EN-DC configuration | Uplink EN-DC Configuration | Legacy can work? | New capability |
| --- | --- | --- | --- | --- |
| 1a | DC_(n)48DA5 | DC_48A_n48A6 | No | DL: Contiguous UL: Non-Contiguous |
| 7 | DC_48A-(n)48AA3 | DC_(n)48AA5 | No | DL: Contiguous(if Option 2 is selected) UL: Contiguous |
| 7a | DC_48A-(n)48AA3 | DC_48A_n48A5 | No | DL: Contiguous(if Option 2 is selected) UL: non-Contiguous |

However, no LS was received from the RAN4. Furthermore, according to the RAN4 agreements, RAN4 will “seek the solution to make such band combination workable. Depending on the solution, RAN4 decide how to handle this band combination in the specification.” Thus, RAN2 can wait for the RAN4’s LS before any new signaling design.

Proposal 3: Ran2 to wait for RAN4’s LS before introduction of new capability on UL/DL separation.

2.3 Signaling for the Contiguous Entry

Besides the above DL/UL separation issue, another issue is that for the case with both contiguous and non-contiguous ENDC BC, the network is unable to know which band entry of LTE is contiguous with NR. Take the above case 7: DC_48A-(n)48AA3 as an example, there would be 3 band Entries:

- BandEntrylist[0] =48A;    BandEntrylist[1] =48A;  BandEntrylist[2] =n48A

Each band entry may refer to a featureset e.g.

- BandEntrylist[0] =48A   Eutra Featureset 1with MIMO layer = 2, bandwidth = 100M

- BandEntrylist[1] =48A   Eutra Featureset 2 with MIMO layer = 4, bandwidth = 30M

- BandEntrylist[2] =n48A  NR featureSet  3 with MIMO layer = 2

Then the question is how does the network know which LTE entry is contiguous with NR entry. There would be 3 cases:

- Case 1: The UE only support contiguous EN-DC with BandEntrylist[0];

- Case 2: The UE only support contiguous EN-DC with BandEntrylist[1];

- Case 3: The UE support contiguous EN-DC with BandEntrylist[0] and contiguous EN-DC with BandEntrylist[1]

However, with the current capability signaling, the network doesn’t know which case is supported by the UE.

Observation 3: For the BC involves both contiguous and non-contiguous EN-DC, with the current signaling, the network can’t know which LTE band Entry can be contiguous with NR band entry.

Thus, from RAN2 aspect, it’s better for the UE to indicate which LTE band Entry is contiguous with NR band entry. RAN2 can also ask RAN4 to confirm this understanding.

Proposal 4: From RAN2 aspect, for the case with both contiguous and non-contiguous part, the UE shall indicate which LTE band Entry (or Entries) can be contiguous with NR band entry.

Proposal 4a: RAN2 ask RAN4 to confirm RAN2’s understanding in the proposal 4.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: According to RAN4’s spec, the “Both” means there are both contiguous and non-contiguous ENDC in one BC.

Observation 2: In RAN2, the “Both” can be used to indicate both contiguous BC and non-contiguous BC to save UE capability signaling.

Proposal 1: For the intraBandENDC-Support, RAN2 to clarify the meaning of “Both” first:

- Option 1: “Both” means there are both contiguous and non-contiguous ENDC in one BC.

- Option 2: “Both” means both contiguous BC and non-contiguous BC with the same band Entries are supported.

Proposal 2: Option 2 (i.e. “Both” means both contiguous BC and non-contiguous BC with the same band Entries are supported) is more effective from signaling overhead aspect.

Proposal 2a: If option 2 was selected, send an LS to RAN4 to indicate RAN2’s decision.

Proposal 3: Ran2 to wait for RAN4’s LS before introduction of new capability on UL/DL separation.

Observation 3: For the BC involves both contiguous and non-contiguous EN-DC, with the current signaling, the network can’t know which LTE band Entry can be contiguous with NR band entry.

Proposal 4: From RAN2 aspect, for the case with both contiguous and non-contiguous part, the UE shall indicate which LTE band Entry (or Entries) can be contiguous with NR band entry.

Proposal 4a: RAN2 ask RAN4 to confirm RAN2’s understanding in the proposal 4.

- Reference

- RP-222646  Discussion on intra-band EN-DC combination.   Huawei, HiSilicon.

- R2-2210538	Discussion on intra-band EN-DC combination	Huawei, HiSilicon	discussion	TEI17

- Annex

Table 5.3B.1.3-2: EN-DC configurations and bandwidth combination sets defined for intra-band EN-DC mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_48A-(n)48AA4,5 | DC_(n)48AA2 DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: The UE supporting these configurations mixed intra-band continuous and non-continuous ENDC with at most two sub-blocks containing more than two component carriers indicates ‘both’ by IE intraBandENDC-Support. |  |  |  |  |  |  |