3GPP TSG-RAN WG2 Meeting #120                                   R2-2212759

Toulouse, France, November, 2022

Agenda Item	: 8.5.2.2 (FS_NR_XR_enh)

Source	: LG Electronics Inc.

Title	: Discussion on the prioritization for XR

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the prioritization for XR traffic.

Discussion

UL grant enhancement for multiple streams

In TR38.838, XR traffic may include different type of streams (e.g., pose/control, scene, video, audio, and other data), which have different QoS requirements. For example, while the video/audio traffic may be generated in 60 fps (1 packet with 16.66ms periodicity), the pose/control data can be generated for each 4 ms. In addition, since each of type of steam may have different PDB, each stream should be handled differently.

- 5.5 AR traffic model … - 5.5.2.3 Model 3A (three streams model A) In Model 3A, three steams are considered. - Stream 1: pose/control - Traffic model/requirement for stream 1 follows clause 5.2. - Stream 2: A stream aggregating streams of scene and video - Follows the statistical parameters shown in Table 5.5-1. - Stream 3: A stream aggregating streams of audio and data Table 5.5.2.3-1: Statistical parameters for stream 3 of AR UL Model 3A (three streams model) Parameters; unit; value. Data rate: R; Mbps; 0.756, 1.12. Periodicity: P; ms; 10. Packet size; byte; mean packet size = R×1e6 × P/1000 / 8. PDB; ms; 30.

In order to support the multiple traffic streams with different QoS requirements, each stream could be allocated to different LCG with different priorities. In this case, when the BSR is transmitted to the network, the network can know the amount of data for each stream, since the BSR reports the volume of available data for each LCG.

On the other hand, current LCP procedure is processed with all logical channels in the UE. Thus, it cannot be guaranteed that the UE uses the received UL grant for a specific stream. In other words, even though the network allocates the uplink grant to meet the QoS requirement for a XR specific stream, the UE may allocate UL grant to transmit the data of other traffic streams, e.g., due to the current LCP procedure and the PBR operation.

Given that the satisfying QoS requirements of the XR traffic is one of the most important objective in XR, it would be beneficial to allocate the uplink grant with a fine control of traffic streams. Therefore, for better scheduling of each stream, it should be allowed to allocate the UL grant to a specific LCG/LCH/RB.

For this purpose, a new type of UL grant can be introduced targeting a specific LCG/LCH/RB. When the UE receives the UL grant targeting a specific LCG/LCH/RB, the UE generates a MAC PDU containing only data associated with the specific LCG/LCH/RB. With this, finer QoS control for XR traffic could be achieved.

Proposal 1. RAN2 should study an enhancement of UL grant in order to support the transmission of a specific LCG/LCH/RB.

BSR and UL grant enhancement considering the remaining time

In XR, a PDU set is an important decoding unit and all SDUs belonging to a PDU set should be transmitted within the PSDB because the frame/video slice may only be decoded in case all packets carrying the frame/video slice are successfully delivered in time. In other words, the application layer may not decode the PDU set if at least one SDU belonging to the PDU set is not successfully transmitted within the PSDB.

Observation 1. All SDUs associated with a PDU set should be successfully transmitted within the PSDB.

According to the current MAC specification, the MAC entity allocates UL grant to a LCH in decreasing priority order of the LCH. Therefore, even if a PSDB of a MAC SDU will expire soon, the MAC entity may generate the MAC PDU without containing the MAC SDU due to LCH priority.

Observation 2. Current LCP procedure does not consider PSDB.

Based on observation 1 and 2, we think the current LCP procedure may not be sufficient for XR. Even if the LCH priority is low, if the remaining time is small, this LCH should be transmitted prior to other LCHs.

For this purpose, we think enhancements to BSR and UL grant are needed. That is, the UE reports a BSR, e.g. emergency BSR, for the data whose remaining time is smaller than a threshold. Then, based on the emergency BSR information, the network provides a UL grant only for the data whose remaining time is smaller than a threshold.

Proposal 2. RAN2 should study an enhancement of BSR and UL grant in order to support the transmission of data whose remaining time is smaller than a threshold.

Conclusion

In this contribution, we show our views on the prioritization for XR traffic.

Proposal 1. RAN2 should study an enhancement of UL grant in order to support the transmission of a specific LCG/LCH/RB.

Observation 1. All SDUs associated with a PDU set should be successfully transmitted within the PSDB.

Observation 2. Current LCP procedure does not consider PSDB.

Proposal 2. RAN2 should study an enhancement of BSR and UL grant in order to support the transmission of data whose remaining time is smaller than a threshold.