3GPP TSG-RAN WG2 #120	R2-2212892

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	6.11.3

Source:	Ericsson

Title:	Integrity measurements definition and missing integrity requirements

Document for:	Discussion, Decision

# 1	Introduction

Support for positioning integrity in general was introduced in Rel 17, while only supported for GNSS. Attributes, configuration parameters and KPIs have been agreed and refined at RAN2#117-e, and were further discussed at RAN2#118-e and RAN2#119-e. From discussions at RAN2#119-bis-e, it was also evident that companies have different understanding of what has been agreed and how measurement definitions shall be understood.

The WID on NR positioning enhancements includes the following objective

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]:

  - The assistance information that will be used to support integrity determination

  - The information that will be used to provide the positioning integrity KPIs and integrity results

  - Support of integrity for UE-based and UE-assisted A-GNSS positioning.

Note: This objective is applicable to NR and E-UTRA.

In this contribution, we address the following matters

- The definition of the Protection Level measurement as part of the integrity KPIs is implicit and not clear enough to avoid confusion

- The network assistance to devices to determine the position estimate reliability is incomplete

# 2	Discussion

## 2.1	Integrity KPI definitions for UE-assisted reliability determination

At RAN2#118-e, the following was agreed in relation to the definition of the Protection Level (PL):

Agreements:

Proposal 3 (modified)	Move the PL definition to TS 38.305/36.305.

Proposal 4	Support appending “protection level and achievable target integrity risk” to 38.305 Section 7.3.4, step 1 paragraph

Proposal 5 (modified)	Support the suggested change to Table 8.1.2.1b-1 and equivalent table in 36.305.

[Chair’s note: This proposal refers to the changes as amended by Swift in the email discussion; see the report for full changes]

36.305 changes to be merged into rapporteur CR.

Agreement:

Keep the definition of PL, and clarify in a NOTE that the PL inequality is valid for all values of the AL.

Extend discussion to Thursday 2022-05-19 0400 UTC to discuss whether P2 is needed.

Now, the following text is in TS 38.05:

Protection Level (PL): A statistical upper-bound of the Positioning Error (PE) that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the required TIR, i.e., the PL satisfies the following inequality: 
	Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < required TIR
When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.
A specific equation for the PL is not specified as this is implementation-defined. For the PL to be considered valid, it must simply satisfy the inequality above.

NOTE: the PL inequality is valid for all values of the AL.

This is an implicit definition of a KPI that UEs can be configured to report to the network based on assistance data including the target integrity risk and information to support determination of the positioning error distribution. This inequality is defined for a general parameter AL and is valid for the set of all possible values of AL. Furthermore, the definition includes “per unit of time” and “TTA”, which are not provided as part of the assistance data.

The definition of the integrity KPI Protection Level is implicit and contains parameters not provided in the assistance data.

It can be questioned if this definition of Protection Level is sufficient, given the interpretation freedom and also whether the device has all the necessary information so that the reported values can be seen as consistent and can be comparable over a set of devices by different vendors, or a set of different environments or parameter configurations. The discussion around the achievable TIR at RAN2#119-bis-e [2] also revealed that companies have different understanding of how the PL and achievable TIR are determined.

It can be questioned if the current definition of PL is sufficient, given the interpretation freedom and missing information in the assistance data.

It can also be questioned if the current achievable TIR definition is sufficient.

Since the KPI Protection Level is a measurement, it is important that it is clearly defined. There is no reason to why a measurement shall be defined as vague as PL currently is. Instead, it is advised to agree on an explicit definition of this measurement. The complementary cumulative density function (CCDF) of the positioning error FPEPE>x needs to be below TIR, and PL is defined as the minimizing argument x – the smallest x among all x that satisfies  FPEPE>x<TIR

PL= argminx FPEx<TIR=argminx ProbPE>x<TIR

Typically, TIR is a very low number, meaning that it can be difficult to determine the PL that exactly corresponds to TIR. For example, it might only be possible to determine the CCDF at discrete values of x. The reported PL based on the definition may therefore correspond to a TIR slightly smaller than the provided TIR. This is the reported achievable TIR.

Figure 1 illustrates the definition of PL, and also how a discrete calculation of the CCDF gives a different achievable TIR than the provided TIR requirement.

Figure 1.Determination of PL and achievable TIR. The objective is to determine a value matching the required TIR (rTIR) – the rPL. However, in case the CCDF calculations are discretized, the calculation ends up in a PL (aPL) that is slightly larger and also corresponds to an achievable TIR (aTIR) that is slightly smaller than the required TIR.

Therefore, it is recommended to update the stage 2 (TS 36.305 and TS 38.305) definitions of PL and achievable TIR according to:

Protection Level (PL): A statistical upper-bound of the Positioning Error (PE) corresponding to a probability of target integrity risk or lower that the positioning error is outside the bound. The KPI PL is defined based on the complementary cumulative density function (CCDF) of the positioning error (PE) FPEx being bound by TIR from above. Explicitly

PL= argminx FPEx<TIR=argminx ProbPE>x<TIR

NOTE: In case the determined PL is not perfectly matching the TIR requirement, then the UE will also report the achievable TIR defined as FPEPL.

We have the following proposal

Update the definition of PL and achievable TIR is stage 2 specifications as in the text proposal in Appendix A.

## 2.2	Network assistance to UE-based position estimate reliability determination

The integrity concept is about to determine whether measurements, estimates or information can be trusted for reliable use. As confirmed at RAN2#119-e, the agreed KPI mode 1 concerns UE determination and reporting of PL to the network as part of location information. This can enable the network to determine reliability of a reported position estimate from the device by comparing to the Alert Limit (AL).

However, there should also be considerations of a time to alert timer (TTA) - to alert that a position estimate is not reliable if the inequality PL> AL is fulfilled over a time period of TTA.

Position estimate reliability is assessed by comparing PL to AL and declare that the position estimate is not reliable if PL > AL over a time given by TTA.

Clearly, network based position estimate reliability determination needs very frequent position estimate and PL reporting to the network in order to be relevant.

A more practical alternative in case the beneficiary of the reliability information is the device itself, is to determine the position estimate reliability in the device. In such a case, the following aspects typically apply:

- UE-based GNSS position estimation is configured (with Rel 18 completed, UE-based RAT-dependent position estimation is an option)

- The position information can be determined very frequently and integrity results can be determined almost continuously, allowing a continuous position estimate reliability determination, provided that integrity requirements in terms of TIR, AL and TTA are available

  - TIR, AL and TTA can be pre-configured or made available from higher device layers

  - TIR, AL and TTA could be provided via LPP

- Position estimate reliability can be directly reported to higher layers of the device to promptly notifying layers using the position estimate, for example in safety critical use cases

- The agreed mode 1 PL reporting can be combined to allow the network to gather some SLA information

UE-based integrity with position estimate reliability determination has many advantages suitable for navigating devices. However, TIR, AL and TTA are needed, either pre-configured, proprietary from higher layers, or provided by the network.

UE-based with position estimate reliability determination is incomplete without signalling of TIR, AL and TTA

It is important to note that the need for TIR, AL and TTA to make UE-based integrity complete involves adding AL and TTA to the ProvideAssistanceData. This is different from the mode 1 and mode 2 reporting discussion, where mode 2 would have required AL and TTA to be included in the RequestLocationInformation. Mode 2 was not agreed, and even if it would have been agreed, it would not have made UE-based integrity complete, since it requires TIR, AL and TTA to be included in the ProvideAssistanceData.

Based on the discussion, we have the following proposal

Add TIR, AL and TTA to the integrity assistance data to make UE-based integrity complete

The TIR, AL and TTA inclusion was discussed at RAN2#119-e [1], and one main concern was whether the UE was required to use the provided TIR, AL and TTA or not. Therefore, we would like to make this aspect very clear by proposing the following logic

- If the device indicates a capability to support integrity reliability determination OR if the device requests integrity requirements for integrity reliability determination, THEN the device shall use provided integrity requirements for integrity reliability determination

- If the device does not indicate capability AND does to request integrity requirements, THEN the device uses other available integrity requirements for integrity reliability determination

The logic would be represented in field descriptions of the integrity requirements attributes TIR, AL and TTA

Define in TIR, AL and TTA field descriptions that these are mandatory to use for integrity reliability determination if the device has indicated a corresponding capability or has requested for TIR, AL and TTA as part of the assistance data.

The required LPP updates to support integrity requirements to make UE-based integrity complete are provided in Appendix B.

Agree to the 37.355 text proposal in Appendix B to make UE-based integrity complete

# Conclusion

In the previous sections we made the following observations:

Observation 1	The definition of the integrity KPI Protection Level is implicit and contains parameters not provided in the assistance data.

Observation 2	It can be questioned if the current definition of PL is sufficient, given the interpretation freedom and missing information in the assistance data.

Observation 3	It can also be questioned if the current achievable TIR definition is sufficient.

Observation 4	Position estimate reliability is assessed by comparing PL to AL and declare that the position estimate is not reliable if PL > AL over a time given by TTA.

Observation 5	UE-based integrity with position estimate reliability determination has many advantages suitable for navigating devices. However, TIR, AL and TTA are needed, either pre-configured, proprietary from higher layers, or provided by the network.

Observation 6	UE-based with position estimate reliability determination is incomplete without signalling of TIR, AL and TTA

Based on the discussion in the previous sections we propose the following:

Proposal 1	Update the definition of PL and achievable TIR is stage 2 specifications as in the text proposal in Appendix A.

Proposal 2	Add TIR, AL and TTA to the integrity assistance data to make UE-based integrity complete

Proposal 3	Define in TIR, AL and TTA field descriptions that these are mandatory to use for integrity reliability determination if the device has indicated a corresponding capability or has requested for TIR, AL and TTA as part of the assistance data.

Proposal 4	Agree to the 37.355 text proposal in Appendix B to make UE-based integrity complete

# Appendix A – Text Proposal to TS 36.305/TS 38.305

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

[…]

Protection Level (PL): A statistical upper-bound of the Positioning Error (PE) corresponding to a probability of target integrity risk or lower that the positioning error is outside the bound. The KPI PL is defined based on the complementary cumulative density function (CCDF) of the positioning error (PE) FPEx being bound by TIR from above. Explicitly

PL= argminx FPEx<TIR=argminx ProbPE>x<TIR

NOTE: In case the determined PL is not perfectly matching the TIR requirement, then the UE will also report the achievable TIR defined as FPEPL.

# Appendix B – Text Proposal to 37.355

### 6.4.1	Common Lower-Level IEs

[…]

#### –	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

#### –	IntegrityRequirementsSup

The IE IntegrityRequirementsSup is used to indicate support of position estimate reliability determination based on integrity requirements attributes provided in the assistance data.

-- ASN1START

IntegrityRequirementsSup-r17::= SEQUENCE {

...

}

-- ASN1STOP

### 6.4.2	Common Positioning

[…]

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]],

[[

integrityRequirementsSup-r17	IntegrityRequirementsSup-r17	OPTIONAL	-- Cond IntReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| IntReq | This field is mandatory present if the target device supports IntegrityRequirements; otherwise it is not present. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

integrityRequirementsSup This field specifies that position estimate reliability determination based on provided integrity requirements is supported by the target device.

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]],

[[

integrityRequirementsReq-r17	NULL					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |
| IntReqReq | This field is mandatory present if the target device requests integrity requirements for position estimate reliability determination; otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.

integrityRequirementsReq This field, if present, indicates a request for integrity requirements..

#### –	CommonIEsProvideAssistanceData

The CommonIEsProvideAssistanceData carries common IEs for a Provide Assistance Data LPP message Type.

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

periodicAssistanceData-r15	PeriodicAssistanceDataControlParameters-r15

OPTIONAL	-- Cond PerAD

]],

[[

integrityRequirements-r17	IntegrityRequirements-r17	OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerAD | The field is mandatory present in a periodic assistance data delivery session. Otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

segmentationInfo This field indicates whether this ProvideAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceData This field indicates a periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

integrityRequirements This field provides integrity requirements which the device shall use for UE-based position estimate reliability deterimation if the device has indicated support via IntegrityRequirementsSup or requested the integrity requirements via integrityRequirementsReq.

[…]

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	DL-PRS-ID-Info

The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-ID-Info-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-DL-PRS-ResourceID-List-r16	SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF

NR-DL-PRS-ResourceID-r16

OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16

OPTIONAL  -- Need ON

}

-- ASN1STOP

DL-PRS-ID-Info field descriptions

nr-DL-PRS-ResourceID-List This field provides a list of DL-PRS Resource IDs under the same DL-PRS Resource Set.

#### –	IntegrityRequirements

The IE IntegrityRequirements provides the requirement parameters the device needs to assess positioning integrity

-- ASN1START

IntegrityRequirements-r17 ::= SEQUENCE {

targetIntegrityRisk-r17     		INTEGER (10..90),

horizontalAlertLimit-r17			INTEGER (0..50000),

verticalAlertLimit-r17				INTEGER (0..50000)				OPTIONAL, -- Need ON

timeToAlert-r17			        	INTEGER (1,2000)				OPTIONAL, -- Need ON

...

}

-- ASN1STOP

IntegrityRequirements field descriptions

targetIntegrityRisk This field indicates the Target Integrity Risk (TIR) for the integrity principle of operation by the device. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

horizontalAlertLimit This field indicates the horizontal alert limit for the integrity principle of operation by the device along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres.

verticalAlertLimit This field indicates the vertical alert limit for the integrity principle of operation by the device. Scale factor 0.01 metre; range 0 – 500 metres. To be compared to the horizontal protection level determined by the device.

timeToAlert The maximum allowable elapsed time from when the protection level (PL) exceeds the Alert Limit (AL) until the function providing positioning integrity annunciates a corresponding alert. Scale factor 0.1 second.