3GPP TSG RAN2 Meeting #120	R2-2212911

Toulouse, 14 – 18 November, 2022

Agenda item:		8.19

Source:	Deutsche Telekom, Ericsson, Telecom Italia, Thales

Title:	Considerations on SENSE feature

Document for:		Discussion and Decision

# Introduction

In the previous RAN2 meeting, an LS on SENSE feature was received from CT1 [1] asking RAN2 to look into the SA1 requirement for SENSE feature and provide feedback on RAN2 impact, if any. RAN2 could not reach an agreement and a decision is expected to be taken at the next meeting [2].

In this contribution, our intention is to provide some further points for consideration, from an operator point of view, when discussing the SENSE feature and possible implications on RAN2 procedures, taking also into account potential massive impact on established operator deployments.

# Discussion

The Signal Level Enhanced Network Selection (SENSE) feature was introduced to address data connectivity problems experienced in the field by an increasing number of stationary and non-supervised devices (i.e., immobile IoT devices) due to current network selection procedures that consider only the cell selection criteria as broadcast by the PLMN and the priority of networks. To overcome this problem, SA1 studied and introduced a requirement for Rel-18 SENSE [3] that allows the network operator to take the signal level into account during the initial steps of network selection, after switch-on or recovery from loss of coverage and during all steps of the periodic re-selection. Specifically, the feature allows the home network operator to configure a single threshold, "Operator controlled signal threshold per access technology" (on the USIM), specific for a certain Access Technology and applicable to all PLMNs with the corresponding access technology combinations.

From an operator’s point of view, the PLMN selection and related (suitable) cell selection and reselection procedures are well-established procedures, already functioning properly since UMTS (or even GERAN) times and taken into account for highly optimised cell planning, e.g., in many airports or country border crossings, where PLMN selection mainly occurs. For many years, MNOs have planned properly these areas so that mobile subscribers are offered seamlessly services of high quality, while allowing an optimised PLMN selection performance to the UE. Any possible change on current PLMN selection and related (suitable) cell selection and reselection procedures should not create inconsistencies between old and new solutions/terminals; especially operators do not see any need to touch these long-established procedures which were consistent over 4 generations of mobile technologies.

Observation 1: The PLMN selection and related (suitable) cell selection and reselection procedures are well-established procedures, already functioning properly and configured in many airports or country border crossings where PLMN selection mainly occurs. Any possible change on current PLMN selection and related (suitable) cell selection and reselection procedures should not create inconsistencies between old and new solutions/terminals.

Given that the SENSE feature refers to network selection, that is a NAS procedure, and aims to address a very specific issue identified for stationary IoT devices, which are in a permanent roaming situation (i.e., global IoT USIM), as analysed in [4], we see no impact on legacy interfaces between upper layers and AS layer for support of PLMN selection. Further, there is no need for any further optimisation in RAN2 for the cell selection process, as the well optimised network parameterisation will ensure that the UE will select a suitable cell, while the subsequent cell reselection process will bring the UE to the most suitable cell anyway.

If the SENSE threshold (which is acquired from e.g., the SIM card) were to be used in cell (re)selection procedures, this could cause inconsistent behaviour. This is because the RAN will attempt to steer UEs using parameters in system information, but some (to the RAN, seemingly random set of) UEs would behave differently if they were to apply the SENSE-threshold in cell (re)selection. This “interference” by the SENSE-threshold with the cell (re)selection related parameters in system information would create problems for the operator to tune their networks.

Proposal 1: Having analysed the SENSE feature and its applicable use case scenarios (stationary UEs), no impact on RAN2 specifications is foreseen to support the SENSE feature, and no further need for changes to the well-established cell selection procedures have been identified justifying additional work in RAN2.

# Summary and Conclusions

In this document, we have presented some further points for consideration, from an operator point of view, when discussing the SENSE feature and possible implications on RAN2 procedures and we propose the following:

Observation 1: The PLMN selection and related (suitable) cell selection and reselection procedures are well-established procedures, already functioning properly and configured in many airports where PLMN selection mainly occurs. Any possible change on current PLMN selection and related (suitable) cell selection and reselection procedures should not create inconsistencies between old and new solutions/terminals.

Proposal 1: Having analysed the SENSE feature and its applicable use case scenarios (stationary UEs), no impact on RAN2 specifications is foreseen to support the SENSE feature, and no further need for changes to the well-established cell selection procedures have been identified justifying additional work in RAN2.

# References

[1]		R2-2209304, “LS on SENSE feature” (C1-225338)

[2]       R2 119bis-e Chair Notes

[3]		3GPP TS 22.011, “Service Accessibility”

[4]		R2-2210618, “Discussion on SENSE feature”