3GPP TSG-RAN WG2 #120	R2-2212996

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.16.1

Source:	Ericsson, Qualcomm Inc.

Title:	Rapporteur remarks and contributions overview

Document for:	Discussion, Decision

# 1	Introduction

As expected from what is proposed by the Work Plan (R2-2210677), most inputs to the “AIML for NR Air Interface” SI in RAN2 have been provided to the “AIML Methods” sub item (AI 8.16.2). For this sub-AI, the Rapporteur identifies that the following are the most addressed topics/aspects:

- NW-UE collaboration levels,

- model transfer/delivery, and

- model ID and its usage.

The last two, evidently come from the following agreements/FFSs reached during RAN2#119-bis-e:

R2 assumes that a model is identified by a model ID. Its usage is FFS. General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

In a perhaps lesser extent and among diverse topics, contributions also cover the following main aspects:

- data collection,

- model registration,

- mobility-related aspects.

The Rapporteur believes that RAN2 could spend some time discussing the topics mentioned above. For this, some observations and proposals are included in this document.

However, as indicated by the Chair, the Rapporteur would like to emphasize that this document is not intended as a summary of the contributions that have been submitted to this Agenda Item. But rather with the hopes that it can provide some “guide” for the online discussion.

On the other hand, we notice that less documents were submitted to the “Use case specific aspects” sub item (AI 8.16.3). And since RAN2 did not address this during the previous meeting, most proposals are related to what RAN2 should focus on. The discussion is then, arguably, kept in a high-level. The Rapporteur also provides some general proposals on these matters, but by acknowledging that there is a need for further RAN1 input/development to further progress with the use cases in RAN2.

# 2	Discussion

## 2.1	AIML scenarios

To have “a clearer picture” let us come back to the scenarios identified for the use cases.

According to the Rapporteur’s understanding the “AIML settings” that will be discussed are the describes by:

- One-sided AI/ML model

  - The AIML operation/functionality resides, solely, in the UE or in the NW side.

- Two-sided model

  - The AIML operation/functionality resides simultaneously in the UE and in the NW side.

For these “settings”, RAN1 agreed the following 3 levels of NW-UE collaboration (RAN1#109-e’s agreement):

Take the following network-UE collaboration levels as one aspect for defining collaboration levels 1. Level x: No collaboration 2. Level y: Signaling-based collaboration without model transfer 3. Level z: Signaling-based collaboration with model transfer

As per the “AIML settings”, the NW-UE collaboration levels above, and the use cases discussed in RAN1, the Rapporteur observes that RAN2 could consider the following scenarios:

- Scenario 1: one-sided AI/ML model at NW side or UE side (there should be no spec. impact)

- Scenario 2: One-sided AI/ML model at NW side, with UE-related model Life Cycle Management (LCM) signalling,

- Scenario 3: One-sided AI/ML model at UE side, with NW-related model LCM signalling,

- Scenario 4: Two-sided AI/ML model.

Where for each the different procedures (e.g., data collection, model monitoring, model transfer, etc.) could eventually apply.

For clarity, the Rapporteur would like to highlight that during RAN1#109, the following definition was agreed for “model transfer”:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

While the following definition (Working Assumption) was made for “model delivery” during RAN1#110:

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

For “AIML methods” and the different use cases, RAN2 could refer to the following scenarios. Scenario 1: one-sided AI/ML model at NW side or UE side (none or limited spec. impact). Scenario 2: One-sided AI/ML model at NW side, with UE-related model LCM signalling. Scenario 3: One-sided AI/ML model at UE side, with NW-related model LCM signalling. Scenario 4: Two-sided AI/ML model, with model LCM signalling.

As per RAN1’s ‘model transfer’ definition, RAN2 might need to wait for RAN1 input on whether delivery may contain a full model or a partial model (e.g., some parameters of a model structure).

## 2.2	NW-UE collaboration

RAN1 agreed the following during RAN1#110bis-e (see R1-2210690):

Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

By focusing on the highlighted text above, and as per companies’ input, the Rapporteur observers the following.

When there is no NW-UE collaboration (i.e., level ‘x’), RAN2 could in principle assume that no specification impact.

The above leaves RAN2 to focus on levels ‘y’ and ‘z’. To differentiate these cases, RAN1 has captured the following during RAN1#110bis-e:

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

According to this, it should then be quite straightforward for RAN2 to agree the following.

RAN2 to focus on analysing signalling for NW-UE collaboration. For this consider the cases when the AI/ML model delivery is transparent to 3GPP signalling and when it is not.

While waiting for RAN1 input, RAN2 to study procedures to support the delivery of an AI/ML model from one entity to another.

## 2.2	Model transfer and delivery

Contributions discuss model transfer and delivery aspects given the following agreement reached during the last meeting:

General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

On this matter, though, the Rapporteur would like to emphasize that model transfer discussions are still ongoing in RAN1. This includes feasibility aspects of this.

RAN2 then have no clarity on what a model is, its format, or even the eventual requirements for transferring one.

However, for the sake of progress it seems advisable to start with a “general” (high-level) discussion in RAN2, perhaps focusing on model delivery.

RAN1 discussion on ‘model transfer’ is ongoing (even the feasibility is debated in that WG). Therefore, since there is no clarity in RAN2 as to what a model is, for the time being we can keep a high-level discussion on this matter. And, perhaps, focus on ‘model delivery’ aspects.

Considering the above, and some of the companies’ proposals, RAN2 could start by discussing the following.

While waiting for RAN1 input, RAN2 to further develop and analyse pros and cons of delivering AIML models using: (A) CP: RRC-based solutions. (B) UP: analyse potential enhancements needed and spec. impact. FFS on (sub)solutions.

For next meeting, RAN2 should start by studying associated procedures for supporting CP or UP methods for model delivery. Motivations and drawbacks for each should be clearly stated.

## 2.3	Model ID

While RAN2 agreed the following during the last meeting:

R2 assumes that a model is identified by a model ID. Its usage is FFS.

The Rapporteur would like to stress that RAN1 reached the following agreement during RAN1#110bis-e (see R1-2210690):

Agreement Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models FFS: Detailed discussion of model ID with associated information and/or model functionality. FFS: usage of model ID with associated information and/or model functionality based LCM procedure FFS: whether support of model ID FFS: the detailed applicable AI/ML operations

Therefore, even when challenging, it would be advisable to try to limit the discussion strictly to RAN2-related matters. Then comes the question: “What is within RAN2 scope regarding these IDs?”.

On this matter, and taking companies’ input, it appears that this could be related to the uniqueness of the IDs, the formats and what they indicate (e.g., supported features, vendor info., etc.), their usage towards Life Cycle Management (LCM), etc. Therefore, the Rapporteur suggests starting by discussing the following.

While waiting for RAN1 input, RAN2 to discuss views on “model IDs”. Start by discussing the ID needs and what an ID indicates (e.g., a model?, a feature/functionality?, etc). From this, RAN2 could discuss on whether to have a standardized ID, (globally) unique, mandatory, etc.

## 2.4	Data Collection

While companies’ documents discuss this aspect less than the ones above, some concrete proposals have been made.

For this reason, RAN2 can start by spending some time discussing around the following.

While waiting for RAN1 input, RAN2 to study CP and UP solutions for data collection. Motivations to reuse existing mechanisms or introduce new ones should be clearly stated. RAN2 to consider: UE reports/configuration, additional (new) data that could be required by NW and UEs, entities that will collect the data, etc.

For data collection, RAN2 should consider the use cases’ main “functionality” and their concerning Life Cycle Management aspects. From this, RAN2 can then analyse whether different solutions or a general solution could be used.

## 2.5	Model registration and mobility aspects

RAN1 is currently discussing how to define “model registration”. However, as brought up in some companies’ contributions, RAN2-related signalling and procedures could be needed for a UE to be able to “register to a AIML model”.

The Rapporteur acknowledges that there are still several questions marks on this matter, e.g., What is a model? How is a model identified? And, even, what is meant by ‘model registration’.

As this topic has been brought up in companies’ contributions to this meeting, it appears beneficial to understand the motivations to discuss this in RAN2 at this point.

In this regard, the Rapporteur’s view is that there might not even be a real need for a UE to “register to a model”. In fact, the procedure could instead be linked to allowing a UE to indicate support for AIML-based features (and the eventual feature “version ID” or, “model ID” in case if multiple versions/models are supported for the feature), and then to enable the UE to get such feature via configuration.

Discuss what companies understand by “model registration” and whether this is within RAN2 scope.

Depending on the outcome of the previous proposal, RAN2 could eventually discuss the following.

Depending on the outcome of P8, RAN2 can study solutions and specification impact for model registration. For this consider scenarios on which this is needed, potential signalling enhancements, entities that could be involved, and the information that is needed to be exchanged from entity to entity, etc.

Furthermore, some companies have proposed to analyse impacts related to UE mobility when considering different AIML related procedures and operation, e.g., for model transfer, model selection, etc. At this point it seems not fully clear; how and what will RAN2 need to discuss regarding mobility aspects. However, mobility are within RAN2 reach. Therefore, the Rapporteur proposes the following.

RAN2 could discuss UE mobility aspects when encountered with any specification impact related to operating AIML for PHY procedures.

## 2.4	Use case-related discussion

### 2.4.1	CSI feedback enhancements

As per RAN1 agreements and companies’ input to AI 8.16.3, the Rapporteur suggests starting by addressing the following.

Focus on Spatial-Frequency domain CSI compression using two-sided AI model. RAN2 can start by studying configuration enhancements, signalling needed for UE/gNB data collection, mechanisms for performance monitoring in UE/gNB (pending RAN1 input on KPIs to monitor), signalling needed for model performance monitoring.

### 2.4.2	Beam management enhancements

As per RAN1 agreements and companies’ input to AI 8.16.3, the Rapporteur suggests starting by addressing the following.

Study RAN2-related impacts to Spatial-domain DL beam prediction and, to Temporal DL beam prediction.

For the beam management use cases, RAN2 can start by studying configuration enhancements, signalling needed for UE/gNB data collection, mechanisms for performance monitoring in UE/gNB (pending RAN1 input on KPIs to monitor), signalling needed for model performance monitoring.

### 2.4.3	Positioning accuracy enhancements

RAN1 has agreed to study the following approaches:

- Direct AI/ML positioning

  - The AI/ML model is directly producing the UE location as output

- AI/ML assisted positioning

  - The AI/ML model is producing an existing or new measurement report that is used to estimate the UE location using legacy positioning methods (e.g., triangulation).

For these, the following cases have been agreed by RAN1, which can be mapped to the Scenarios in Proposal 1:

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

It seems clear to the Rapporteur that further RAN1 input is needed for RAN2 to get into details discussions on positioning matters. But as per the proposals observed in AI 8.16.2 and 8.16.3, RAN2 could start by discussing the following.

While we wait for RAN1 input, study RAN2-concerned entities involved in AIML-based positioning cases. From this, consider whether signalling and procedure enhancements are needed for data collection and model monitoring.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	For “AIML methods” and the different use cases, RAN2 could refer to the following scenarios. Scenario 1: one-sided AI/ML model at NW side or UE side (none or limited spec. impact). Scenario 2: One-sided AI/ML model at NW side, with UE-related model LCM signalling. Scenario 3: One-sided AI/ML model at UE side, with NW-related model LCM signalling. Scenario 4: Two-sided AI/ML model, with model LCM signalling.

Observation 2	As per RAN1’s ‘model transfer’ definition, RAN2 might need to wait for RAN1 input on whether delivery may contain a full model or a partial model (e.g., some parameters of a model structure).

Observation 3	When there is no NW-UE collaboration (i.e., level ‘x’), RAN2 could in principle assume that no specification impact.

Observation 4	RAN1 discussion on ‘model transfer’ is ongoing (even the feasibility is debated in that WG). Therefore, since there is no clarity in RAN2 as to what a model is, for the time being we can keep a high-level discussion on this matter. And, perhaps, focus on ‘model delivery’ aspects.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to focus on analysing signalling for NW-UE collaboration. For this consider the cases when the AI/ML model delivery is transparent to 3GPP signalling and when it is not.

Proposal 2	While waiting for RAN1 input, RAN2 to study procedures to support the delivery of an AI/ML model from one entity to another.

Proposal 3	While waiting for RAN1 input, RAN2 to further develop and analyse pros and cons of delivering AIML models using: (A) CP: RRC-based solutions. (B) UP: analyse potential enhancements needed and spec. impact. FFS on (sub)solutions.

Proposal 4	For next meeting, RAN2 should start by studying associated procedures for supporting CP or UP methods for model delivery. Motivations and drawbacks for each should be clearly stated.

Proposal 5	While waiting for RAN1 input, RAN2 to discuss views on “model IDs”. Start by discussing the ID needs and what an ID indicates (e.g., a model?, a feature/functionality?, etc). From this, RAN2 could discuss on whether to have a standardized ID, (globally) unique, mandatory, etc.

Proposal 6	While waiting for RAN1 input, RAN2 to study CP and UP solutions for data collection. Motivations to reuse existing mechanisms or introduce new ones should be clearly stated. RAN2 to consider: UE reports/configuration, additional (new) data that could be required by NW and UEs, entities that will collect the data, etc.

Proposal 7	For data collection, RAN2 should consider the use cases’ main “functionality” and their concerning Life Cycle Management aspects. From this, RAN2 can then analyse whether different solutions or a general solution could be used.

Proposal 8	Discuss what companies understand by “model registration” and whether this is within RAN2 scope.

Proposal 9	Depending on the outcome of P8, RAN2 can study solutions and specification impact for model registration. For this consider scenarios on which this is needed, potential signalling enhancements, entities that could be involved, and the information that is needed to be exchanged from entity to entity, etc.

Proposal 10	RAN2 could discuss UE mobility aspects when encountered with any specification impact related to operating AIML for PHY procedures.

Proposal 11	Focus on Spatial-Frequency domain CSI compression using two-sided AI model. RAN2 can start by studying configuration enhancements, signalling needed for UE/gNB data collection, mechanisms for performance monitoring in UE/gNB (pending RAN1 input on KPIs to monitor), signalling needed for model performance monitoring.

Proposal 12	Study RAN2-related impacts to Spatial-domain DL beam prediction and, to Temporal DL beam prediction.

Proposal 13	For the beam management use cases, RAN2 can start by studying configuration enhancements, signalling needed for UE/gNB data collection, mechanisms for performance monitoring in UE/gNB (pending RAN1 input on KPIs to monitor), signalling needed for model performance monitoring.

Proposal 14	While we wait for RAN1 input, study RAN2-concerned entities involved in AIML-based positioning cases. From this, consider whether signalling and procedure enhancements are needed for data collection and model monitoring.

# 4	References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec 2021

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#96, Budapest, Hungary, June 2022

R1-2205695, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)	“, Ad-hoc Chair (CMCC), RAN1#109-e, May 2022

R1-2208145, “Session notes for 9.2 (Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#110, Toulouse, August 2022

R2-2210677, “RAN2 Work Plan for Rel-18 SI on AI/ML for NR air interface”, Ericsson, Qualcomm Inc., RAN2#119-e, Oct. 2022

R1-2210690, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, CMCC, RAN1#110bis-e, Oct. 2022