3GPP TSG-RAN WG2 Meeting #120	R2-2213009

Toulouse, France, November, 2022

Source: 	Session Chair (Apple)

Title:	Report from NC Repeater breakout session

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT120][700][NCR] Organisational Sasha – NCR (Apple)

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline:  Friday 2022-11-25 1700

[AT120][701][Follow up on discussions]  (ZTE)

Scope:

- Further details of the agreed proposals

- All the proposals which have not concluded online

- Can also discuss other proposals from the papers treated online

Intended outcome: summary for the online CB session with hopefully agreeable proposals, report in R2-2213061

Deadline:  CB session

## 8.1	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-222673)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.1.1	Organizational

Including LSs and any rapporteur inputs.

Note: LS R2-2211173 is moved to AI 8.1.3.

### 8.1.2	Signalling for side control information

Signalling and procedures for for side control information, based on RAN1 agreements. Additionally, any other RAN2 reletated aspects, if needed.

Note: the following documents will be discussed online.

R2-2211908	Consideration on NCR open issues	ZTE Corporation, Sanechips	discussion	Rel-18	NR_netcon_repeater	Late

Proposal 1: 	NCR-MT can optionally support RRC_INACTIVE state with the assumption that enhancement specifically for NCR-MT is not needed.

Proposal 2: 	NCR-MT can indicate a maximum number of supported DRBs as part of UE capability.

Proposal 3: 	NCR-MT can indicate whether it supports SRB2 configuration without DRB.

Proposal 4: NCR-MT should mandatorily support SSB based RLM.

Proposal 5: For FR2, NCR-MT should mandatorily support SSB based BFD and BFR.

Proposal 6: NCR-MT can optionally support cell (re)selection and handover.

Proposal 7: NCR-MT can optionally support RRM measurements in RRC_IDLE/INACTIVE and in RRC_CONNECTED.

Proposal 8: 	The capabilities of NCR-MT and the capabilities of NCR-Fwd are reported separately to the gNB.

Proposal 9: 	RAN2 should focus on mandatory features that are supported by NCR-MT, all optional features are by default applicable to NCR-MT unless explicitly excluded.

Chair: do we discuss all the features one by one or take a “generic” approach?

Proposal 10: RAN2 to discuss NCR-MT features based on above tables and capture the conclusion in TS 38.306.

R2-2211198	Further discussion on the necessary aspects to support NCR	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

Proposal 1: RAN2 to inform RAN1 that RRC signaling is recommended if the side control information is semi-static or requires higher reliability.

Proposal 2: RRC_INACTIVE is not supported for NCR-MT.

Proposal 3: The NCR-Fwd should be in OFF state when the NCR-MT is in RRC_IDLE.

Proposal 4: NCR-MT ignores cellReservedForOtherUse for cell barring determination, but the NPN capable NCR-MT should consider cellReservedForOtherUse for determination of an NPN-only cell.

Proposal 5: Introduce an NCR-support indication in SIB1.

Proposal 6: Send an LS to ask RAN1 which of the features are applicable to NCR, and whether they are mandatory or optional.

Proposal 7: Cell re-selection and RRM measurements in RRC_IDLE are supported as mandatory.

Proposal 8: HO and RRM measurements in RRC_CONNECTED are not supported.

Proposal 9: The maximum number of DRB can be discussed during the capability phase.

R2-2212492	Discussion on signalling aspects for NCR	Ericsson	discussion	Rel-18	NR_netcon_repeater

Proposal 1	When NCR-Fwd is ON, NCR-MT can be in any RRC states.

Proposal 2	RAN2 to work on mechanisms to separately control NCR-Fwd ON/OFF states and NCR-MT RRC states.

Proposal 3	Introduce an optional 1-bit indication in SIB1 to signal NCR support. FFS on whether this should also be done for NPNs.

Proposal 4	The NCR-MT does not support the RRC_INACTIVE state.

Proposal 5	The NCR-MT shall support at most 1 DRB.

DISCUSSION

NCR support in SIB:

(R2-2211198) Proposal 5: Introduce an NCR-support indication in SIB1.

(R2-2212492) Proposal 3: Introduce an optional 1-bit indication in SIB1 to signal NCR support. FFS on whether this should also be done for NPNs

ZTE, Mediatek, Fujitsu: we support this

Fujitsu: but not NPN

LG: support, and it should be per-PLMN as in IAB to support RAN sharing; in IAB it is also per NPN

Intel: this bit is only indicating that MT can connect to gNB, it does not say anything about FWD. The relationship between MT and FRD is not clear.

Chair: I suggest to discuss these details offline

ZTE: it can be per PLMN

QCOM: we agree, should be the same as in IAB. This should indicate that the parent support NCR operation.

Huawei: we agree it should as IAB

- Introduce an NCR-support indication in SIB1 per PLMN; whether it is also per NPN is FFS

RRC_INACTIVE:

(R2-2211908) Proposal 1: 	NCR-MT can optionally support RRC_INACTIVE state with the assumption that enhancement specifically for NCR-MT is not needed.

(R2-2211198) Proposal 2: RRC_INACTIVE is not supported for NCR-MT.

(R2-2212492) Proposal 4	The NCR-MT does not support the RRC_INACTIVE state.

MediaTek: agree with E///, no need to support INACTIVE

QCOM: it is supposed to be low complexity, but low complexity may mean you re-use “what you have”. It is better to keep it optional.

Apple: disagree with QCOM about complexity. Agree with Mediatek and E///

Samsung: disagree with Apple, support keeping it optional

LG: agree with QCOM; supporting inactive is not a problem but we should also discuss how it relates to ON/OFF

E///: supporting RRC_INACTIVE requires RAN4 involvement

AT&T: we support QCOM, mirroring IAB is a good model

- WA: RRC_INACTIVE is optionally supported without any specific enhancements

Apple: does optional support mean we introduce new capability

ZTE: capability is already there

Apple: that would be a different capability

NEC: support the current proposal

Samsung: it is true it is not legacy capability, but we have done something similar in IAB

Intel: we also support the current proposal

On/Off:

(R2-2211198) Proposal 3: The NCR-Fwd should be in OFF state when the NCR-MT is in RRC_IDLE

(R2-2212492) Proposal 1	When NCR-Fwd is ON, NCR-MT can be in any RRC states.

(R2-2212492) Proposal 2	RAN2 to work on mechanisms to separately control NCR-Fwd ON/OFF states and NCR-MT RRC states.

E///, MediaTek, Samsung, Apple, ZTE, Intel, QCOM: agree with E///, that ON/OFF and RRC states should be decoupled

Nokia: we have symphony with this, but wonder if it RAN1 decisions would impact this

Huawei: what is meant by “decoupled”? In Idle there would be no network control, so we need to discuss those details

QCOM: even if it is in idle it can operate, but of course the network would not have immediate control

E///: “independent” instead of “decoupled”; open to discuss details

LG: better wording is “When NCR-Fwd is ON, NCR-MT can be in any RRC states”

Kyocera: agree with HW, NCR-MT is normally in connected

ZTE: this has been discussed in RAN1, and RAN1 agreed FWD can work in any state

- Continue offline

Max number of DRBs:

(R2-2211908) Proposal 2: NCR-MT can indicate a maximum number of supported DRBs as part of UE capability.

(R2-2211198) Proposal 9: The maximum number of DRB can be discussed during the capability phase.

(R2-2212492) Proposal 5: The NCR-MT shall support at most 1 DRB.

QCOM, LG, Samsung: no reason to have any restrictions, should be up to implementation

MediaTek, Apple: at most 1, only for OAM

ZTE: our proposal was capability, but we are OK with 1 DRB

HW: there can be multiple traffic flows from OAM, should discuss further

CMCC: at least 1 DRB should be supported, define capability

Nokia: in the future we may need more than 1, same view as LG

QCOM: restricting to 1 means more effrot

- Continue offline

Cell (re)selection and handover:

(R2-2211908) Proposal 6: NCR-MT can optionally support cell (re)selection and handover.

(R2-2211908) Proposal 7: NCR-MT can optionally support RRM measurements in RRC_IDLE/INACTIVE and in RRC_CONNECTED.

(R2-2211198) Proposal 7: Cell re-selection and RRM measurements in RRC_IDLE are supported as mandatory.

(R2-2211198) Proposal 8: HO and RRM measurements in RRC_CONNECTED are not supported.

LG: Removing cell reselection will have big spec impact; safer to support reselection

Intel: we haven’t discuss the relationship between MT and FWD, what reselection of MT means for FWD?

Intel: can NCR-MT and NCR-FWD cells be different?

Vivo: cell reselection should be supported, but not HO

AT&T, QCOM: NCR may be stationary but they mobility should be supported

ZTE: response to Intel: NCR-MT is connected to a specific cell but FWD will amplify all the cells on the frequency; in this discussion cell reselection is for NCR-MT

E///, QCOM: cell (re)selection and HO go “hand in hand”

Nokia: support cell (re)selection but not necessarily HO

- Continue offline

[AT120][701][Follow up on discussions]  (ZTE)

Scope:

- Further details of the agreed proposals

- All the proposals which have not concluded online

- Can also discuss other proposals from the papers treated online

Intended outcome: summary for the online CB session with hopefully agreeable proposals, report in R2-2213061

Deadline:  CB session

NCR-MT capabilities:

(R2-2211908) Proposal 10: RAN2 to discuss NCR-MT features based on above tables and capture the conclusion in TS 38.306.

Chair: Long email discussion between the meetings for capablities?

HW: too early

Note: the following document will be discussed online if time permits.

R2-2212309	Signalling for NCR side control information	 MediaTek Inc.	discussion	Rel-18

Proposal 1: The configuration for receiving the side control information signalling is indicated in RRC

Proposal 2: RAN2 to use RRC for semi-static beam information, MAC CE for dynamic beam information.

Proposal 3: RAN2 to use MAC CE for timing information.

Proposal 4: RAN2 to use RRC message for UL-DL TDD configuration.

Proposal 5: RAN2 to use RRC message to configure ON-OFF information (i.e., DRX-like mechanism).

DISCUSSION

Note: this paper was not discussed due to lack of time

R2-2213061 Report of [AT120][701][NCR] Follow up on discussions (ZTE)

Proposal 0	The cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to.

Note: whether the NCR-Fwd can forward other cells is up to implementation.

QCOM: support the proposal and suggest to make the note part of it.

NEC: support the intention but wonder how the “cell of FwD” is defined?

Samsung: there is no “NCR-Fwd cell” per se

Proposal 1	NCR-MT indicates the maximum number of supported DRB in UE capability, value range {1, 16}. If absence, the NCR-MT does not support DRB.

AT&T: there is a typo

Proposal 2	SRB2 is mandatory feature for NCR-MT.

Proposal 3	On NCR-Fwd ON/OFF:

- When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB.

- After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB.

- After NCR-MT enters RRC_IDLE mode upon RRC Release, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB.

  - FFS we specify a mechanism to trigger the NCR-MT back to RRC_CONNECTED mode.

Huawei: there may be a problem when gNB releases NCR-MT to idle, it may not be able to page it to bring it back to CONNECTED. Suggest FFS for 3rd bullet.

ZTE: understand HW’s concern, however we already agreed that IDLE is supported. How to bring NCr-MT back to connected is FFS and will be addressed.

Samsung: agree with ZTE

Intel: agree with Samsung, we can “fix” this by rewording “if IDLE is supported”

HW: not clear why would gNB release NCR-MT to idle

ZTE: use case confirmed by RAN1

Proposal 4	On NCR-MT RLF:

- After RLF is declared by NCR-MT, NCR-MT can perform cell selection and trigger RRC re-establishment;

- If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF;

- During RRC re-establishment procedure, NCR-Fwd is OFF.

LG: whether NCR-Fwd is OFF when there is no suitable cell should be left to implementation

Samsung: wouldn’t this cause interference? We prefer to specify it

Nokia, ZTE, QCOM: agree with Samsung

Proposal 5	NCR-MT mandatorily support cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE.

Proposal 6	In Rel-18, NCR-MT does not support handover and RRM measurements in RRC_CONNECTED.

BT: how we plan to do beam management?

E///: for beam management it is RLM, the proposal (RRM) is for mobility

Proposal 7	For reporting the capabilities of NCR-MT, the existing UECapabilityEnquiry and UECapabilityInformation messages are reused.

Proposal 8	In NCR-MT capability discussion, to focus on mandatory features that are required for NCR-MT.

Proposal 10	(Same as IAB-MT) All existing optional features are considered as applicable to NCR-MT unless explicitly excluded.

FFS on taking IAB specified features as a baseline for future discussion.

ZTE: companies are encoloured to provide contributions on mandatory features for the next meeting.

Agreements gNB cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation NCR-MT indicates the maximum number of supported DRB in UE capability, values {1, 16}. If absent, the NCR-MT does not support DRB. SRB2 is mandatory feature for NCR-MT. On NCR-Fwd ON/OFF: - When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB. - After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB. - Release to RRC-IDLE is FFS. On NCR-MT RLF: - After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment; - If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF; - During RRC re-establishment procedure, NCR-Fwd is OFF. NCR-MT mandatorily support cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE. In Rel-18, NCR-MT does not support handover and RRM measurements in RRC_CONNECTED. For reporting the capabilities of NCR-MT, the existing UECapabilityEnquiry and UECapabilityInformation messages are reused. In NCR-MT capability discussion, to focus on mandatory features that are required for NCR-MT. All existing optional features are considered as applicable to NCR-MT unless explicitly excluded (Same as IAB-MT). FFS on taking IAB specified features as a baseline for future discussion. NPN capable NCR-MT should consider cellReservedForOtherUse for determination of an NPN-only cell.

Proposal xx: NCR-MT ignores cellReservedForOtherUse for cell barring determination, but the NPN capable NCR-MT should consider cellReservedForOtherUse for determination of an NPN-only cell.

HW: NCR-MT can be in NPN cell

AT&T: what “consider” means in this context?

HW: the wording is from the existing spec for IAB-MT, it means that NCR-MT should read this parameter to understand the cell ins NPN

Intel: this implies NCR-MT can connect even if it is barred?

Note: the following documents are not expected to be discussed online due to lack of time.

R2-2211376	Discussion on NCR Functionality and UE Capability	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2211474	Configuration of Network-controlled Repeater	Qualcomm Inc.	discussion	Rel-18	NR_netcon_repeater

R2-2211521	NCR-MT RRM functions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_netcon_repeater

R2-2211695	Discussion on Signaling for side control information	Apple	discussion	DUMMY

R2-2211802	Discussion on Signaling for Side Control Information	vivo	discussion	Rel-18

R2-2211857	Discussion on state transition for NCR-MT	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2211915	Considerations on side control information	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2211976	On NCR Features supported	Samsung R&D Institute UK	discussion	Rel-18	NR_netcon_repeater

R2-2212017	Discussion on open issues for NCR-MT	Lenovo	discussion	Rel-18

R2-2212143	Signaling for side control information and RRM functions	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212498	Considerations on NCR remaining issues	NEC Corporation	discussion

R2-2212525	Further consideration of network-controlled repeaters 	Kyocera 	discussion	Rel-18

R2-2212621	Discussion on signaling for side control information	CMCC	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212731	RLM/RRM support for NR network-controlled repeaters	AT&T	discussion

R2-2212791	Discussion on signalling for side control information	China Telecom 	discussion

R2-2212920	Resolving open issues for NCR.	LG Electronics	discussion	Rel-18

### 8.1.3	Repeater management

Including Identification and authorization of network-controlled repeaters, taking into accout feedback from SA3 (S3-223080).

Note: the following documents will be discussed online.

R2-2211173	Reply LS on NCR Solutions (S3-223080; contact: ZTE)	SA3	LS in	Rel-18	FS_NR_netcon_repeater	To:RAN3	Cc:RAN2, SA2, SA5

Moved from 8.1.1

Chair: can we leave this topic to RAN3?

QCOM: agree, RAN3 are handling this issue

Huawei: agree it can be left to RAN3

- RAN2 will not treat this topic under the assumption it will be handled by RAN3

R2-2211475	Management of Network-controlled Repeater	Qualcomm Inc.	discussion	Rel-18	NR_netcon_repeater

Proposal 1: Solution 2 is deprioritized since it does not provide inter-vendor inter-operability and it does not meet SA3’s security requirements.

Proposal 2: Solutions 1, 3 and 4 rely on CN-based authorization, which is in RAN3 scope.

Proposal 3: Discussions on the RAN validation function are pending the selection of Solution 1 by RAN3.

Note: the following documents are not expected to be discussed online due to lack of time.

R2-2211199	Way forwad for NCR management	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

R2-2211377	Authorization and verification of NCR: RAN2 impact	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2211522	Down-selection of NCR management solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_netcon_repeater

R2-2211696	Discussion on NCR repeater management	Apple	discussion	DUMMY

R2-2211803	Discussion on NCR Management	vivo	discussion	Rel-18

R2-2211858	Discussion on NCR management	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2211881	Repeater management	Samsung R&D Institute UK	discussion

R2-2211909	Consideration on NCR management	ZTE Corporation, Sanechips	discussion	Rel-18	NR_netcon_repeater

R2-2211916	Clarifications about NCR management solutions based on SA3 reply	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2212018	Discussion on repeater management for NCR-MT	Lenovo	discussion	Rel-18

R2-2212144	Management of Network-Controlled Repeater	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212493	Discussion on capabilities and remaining issues for NCR	Ericsson	discussion	Rel-18	NR_netcon_repeater

R2-2212497	Down-selection of NCR management solutions	NEC Corporation	discussion

R2-2212499	Management of Network-controlled repeater	Philips International B.V.	discussion	Rel-18

R2-2212609	Discussion on NCR management	Rakuten Mobile, Inc	discussion	Rel-18

R2-2212622	Discussion on the network-controlled repeater management	CMCC	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212793	Discussion on identification and authorization of Network-Controlled Repeaters	China Telecom	discussion

R2-2212853	NCR management	MediaTek Inc.	discussion	Rel-18