3GPP TSG RAN WG2#120	R2-2211105

Toulouse, 14th - 18th November, 2022

3GPP TSG RAN WG1 Meeting #110	R1-2208194

Toulouse, France, August 22nd – 26th, 2022

Title:	LS to RAN2 on two overlapping LTE-CRS patterns in Rel-18 DSS

Response to:

Release:	Release 18

Work Item:	NR_DSS_enh

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Xianghui Han

Tel. Number:

E-mail Address:	han.xianghui@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

In the RAN1#110 meeting, RAN1 agrees to introduce the following two RRC parameters, namely lte-CRS-PatternList3 and  lte-CRS-PatternList4, for support of two PDSCH-overlapping LTE-CRS patterns for Rel-18 DSS.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

RAN1 also reaches the following agreements for support of two PDSCH-overlapping LTE-CRS patterns.

Agreement - Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS. - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies). - Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16. - Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged. Agreement - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS. - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured only if lte-CRS-PatternList3-r18 is configured in ServingCellConfig. - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’ - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied. - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e., - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.” Agreement lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applicable to 15 kHz SCS PDSCH only.

Based on above agreements, it is RAN1’s understanding that RAN2 needs to make necessary modifications for the following aspects.

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 in Rel-18 ASN.1.

- The RRC parameter crs-RateMatch-PerCoresetPoolIndex, which applies in the current TS38.331 to lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, should have its applicability extended to lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 (if configured).

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-WG1 Meetings:

TSG-WG1 Meeting #110bis-e	10th – 19th October 2022			Online

TSG-WG1 Meeting #111		14th – 18th November 2022			TBD