3GPP TSG RAN WG2#120	R2-2211116

Toulouse, 14th - 18th November, 2022

3GPP TSG RAN WG4 Meeting #104-e	R4-2214487

Electronic Meeting, 15 August – 26 August, 2022

Title:	Reply LS on RRM relaxation for Redcap

Response to:	R2-2206418

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	xusheng wei

#### E-mail Address:	xusheng.wei@vivo.com

Attachments:

1. Overall Description:

RAN4 would like to thank RAN2 for the incoming LS in R2-2206418.

RAN4 confirms that cases#8 in the previous LS [R2-2204487/R4-2207109] is supported by RAN4, in addition case 9 had already been determined to be supportable in previous LS [R4-2210598].

Regarding the following question from RAN2’s LS [R2-2206418] as copied below, RAN4 concludes the modification is reasonable and there is no concern from RAN4’s point of view.

RAN2 assume to change the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED, pending confirmation by RAN4. Please find the below TP (which has been agreed and will be implemented in TS 38.331) for your reference:

#### 5.7.4.4 Relaxed measurement criterion for a stationary UE The relaxed measurement criterion for a stationary UE is met when: - (SrxlevSS-RSRP RefStationaryConnected – SS-RSRPSrxlev) < SSearchDeltaP-StationaryConnected, Where: - Srxlev SS-RSRP = current Srxlev value of the PCell current L3 RSRP measurement of the PCell based on SSB (dB). - Srxlev SS-RSRP RefStationaryConnected = reference SS-RSRP Srxlev value of the PCell cell (dB), set as follows: - At the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or - If (Srxlev SS-RSRP–Srxlev SS-RSRP RefStationaryConnected) > 0; or - If the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected: - The UE shall set the value of Srxlev SS-RSRP RefStationaryConnected to the current Srxlev SS-RSRP value of the serving cell.

RAN2 would like to confirm with RAN4 whether it is reasonable, or whether there is any concern from RAN4.

RAN4 respectfully asks RAN2 to take the above information into account for their future work.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account for their future work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #104 bis-e	10 Oct. – 19 Oct., Electronic Meeting

TSG-RAN4 Meeting #105 	14 Nov. – 18. Nov. 2022, Meeting