3GPP TSG-RAN WG2 Meeting #120	R2-2211717

Toulouse, France, 14 November – 18 November 2022

Agenda item:	8.5.4.2

Source:	Apple

Title:	Configured Scheduling and UE-Assistance Information for XR

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

During RAN2 #119bis-e, we have reached the following agreements for XR-specific scheduling enhancements [1]:

- RAN2 #119bis-e Agreements 1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting. 2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

In this paper, we present some of our views on both configured scheduling and UE assistance information for XR.

# Discussions

## Enhancements of Configured Scheduling with Multi-PxSCH

Multi-PxSCH scheduling basically means that a single UL grant or a single DL resource assignment can be used to allocate a group of multiple independent PUSCH and PDSCH respectively.   The intention of Multi-PxSCH is mainly to deal with XR traffics with variable and potentially large sizes. In particular, when a large trunk of packet arrives, it is desirable to schedule multiple resources at once in order to timely accommodate the large amount of data. From our point of view, Multi-PxSCH is particular useful for the PDU Set concepts of XR where all PDUs in the PDU set have the same requirement, as shown in Figure 1:

Figure 1 An illustration of PxSCH cluster allocation for transmission of a PDU Set.

Note that Figure 1 is just an illustration, the mapping relationship between a PDU and a PxSCH within the cluster is not necessarily one-to-one.

One main direction that is being investigated is CG/SPS with multiple TB in each periodic cycle. That is, in every CG/SPS occasions, multiple PUSCH/PDSCH are provisioned so data packets with larger sizes can be handled. Since the packet size varies from time to time for XR use cases, and the packet may arrive quite late due to jitter, it is questionable whether this is an efficient approach as very likely only a subset of PxSCH in many of the CG/SPS occasions will be used for transmission, which leads to resource wastage and therefore contradicts to the WI objective of power saving and capacity enhancements.

On the other hand, it has been argued that the existing framework based on multiple CG or SPS configurations can already resolve the problem, as the resource from another CG/SPS configuration can be used to “complete” the transmission of buffered data, if the resource from one CG/SPS configuration is insufficient. However, although such approach does not require further specification change, it may also be quite wasteful as the resources of the other CG/SPS configurations may not be used most of the time, but gNB/UE may still need to decode them blindly.

Observation 1: Both multi-PxSCH per CG/SPS cycle and multi-CG/SPS configurations may be quite inefficient due to resource over-provisioning.

To avoid resource wastage while making sure there is enough resource for a large-size packet to be transmitted in time, we think the gNB can simply instruct a UE to activate a number consecutive CG/SPS occasions, in order to accommodate the packet in the buffer. Note that a set “consecutive CG/SPS occasions” is virtually the equivalent to a multi-PxSCH allocation (especially if the CG/SPS periodicity is configured to be sufficiently short, which can be managed by gNB implementation). Both gNB and UE can assume the CG/SPS resources outside this “activation range” are not used, and can refrain from decoding them to save resource/power. This is akin to Type-2 CG, but the key difference is that, the UE deactivates the CG/SPS autonomously (without further instruction from the gNB) after an indicated number of CG/SPS occasions have been activated. As the gNB knows how much data it intends to transmit on DL, and how much data the UE intends to transmit on UL (based on e.g. BSR), it can indicate the number of required consecutive CG/SPS accordingly. This is shown in Figure 2:

Figure 2 Multi-PxSCH allocation based on dynamic indication of number of consecutive CG/SPS occasions to be activated.

It is also worth noting that, since each CG/SPS occasion is seen as an independent transmission opportunity, each of them is associated to a particular HARQ process. With the mechanism described above, we believe the specification impact is much less as this is based on the existing CG/SPS framework where the HARQ PID determination for each CG/SPS occasion is well defined, and therefore we do not need to refine HARQ process determination for each TB allocation in a PxSCH cluster, as in the approach based on multi-PxSCH per CG/SPS cycle.

Proposal 1: The gNB can instruct a UE to temporarily activate a specific number of consecutive CG/SPS occasions in order to completely transmit/receive a large-size trunk of data (e.g. a PDU set) on time.

## UE-Assistance Information for XR Scheduling

An XR application server may assist the RAN with information about XR-specific characteristics of a traffic flow where the provision of such information happens through the CN. Based on the latest SA2 progress, TSCAI framework introduced for IIoT can be reused for the CN for provide XR traffic characteristics to RAN. Nevertheless, the accuracy of this information is gated by factors such as the refresh interval that can be applied for each UE and traffic flow, the available signalling capacity, and the delay within the CN. Furthermore, certain characteristics, such as arrival of critical information, jitter, buffer residency time, may suddenly change at either side of the connection. In some cases, the signalling of feedback/adjustment information via the application layer from the UE to the server and then back from the server to the RAN may simply take too long.

Moreover, the assistance information from the CN to the RAN currently exists for 5G advanced interactive services and URLLC where TSCAI is optional. When TSCAI is unavailable, the network may in some cases rely on pre-configured QoS flows using QoS mapping tables, which is less dynamic. Accurate and up-to-date information on expected burst timings helps reduce collisions when the network can utilize this information to allocate radio resources accordingly. It seems reasonable to assume that a gNB may not aways receive sufficient information from within the network. Compared to URLLC/IIoT with TSN where the gate schedule is relatively constant, packet arrival patterns are more dynamic in XR. In particular, TSN can control packet arrival cycles from a single network element such as the CNC, but for XR, a pattern update can originate from multiple entities in the system. This means the provision of XR assistance information becomes more complex.

Thus, it is questionable whether TSCAI from CN can timely capture XR traffic characteristic variation caused by UE-side activities, and whether is always available. From our perspective, TSCAI from CN is not always adequate for XR as traffic characteristics can change by activities at either side of the connection.

Observation 2: TSCAI from CN may not be always be adequate for XR scheduling, especially when UE-side application activities are considered.

Based on the observation above, it makes sense for RAN2 to address the cases where TSCAI is insufficient to provide up-to-date XR traffic characteristics to the RAN, or when inherent characteristics require a fast adjustment of the current allocation. As agreed in RAN2 #119bis-e, we can further discuss the enhancements that allow the UE to provide assistance information to the gNB, which may be applied as a complementary source (or even a sole source) of information (in addition to TSCAI) that facilitates RAN in terms of scheduling. A UE may also indicate a preference for a parameter update when required, for example, in use-cases heavily relying on CG/SPS. This helps ensure radio resources are allocated appropriately. On a high level, the network may configure the UE to enable/disable scheduling assistance information, e.g., at QoS flow establishment or through an update at a later time.

In light of this, RAN2 can further explore the procedure of enabling UE-Assistance Information for XR scheduling. Specifically, we think a procedure based on the following principal can be considered:

- The network can first configure a UE to provide scheduling assistance information. For example, the gNB may configure/request a UE to send UAI relating to XR traffics over RRC.

- Based on the configuration, the UE AS can interact with the application layer and indicate the need of information relating to XR traffic characteristics.

- The application layer provides information to the UE AS based on the request. The information may include traffic periodicity and expected packet size etc.

- The UE AS sends the information (provided by the application layer) to gNB, based on UAI with extended parameter sets.

- Based on the received UAI, the gNB conducts scheduling accordingly (e.g. configure or re-configure CG for UL transmission)

To have a foundation for further discussions about UAI for XR in the WI phase, the high-level principle described above can be captured in the TR.

Proposal 2: A high-level principle/procedure of UE-Assistance Information relating to XR traffic characteristics can be captured in the TR for further discussions in the WI phase.

# Conclusions

This contribution provides our views on configured scheduling enhancement for XR and UE-Assistance information relating to XR traffic characteristics. We have made the following observations:

Observation 1: Both multi-PxSCH per CG/SPS cycle and multi-CG/SPS configurations may be quite inefficient due to resource over-provisioning.

Observation 2: TSCAI from CN may not be always be adequate for XR scheduling, especially when UE-side application activities are considered.

Based on these observations, we propose:

Proposal 1: The gNB can instruct a UE to temporarily activate a specific number of consecutive CG/SPS occasions in order to completely transmit/receive a large-size trunk of data (e.g. a PDU set) on time.

Proposal 2: A high-level principle/procedure of UE-Assistance Information relating to XR traffic characteristics can be captured in the TR for further discussions in the WI phase.

# References

- R2-2210802, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE, Vice Chairman (Nokia), RAN2#119bis-e, Online, October 2022.