3GPP TSG-RAN WG2 #120	R2-2211954

Toulouse, France, November 2022

Agenda Item:	8.3.3

Source:	OPPO

Title:	Discussion on SSB/SIB-less

Document for:	Discussion, Decision

Introduction

This paper will provide our considerations on SSB/SIB-less, for which RAN2#119bis-e meeting reached the following agreements.

- For SSB/SIB-less solution, RAN2 starts with multi-carrier case

- RAN2 assumes that the SSB-less solution for inter-band CA in connected mode we can consider to use the intra-band CA mechanism as a baseline/starting point. FFS whether there are other impacts for RAN2 according to other WGs discussion

- For SIB-less/SSB-less, capture the solutions in more details over the email discussion and clarify the definition on anchor cell.  (e.g. 1) non-anchor NES cell doesn’t transmit SSB and SI 2) non-anchor cell doesn’t transmit SIB) FFS for paging in both mechanisms.

Discussion

In the RAN2#119bis-e meeting, further study is recommended on the SSB/SIB-less method which is assumed to be studied starting from the multi-carrier scenario. Based on the agreement achieved online, the rapporteur has summarized two directions below for the SIB-less/SSB-less method:

- Case1: The anchor cell transmits SIs for NES cells, and NES cells transmit neither SSBs nor SIs;

- Case2: The anchor cell transmits SIs for NES cells, and NES cells transmit SSBs but not SIs.

We would like to analyse the implementation of the SSB/SIB-less method case by case.

- Case 1: The anchor cell transmits SIs for NES cells, and NES cells transmit neither SSBs nor SIs

In this case, neither SSB transmission nor SIB transmission is allowed on the NES cell. As a result, due to the lack of SSB information, the UE can not detect such a cell and access this cell directly (or without other cells’ help). We understand that Case 1 is a typical SSB-less case. And for the SSB-less case, RAN2 already agrees to take inter-band CA in connected mode as a baseline. To simplify the spec and reduce the UE complexity, we prefer the RAN2 to prioritize the CA solution for Case 1 and to check whether/how to enhance the current CA mechanism.

In the case that the NES cell transmits neither SSB nor SI message, RAN2 considers the NES cell as a serving cell other than SpCell and checks whether/how to enhance the current CA mechanism.

- Case2: The anchor cell transmits SIs for NES cells, and NES cells transmit SSBs but not SIs.

In this case, the UE can obtain SSB from the NES cell but can not obtain the SI message from the NES cells. Although it can have a direct energy-saving gain due to no SI message transmission, the potential side impact is how to ensure that the system information of the NES cell can be provided to the idle UE who intends to camp on/connect to the network. One resolution is to provide the system information of the NES cell via the SI message transmitted by the anchor cell. One follow-up issue is how the UE knows the association between the NES cell and the anchor cell, which in our understanding can be studied in the WI stage.

Figure 1 gives an example of the anchor cell carrying the NES cell’s system information. In detail, if the gNB decides to enable the network energy saving for the NES cell (i.e. Cell1), the NES cell may transmit its SSB but omit its SIB1/OSI. Typically, the only transmitted SI in Cell 2 for Cell 1 is SIB1. Accordingly, the UE that tends to access the NES cell can obtain the system information of the NES cell from the anchor cell’s SIB (i.e. Cell2’s SI).

Figure 1 Illustration of obtaining the SI of the NES cell

To simplify the UE behaviour, we prefer the UE camps the anchor cell, i.e. Cell 2, otherwise, we need to consider more SI information delivery, how to do cell reselection, etc.

Since the RACH configuration is contained in SIB1, once the system information of the NES cell can be obtained from the anchor cell, the UE can use such a RACH configuration to access the NES cell and the current RACH procedure can be considered as a baseline.

In the case that the NES cell transmits SSB but not the system information, the SIB1 of the NES cell can be delivered via the SI of the anchor cell. FFS on how the UE knows the association between the NES cell and the anchor cell.

In the case that the NES cell transmits SSB but not the system information, the UE camps on the anchor cell.

In the case that the NES cell transmits SSB but not the system information, the UE can access the NES cell by using the NES cell-related RACH configuration which is indicated by the anchor cell.

Conclusion

We have the following proposals:

Proposal 1	In the case that the NES cell transmits neither SSB nor SI message, RAN2 considers the NES cell as a serving cell other than SpCell and checks whether/how to enhance the current CA mechanism.

Proposal 2	In the case that the NES cell transmits SSB but not the system information, the SIB1 of the NES cell can be delivered via the SI of the anchor cell. FFS on how the UE knows the association between the NES cell and the anchor cell.

Proposal 3	In the case that the NES cell transmits SSB but not the system information, the UE camps on the anchor cell.

Proposal 4	In the case that the NES cell transmits SSB but not the system information, the UE can access the NES cell by using the NES cell-related RACH configuration which is indicated by the anchor cell.

Reference

[1] RP-213554  New SI: Study on network energy savings for NR

[2] R2-2211428, Report of [POST119bis][304][NES] TP on cell selection／reselection and SSB／SIB-less (Huawei)

TP

### 6.1.2	SCell without SSB in inter-band CA

- 6.1.2.x	Higher layer procedures

The SCell without SSB in intra-band CA is considered as baseline, i.e., for a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the SpCell or the SCell, of the cell group.

Editor's note: impacts in RAN2 may need further analysis pending on other WGs progress.

#### 6.1.2.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

### 6.1.3	NES Cell without SIB

- 6.1.3.x	Higher layer procedures

NES cell without SIB is only applicable in multi-carrier scenario, where there is an anchor cell and one or multiple NES cell(s) without SIB in different carrier(s).

Anchor cell is a cell where NES-capable UE assumes SSB, system information and paging are transmitted.

Editor's note: FFS: The system information transmitted by anchor cell also includes the necessary information for NES-capable UEs to access via an NES cell without SIB.

An NES cell without SIB omits the transmission of SIB, or omits the transmission of both SSB and SIB.

In the case that the NES cell transmits neither SSB nor SI message, the NES cell is considered as a serving cell other than SpCell.

In the case that the NES cell transmits SSB but not the system information, the SIB1 of the NES cell can be delivered via the SIB of the anchor cell. The UE camps on the anchor cell and can access the NES cell by using the NES cell-related RACH configuration which is indicated by the anchor cell.

Editor's note: FFS the details on how to support NES cell without SIB or without SSB and SIB.

Editor's note: FFS whether paging enhancements, e.g. no paging transmission, is applied to the scenarios where SSB and/or SIB is not transmitted on the NES cells.

Editor's note: FFS on how the UE knows the association between the NES cell and the anchor cell.

#### 6.1.3.z	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.