3GPP TSG RAN WG2 Meeting #116-b electronic	R2- 2212072

Toulouse, France  Nov 13 –Nov 18, 2022

Agenda Item:	8.2.4 - LPHAP

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	SRS Configuration for supporting LPHAP

Document for:	Discussion & Decision

# Introduction

The SID [1] for study on expanded and improved NR positioning enhancements in Rel. 18, lists, among others, the following as objective:

- Improved accuracy, integrity, and power efficiency - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state.

In RAN2#119-bis-e [2], we reached the following agreements regarding SRS configuration for LPHAP scenarios.

Agreements

Proposal 3 (modified): RAN2 agree to study enhancements on SRS configuration (12/15). Further study the following candidate enhancements on SRS configuration, including the possible interference and changes of spatial relations problems.

a) Validity area mechanism; (12/13)

b) SRS update mechanism; (10/13)

c) Pre-configure multiple SRS, which could include broadcast transmission of configurations with

UE-specific determination along with the network of a configuration; (9/13)

FFS if item c would require network nodes to measure multiple SRS configurations for the same UE simultaneously.

LS to RAN1 to ask them about interference issues with SRS configurations across multiple cells and about the validity of SRS parameters.

In this contribution, we provide our views on way forward on uplink reference signal configuration in LPHAP scenario.

# Configuration for positioning SRS in LPHAP scenario

## Issue 1: Area of validity of SRS configuration

In Rel. 16, the positioning SRS configuration was provided to the UE by the serving cell and could be assumed to be orthogonalised among the UEs served by the cell. Furthermore, SRS transmission would be discontinued when transiting to the RRC_INACTIVE or upon handover.

In Rel. 17, RRC_INACTIVE is supported at least for deferred MT-LR. The UE could continue transmitting SRS as long as the increase/decrease of RSRP for time alignment validation was within the threshold. Furthermore, the UE stops transmitting SRS when cell reselection occurs. As a result, the UE needs to initiate random access procedures to reacquire SRS configuration for the new cell. The additional procedures needed to acquire new SRS configuration each time a cell is reselected, can be avoided or reduced if a UE can be provided a configuration that is valid in a group of cells.

Figure 1: Depiction of areas where the SRS configuration provided is valid.

In Figure 1, we depict three areas where in each area four cells are configured. For example, cells {1, 2, 3, 6} are configured to be in Area-1. A UE that has acquired SRS in Cell 1 can continue using the same SRS configuration when the UE is camped within Cell 2, Cell 3 and Cell 6. This means, the UE can spare signalling needed to acquire new SRS and thus reduce power consumption in comparison to Rel. 17 mechanism. However, the configuration of SRS needs to be reserved in multiple cells within an area.

Observation 1: Configuring SRS with a validity area of more than one cell reduces signalling needed to acquire SRS on handover.

Proposal 1: UE shall be configured with SRS configuration(s) which is valid in a certain area, such as list of cells.

For the downlink in Rel. 17, preconfigured assistance data consisting of multiple instances of assistance data where each instance is applicable to a certain area in the network was specified. The same concept can be extended to the SRS where a UE can be configured with multiple instances of SRS configurations, from which one SRS configuration can be activated. The instance of SRS configuration that is activated may be determined either by

- Validity area, which may for example be signaled by list of cells.

- Measurement, where a configuration may be selected based on outcome of measurement of a downlink reference signal.

## Issue 2: Contention vs. Capacity due to UE multiplex

In Rel. 17, the SRS configuration is released when the UE moves from RRC_CONNECTED to RRC_IDLE. One possibility is to provide a UE with SRS for positioning while the UE is in RRC_IDLE mode or RRC_INACTIVE mode. Alternatively, a UE can be allowed to use the configuration provided during the RRC_CONNECTED mode to be continued to be used during RRC_INACTIVE and RRC_IDLE for LPHAP scenarios.

If we assume that a UE may transmit with the same configuration across multiple cells, then the resource configuration needs to be reserved in each of the cells. This leads to a large cost in capacity which increases with the number of size of validity area.

Alternatively, multiple UEs are configured with same (common) configuration, then we may have contention when two UEs try to transmit SRS at the same time. This is similar to PRACH, where two UEs may simultaneously initiate PRACH transmissions and collide. The timing measurement can still be determined if the UEs use different sequences or different cyclic shifts.

Observation 2: Contention scenarios similar to PRACH can occur when transmission of SRS in RRC_IDLE is enabled.

Noting that for positioning measurements signal to noise ratio (SNR) requirements tend to be much lower than that for data communications, we can assume that the UEs can share same REs but be differentiated in parameters such as sequences. Therefore, the common configuration for LPHAP UEs can be broadcasted, whereas the UE-specific part can be signaled close to the transmission time or determined independently by the UE based on identifiers. This reduces the amount of signaling required, as the SIBs can be acquired by the UE around the time the UE acquires the system information for camping in a cell.

Observation 3: Two UEs may have transmission on same REs within the positioning area, and be separated along code-domain (e.g. sequences or cyclic shifts).

An important consideration for LPHAP devices is a longer battery life, which inadvertently leads to a longer period between two consecutive transmissions. Furthermore, the transmission could be triggered by deferred location request events (defined in TS23.273), such as the UE entering into a target area or motion and so on. This could lead to a situation that the UE may need to transmit in one of the configured resources in contention.  However, to reduce the number of collisions and retransmissions, a UE could be configured to monitor certain downlink resources (occasional DL-PRS), whereby the presence or absence of occasional DL-PRS could indicate whether some other UE is currently using the resource of not.

Figure 2: Mapping the downlink PRS (a reservation resource) which a UE needs to monitor before transmitting an uplink reference signal. The occasional DL-PRS (A) tells the UE whether the UL-SRS configuration (B) is in use by other UE or not, thereby avoiding collisions.

In Rel. 17, monitoring a DL-RS for RSRP change has been specified as a criteria to continue or discontinue SRS transmission. The same mechanism can be extended further so that the network transmits a DL-PRS on certain configured resource to indicate to a UE whether the resource is being used by another UE in the network or not. The transmission of a DL-PRS serves as a kind of ‘reservation indicator’, where the UE can determine whether to transmit on a particular instance of a configured SRS resource or to use a different instance of a configured SRS resource.

Proposal 2: A SRS configuration shall be mapped to a downlink reference signal, the measurement on the DL reference signal indicates to the UE whether some other UE in the network is currently using the uplink SRS configuration or not.

## Issue 3: Stopping SRS transmission in RRC_INACTIVE and RRC_IDLE

With the Rel. 17 SRS configuration, the UE has among others parameters such as resources indicating pathloss reference, resource indicating the spatial relation which are not same across the whole positioning area.  By dividing the parameters into common and complementary (UE-specific) parts, trigger could be provided to the UE by providing or removing the complementary parts.

Observation 4: The SRS configuration for resources where positioning signals can be transmitted in RRC_IDLE can be split into common and UE-specific for supporting RRC_INACTIVE positioning.

Figure 3: Splitting the SRS configuration into common and complementary parameter (i.e. a delta configuration)

Especially for MT-LR, it becomes important that the UE in RRC_IDLE has a means to trigger positioning resources. If the configuration is split into common and UE-specific parts, then the reception of UE-specific part could be the implicit request to start transmitting the periodic SRS.

Proposal 3: The SRS configuration shall be divided into common and UE-specific for RRC_INACTIVE. The common configuration shall be coordinated among multiple cells.

Proposal 4: The UE-specific part / complementary parameter (sequences or cyclic shift) may be derived by the UE based on UE identifiers or identifier of the last visited cell or signaled by the cell (if SDT mechanism is active).

Figure 2: The parameters to add/release parameter can be sent over the SDT mechanism.

UE mobility causes effects like change in timing advance, which can cause interference due to deteriorating orthogonality at the TRPs. The UE can monitor mobility by performing measurement on a DL-PRS. The SRS configuration may be triggered or stopped based on RSRP measurement of an associated DL-RS. Likewise, a TA value to be used may be derived based on outcome of measurement on one or more DL-PRS resources.

Proposal 5: The transmission of positioning SRS in RRC_INACTIVE mode shall be stopped if the UE moves away from validity area (consisting a list of cells) or exceeds the validity timer.

The SRS configuration shall be associated with a DL-RS measurement (e.g. DL-PRS).

Proposal 6: The positioning SRS in RRC_INACTIVE mode shall be triggered to stop if the RSRP on an associated DL-RS (SSB or PRS), – reference resource, falls below a certain threshold or rises above a certain threshold configured by the network.

# Conclusions

In this contribution, we have made the following observations:

Observation 1:  The SRS configuration needs to be coordinated among cells within the area where the UE can be expected to transmit SRS while it in RRC_INACTIVE state.

Observation 2: Contention scenarios similar to PRACH can occur when transmission of SRS in RRC_IDLE is enabled.

Observation 3: Two UEs may have transmission on same REs within the positioning area, and be separated along code-domain (e.g. sequences or cyclic shifts).

Observation 4: The SRS configuration for resources where positioning signals can be transmitted in RRC_IDLE can be split into common and UE-specific for supporting RRC_INACTIVE positioning.

Based on the above observations, we make the following proposals:

Proposal 1: UE shall be configured with SRS which is valid in a certain area, such as list of cells.

Proposal 2: A SRS configuration shall be mapped to a downlink reference signal, the measurement on the DL reference signal indicates to the UE whether some other UE in the network is currently using the uplink SRS configuration or not.

Proposal 3: The SRS configuration shall be divided into common and UE-specific for RRC_INACTIVE. The common configuration shall be coordinated among multiple cells.

Proposal 4: The UE-specific part / complementary parameter (sequences or cyclic shift) may be derived by the UE based on UE identifiers or identifier of the last visited cell or signaled by the cell (if SDT mechanism is active).

Proposal 5: The transmission of positioning SRS in RRC_INACTIVE mode shall be stopped if the UE moves away from validity area (consisting a list of cells) or exceeds the validity timer.

Proposal 6: The positioning SRS in RRC_INACTIVE mode shall be triggered to stop if the RSRP on an associated DL-RS (SSB or PRS), – reference resource, falls below a certain threshold or rises above a certain threshold configured by the network.

# References

| [1] | RP-213588, "Revised SID on Study on expanded and improved NR positioning," 3GPP, Online, 2021-03. |
| --- | --- |
| [2] | 3GPP RAN WG2#119bis-e, "Chairman's Notes on Positioning and Relay Session," 3GPP, Online, 2022-10. |