3GPP TSG-RAN WG2 Meeting #120	R2-2212100

Toulouse, France, 14 – 18 November 2022

Agenda item:	7.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on epoch-Time reference for Handover scenarios

WID/SID:	LTE_NBIOT_eMTC_NTN - Release 17

Document for:	Discussion and Decision

# 1	Introduction

At the RAN2 #119bis some agreements related to the definition of epoch time for satellite assistance information were made. In this contribution we discuss the implications of the agreement related to HO/CHO and propose amendments.

# 2	Discussion

At RAN2 #119bis the following was agreed:

In case of HO/CHO, the SFN indicated by epochTime is the frame nearest to the frame where RRC reconfiguration message is received

Based on the discussion at the RAN2 #119bis it seems the implicit understanding is that the frames, referred to in the above agreement, are both link to the serving cell. However, this may be a problematic understanding. The reason is that the epochTime is linked to satellite assistance information for the target cell. The serving cell would not be able to change the epochTime of the information and if the serving cell is not able to transmit the data at the SFN, which is indicated by the epochTime from the target cell it may be an issue. This could for example happen if the cells are not synchronized in terms of SFN or if there is a delay in providing the information from the target cell to the serving cell.

Proposal 1: RAN2 to discuss the cell reference for the frames defined in the RAN2 #119bis agreement for additional impacts to HO/CHO scenarios.

The agreement defines the SFN of the epoch time is the frame nearest to the frame where the RRC message is received. In principle the use of nearest could point to an epoch time in the past i.e. after the RRC message is received. For example, if the RRC message is received in SFN 100 and the epoch time is SFN 50 the nearest epoch time will be the one in the past compared to the one at 50+1024. Therefore, RAN2 should discuss whether the UE can reduce the validity timer, i.e. validity timer – 50 in the previous example, or await the next epoch time.

If the UE cannot use the epoch time of the past, i.e. the UE has to await a future epoch time, the handover may be delayed. This may impact the allocation of Random Access resources and RAN4 requirements regarding handover interruption.

Observation 1: If the UE has to await a future epoch time it will impact Random Access resource allocation and handover interruption requirements.

Proposal 2: RAN2 to discuss how to handle the nearest epoch time, defined in the RRC reconfiguration message for HO/CHO, which occurs before the UE receives the message.

In case of CHO when the execution condition is met the UE first applies the target configuration and starts downlink synchronisation to the target cell as the first step. In that case if the SIB31 is part of the conditional configuration when the UE attempts to apply the target cell’s SIB31 epoch time value the UE has already switched its downlink to the target cell, so having the SFN reference to the serving cell may require the UE to first read system information of serving cell before applying the target cell configuration. This may require some clarification for UE behaviour.

Observation 2: In case of CHO execution whether UE need to wait for reading serving cell SFN before starting execution as per the above agreement is not clear.

If SIB31 information is included and if CHO recovery is enabled, the UE may need to switch to a new cell after experiencing a failure for the selected target cell and apply the RRC Reconfiguration. In this case the UE has already switched out from the serving cell. So using the serving cell as reference for the epoch time is not possible here.

Observation 3: Serving cell SFN reference for epoch time will not work for CHO recovery scenario.

Proposal 3: RAN2 to revisit the agreements on epochTimer reference based on its applicability for CHO and CHO recovery scenarios

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 to discuss the cell reference for the frames defined in the RAN2 #119bis agreement for additional impacts to HO/CHO scenarios.

Observation 1: If the UE has to await a future epoch time it will impact Random Access resource allocation and handover interruption requirements.

Proposal 2: RAN2 to discuss how to handle the nearest epoch time, defined in the RRC reconfiguration message for HO/CHO, which occurs before the UE receives the message.

Observation 2: In case of CHO execution whether UE need to wait for reading serving cell SFN before starting execution as per the above agreement is not clear.

Observation 3: Serving cell SFN reference for epoch time will not work for CHO recovery scenario.

Proposal 3: RAN2 to revisit the agreements on epochTimer reference based on its applicability for CHO and CHO recovery scenarios.

# 4	Reference

[1] RAN2-119bis-e -Chairman notes for IoT-NTN

[2] R2-2210848	[offline-107] RRC corrections	Huawei	discussion