3GPP TSG RAN WG2 #120-e	R2-2212289

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	8.14.4

Source:	ZTE Corporation, Sanechips

Title:	Discussion on QoE measurement for NR-DC

Document for:	Discussion and Decision

Introduction

The new WID of enhancement on NR QoE management and optimizations for diverse services was approved in RAN#96[1]. In which, the following objective is included:

... - Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2]. - Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. - Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting. - Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2]. - Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity. Note 1: The QoE measurements are not performed separately for each leg. - Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios. - Specify the QoE measurement continuity in mobility scenarios in NR-DC. - Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC. - Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

In RAN2#119bis e-meeting, based on the contributions, the following agreements have been achieved:

- Observation: Rel-18 QoE configuration may be created by MN or SN. - Either SRB1 or SRB3 can be used for providing SN configuration to UE (at least for m-based QoE). FFS if this requires additional MN-SN coordination. - In NR-DC scenario, both signalling-based and management-based QoE measurement collection shall be supported. - RAN2 assumes that there is a unique ID for QoE configurations across MN and SN. This can be accomplished by MN-SN coordination (e.g. similar as was done with measIds for NR-DC) - Use SRB4 as baseline for Rel-18 QoE. FFS how we can send QoE reports towards SN (e.g. only SRB4, define new SRB, reuse SRB3, split SRB). Discuss details in the next meeting.

In this contribution, we will firstly discuss the issues of QoE configuration and reporting in NR-DC, and then our proposals will be given.

Discussion

QoE configuration over MN/SN

Based on the previous RAN3 meeting, RAN2 needs to distinguish between legacy QoE and RVQoE for discussion in the NR-DC scenario. For legacy QoE, RAN3 has agreed that MN is responsible for configuring the S-based QoE to UE in the last meeting since only the MN in the NR-DC can receive the S-based QoE configuration from the core network. Thus the MN sends the S-based QoE configuration to the UE through an RRC message (e.g., RRCReconfiguration and RRCResume).

Observation 1: For S-based QoE, only MN can receive the QoE configuration sent from CN, thus it is natural that MN configures the QoE configuration to UE which is also aligned with RAN3’s decision.

Proposal 1: For S-based QoE, MN configures the QoE configuration to UE through an RRC message (e.g., RRCReconfiguration and RRCResume).

QoE measurement reporting over MN/SN

Based on the conclusion of the previous RAN3 meeting, the SN can receive RVQoE reports directly from the UE. So, if the QoE report is sent to SN, RAN2 needs to discuss which SRB is to be used. The MCG SRB (Signalling Radio Bearer) in DC scenario supports SRB1 and SRB2 transmission, and SCG SRB supports SRB3 transmission in the current protocol. Among them, in Rel-17, since the priority of QoE reports is relatively low, the QoE reports separately introduce SRB4 with low priority to transmit the above report. On the question of which SRB to be used for QoE reporting on SN leg, we identified the following options:

- Option 1: SRB3

- Option 2: split SRB4

- Option 3: new SRB, e.g. SRB5

For option 1, since SRB3 contains RRM measurements used for SN to evaluate the radio and make mobility decision, which has higher priority and more strict real-time processing requirement than QoE measurement. Therefore it is better not to use SRB3 to transmit QoE measurement. For option 2, UE can be configured to duplicate the transmission on MCG and SCG in case of split SRB as in legacy NR-DC. This duplication can  help improving the reliability of RRC message transmission. According to comments collected in last meeting, some companies hold the opinion that QoE reports are less important than normal RRC messages, therefore there is no need to use split SRB4 for QoE report transmission. In general we bare fine with option 2/3, and consider both can be helpful for transmitting QoE reports. For option 3, since RAN3 has already agreed that when MN is in overload situation, the network can indicate UE to transmit data on the SN to ease congestion in MN. Considering SRB3 has already been ruled out, a new SRB with a lower priority than SRB3(e.g., SRB5) can be used for report QoE to SN in this condition.For option 2 we understand there might be concerns on the implemenation complexity as well as necessity, but we are open to discuss split SRB4 based on actual use case.

Proposal 2: Introduce new SRB (e.g., SRB5) for  QoE measurement report in SN.

Proposal 3: Whether Split SRB4 is needed can be further discussed based on the actual use case.

Based on the conclusion of the previous RAN3 meeting, the UE in DC switches the reporting leg based on indication from network, and FFS in implicit or explicit way. In our understanding, when one RAN node is in overload situation, there could be available resource in other node for transmission of QoE reports. In this case, it is useful to explicitly indicate UE to switch to a node that can be used for QoE report. Therefore, we think it is necessary to introduce an explicit branch indication information, and NW includes a branch indication through dedicated signaling when providing QoE configuration.

Proposal 4: A branch indication information for QoE report is introduced through dedicated signaling.

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: For S-based QoE, only MN can receive the QoE configuration sent from CN, thus it is natural that MN configures the QoE configuration to UE which is also aligned with RAN3’s decision.

Proposal 1: For S-based QoE, MN configures the QoE configuration to UE through RRC message (e.g., RRCReconfiguration and RRCResume).

Proposal 2: Introduce new SRB (e.g., SRB5) for  QoE measurement report in SN.

Proposal 3: Whether Split SRB4 is needed can be further discussed based on the actual use case.

Proposal 4: A branch indication information for QoE report is introduced through dedicated signaling.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022

- R2-2209330, LS to RAN2 on RAN3 agreement of QoE reporting in NR-DC, August, 2022