3GPP TSG-RAN WG2 Meeting #120                                                                        R2-2212332

Toulouse, France, November, 2022

Agenda Item	:	8.5.3.1

Title	:	DRX Enhancement for XR

Source	:	Google Inc.

Work Item	:	FS_NR_XR_enh

Document for	:	Discussion and Decision

# Introduction

A new Rel-18 Study Item for XR enhancements in NR has been approved in RAN#94. Three main topics are to be studied in RAN1 and RAN2: XR-awareness, XR-specific power saving, and XR-specific capacity improvements.

As clarified in [1] ,the study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166).

The objectives of the Study Item include:

- XR-specific Power Saving (RAN1, RAN2):

  - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

    - C-DRX enhancement.

    - PDCCH monitoring enhancement.

In RAN2#119bis-e, the following agreements have been reached:

At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

In this Tdoc, we discuss DRX enhancements for XR.

# C-DRX Enhancement

## Mismatch between the C-DRX and XR traffic periodicity

One of the main limitations of the Rel-15/Rel-16 C-DRX design is that the C-DRX cycles don’t align with the AR/VR frame rates. In the current C-DRX design, cycles are integer values (Long DRX cycles: [10, 20, 32, 40, 60, 64, …] ms, short DRX cycles: [2, 3, 4, 5, 6, 7, 8, 10, 14, 16] ms). However, and as agreed in Rel-17 XR RAN1 SI and the current Rel-18 RAN1 SI for the evaluation, the packet arrival rates to be evaluated for XR are {30, 60, 90, 120} fps, so the corresponding traffic periodicities of XR are non-integer values of {33.33, 16.67, 11.11, 8.33} ms. There is obviously a mismatch between the XR traffic arrival periodicities and the C-DRX cycles both long and short cycles.

This misalignment causes a time drift between the arrival of the XR video frames and the start of the C-DRX cycles.   The accumulation of the time drift causes the expected arrival time of the XR traffic to fall outside the C-DRX On-Duration.

The misalignment, if not corrected for, leads to larger latency for data scheduling and further reduces the XR/CG system capacity.

The issue has been raised and discussed in RAN1#109e and a list of possible solutions was listed for further study:

- Use of Periodic Pattern of consecutive C-DRX cycles [2]; it consists of using multiple C-DRX cycles in one single C-DRX configuration and the different cycles are applied in a semi-static pattern. For example, for a DL video using 60 fps Frame Rate, a pattern of {17, 17, 16} ms could be used. The total length of the pattern (e.g. L = 50 ms) and the number of cycles in the pattern (e.g. c = 3) can be determined by transforming the frame rate into an irreducible fraction, e.g. 60 fps= 601000=350

This solution allows to fully align the C-DRX cycles and the XR traffic every c cycles. The remaining C-DRX cycles could be shifted forward or backward with respect to the traffic arrival and this can slightly compromise the latency or the power consumption. New C-DRX cycles should be defined to reduce the drift by using different cycles in the pattern.

Hence, this solution requires  defining new C-DRX cycles like 9 ms, 11 ms, 17 ms, 33 ms, 34 ms. Also, additional signalling of the C-DRX pattern (length and cycles) is needed.

- Periodically change the drx-startoffset [2]: it consists of adapting or changing the drx-startoffset periodically. As the drift becomes large after few cycles and the XR traffic arrival time risk to fall outside the DRX On-Duration, drx-startoffset adjustment can be done by the UE by using for example a counter, e.g. correct the offset every 3 cycles.

This solution allows to fully align the C-DRX cycles and the XR traffic periodically. The network configures the adjustement period depending on the length of the On-Duration. Short On-Durations require more frequent offset correction. Longer On-Durations require less frequent offset correction.

- Dynamically change the drx-startoffset [2]: it is similar to the previous solution but it is done dynamically via L1 signalling. The advantage compared to the previous solution is that the network has flexibility and control and can do the adjustment whenever it is needed.

- Single C-DRX configuration with multiple On-Durations [3]: it consists of a single C-DRX configuration with multiple C-DRX On-Durations. Basically, there can be more than one C-DRX On-Duration within one DRX cycle so as to match every few frames arrival

- Multiple active C-DRX configurations [3]: it consists of configuring multiple C- DRX configurations (e.g. nominator of the XR frame rate irreducible fraction) with the same periodicity (e.g. denominator of the XR frame rate irreducible fraction) but with staggered start offsets.

This solution requires more RRC signalling as multiple C-DRX configurations are needed to be configured to the UE. Also, synchronization between the different C-DRX configurations is critical and should be accurately maintained.

- Re-use the 38.321 existing formula with some modification to determine the C-DRX start time. Multiple options in re-using the existing formula:

      - Use the modulo operation on rational numbers and use “floor” operation to both sides of the formula [5].

      - Modify the formula to add an offset correction every specific number of cycles [4].

All these schemes can be further studied and evaluated to select a simple solution, with good performance, minimum implementation effort and with backward compatibility.

Dynamic adjustment is useful if we are addressing the C-DRX periodicity alignment simultaneously with the jitter handling. This requires however the network to have instantaneous or a priori knowledge about the jitter of the traffic burst arriving in the next C-DRX occasion which is not possible according to SA4 feedback [12]. Hence, we prefer to focus on the semi-static solutions for the moment without explicitly excluding/deprioritizing dynamic solutions.

Dynamic solutions can be explored further if more feedback from SA regarding the jitter is received.

- RAN2 does not assume instantaneous jitter is available or predictable unless further inputs from SA.

- Prioritize semi-static solutions for the periodicity alignment between C-DRX and XR traffic.

We prefer to keep the legacy approach by re-using the 38.321 equation with some modification. The equation should remain backward compatible for example by allowing the network to set some parameters to zero or skip the configuration of some parameters to fall-back to the legacy equation. This gives better flexibility at the network side and minimize the implementation changes at both the UE and the network sides. Also, this allows to adopt a single a solution for all sort of traffics.

- Re-use the 38.321 equation with some modification for the start of On-Duration timer .

In the MAC specification [TS 38.321], the formulas to start On-Duration timer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle : [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) .

We propose two possible solutions to update the above formulas:

- Solution 1:

In this solution, we adjust the drift every fixed number of C-DRX cycles and we adjust with a fixed offset. For example for a Frame Rate = 30 fps (33.33 ms periodicity) and drx-Cycle = 32, we will adjust every 3 cycles with an offset equal to 4.

We propose to introduce three new parameters:

- drx-CyclesAdjust: number of cycles to apply a correction shift

- drx-CyclesOffset: the value of the offset to apply every drx-CyclesAdjust cycles

- drx-CorrectionCounter: A counter to apply the offset

And apply the following algorithm:

  - Increment the counter:

drx-CorrectionCounter = 0;

if (([(SFN × 10) + subframe number] modulo [(drx-CorrectionCounter + 1) × drx-CyclesAdjust × drx-Cycle] = drx-CorrectionCounter × drx-CyclesOffset ) && [(SFN × 10) + subframe number - drx-StartOffset > 0 ]  )

drx-CorrectionCounter = drx-CorrectionCounter + 1;

  - Start of On-Duration:

[(SFN × 10) + subframe number] modulo (drx-Cycle) = (drx-StartOffset + [drx-CorrectionCounter × drx-CyclesOffset]) modulo (drx-Cycle), where SFN is the current System Frame Number, and the subframe number is the current subframe number.

The formulas are evaluated in Appendix case-1 and case-2.

Introducing a drx-Cycle = 33 can further reduce the residual error as shown in in Appendix case-3.

gNB can use the frame rate irreducible fraction to determine drx-CyclesAdjust and drx-CyclesOffset.

e.g. if Frame Rate = 30 fps and drx-Cycle = 32:

- 30 fps= 301000=3100

- drx-CyclesAdjust = 3

- drx-CyclesOffset = 100 - drx-CyclesAdjust × drx-Cycle = 4

The advantage of Solution 1 is that it allows to correct the drift by adding a fixed offset every fixed number of cycles. The solution is simple, predictable and requires only an RRC signalling to the UE of two extra parameters drx-CyclesAdjust and drx-CyclesOffset.

- Solution 2:

In solution 2, we take the C-DRX On-Duration time into consideration. Basically, if the traffic arrival time is within the On-Duration there is no need to correct the drift between the XR traffic arrival and the C-DRX cycles. The formula can be written as:

[(SFN × 10) + subframe number] modulo (drx-Cycle) = (drx-StartOffset + floor(m × d)) modulo (drx-Cycle)

The m is increased by 1 every n drx-Cycles and d = n × (xr_periodicity - drx-Cycle). The n can be given by gNB or a pre-defined value.

The formula is evaluated in Appendix case-4.

The advantage of this solution is that we correct the drift only when needed and there is no need to correct as long as the expected arrival time is still within the On-Duration.

## C-DRX Enhancement to handle the jitter

XR and Media traffic arrives at the 5G-RAN with a jitter. The main source of the jitter is the video encoder during the video encoding process which takes different durations in encoding video frames depending on the frames types, content and sizes. Hence, the traffic arrival is no longer periodic but quasi-periodic with a random jitter.

The jitter causes extra delay to the XR traffic if the data arrives before the DRX On-Duration start time or extra power consumption if the data arrives after the DRX On-Duration start time.

RAN1 in the Rel-17 SI, has modelled the XR traffic using a statistical approach. The jitter was modelled as a truncated gaussian distribution with 2 ms standard deviation and +/-4 ms range

A list of possible solutions was listed in RAN1#109e for further study:

- Dynamically advance or delay the DRX starting time based on UE experienced jitter [6]. In this proposal, the gNB does jitter estimation and then whether it signals the jitter statistics/estimation to the UE or it can signal a command to advance or delay the DRX On-Duration. Similar solution was proposed to resolve the misalignment between DRX On-Duration and XR traffic occasion by dynamically adjusting the DRX start offset. This solution has the advantage of addressing both the jitter issue and the misalignment issue with one single solution. It requires however the gNB to do the jitter estimation and correct for it.

- The use of wake-up signals (WUS) and designing low power WUS to reduce the power consumption for the jitter handling. A gap from the WUS is used to determine the DRX starting time, the WUS payload could also be used to determine the gap. The main concern with the use of WUS is that it still requires the UE to monitor (and decode if payload) the WUS, hence comes with extra power consumption and undermines the purpose of using the C-DRX.

- Two stages C-DRX configuration; one coarse C-DRX cycle which is aligned with the XR traffic periodicity and a second finer C-DRX cycle within the On-Duration of the coarse cycle [4]. The advantage is to introduce further micro-sleeps in the On-Duration of the coarse cycle, hence reducing the power consumption while still using larger On-Duration to capture the traffic jitter. The drawback is the latency penalty as traffic arriving during the micro-sleeps still needs to wait for the On-Durations of the fine C-DRX cycle. The design can be further improved by adopting non-uniform On-Durations pattern for the fine C-DRX cycle by having larger On-Durations when the jitter probability is higher and smaller On-Durations when the jitter probability is lower.

- Prioritize the two stages C-DRX configurations to handle the jitter.

- Adjustment of the C-DRX On-Duration based on the traffic arrival. The adjustment could be an extension, e.g. PDCCH monitoring in an extra time duration after the end of the On-Duration if the data is not received in the On-Duration [10]. The adjustment could also be an early termination, e.g. stopping the On-Duration at the first data arrival and then relying on the Inactivity Timer [9].

- Implicit SSSG switching conditioned on data reception: the UE can switch implicitly within the DRX On-Duration from sparse PDCCH monitoring to dense PDCCH monitoring after the data arrival [3][8].

## C-DRX Enhancement for multiple XR traffic flows

XR traffic is composed of multiple data flows, like video, audio, pose/control information, haptic, etc. These flows have different traffic characteristics (periodicities, offsets, packet sizes, etc.) and different QoS requirements.

The adoption of multiple simultaneously active C-DRX configurations to support the multi-flow XR traffic has been discussed in RAN1#109e [11]. The main motivation is to reduce the UE power consumption by having a dedicated and optimized C-DRX configuration per XR traffic flow. For example, different C-DRX configurations could be configured for video, audio, haptic flows. Even further, different C-DRX configurations could be used for I-frames vs. P-frames.

For example, the DL video and audio traffics can have different periodicities, different jitters and different packet sizes and relying on the same C-DRX configuration can compromise the QoS requirement of one of them. The same applies in UL, if traffics rely on dynamic scheduling.

- Support multiple simultaneous DRX configurations to serve multiple XR data flows with different characteristics and different QoS requirements

## Dynamic Adaptation of C-DRX Configuration

The adoption of dynamic adaptation of the C-DRX configuration parameters (On-Duration, Inactivity Timer, Start Offset) has been mentioned in RAN1#109e [11].

Although this technique can help address the jitter and the periodicity alignment issues, it requires the network to have a priori knowledge about the jitter of the traffic burst arriving in the next C-DRX occasion. This requires the network to have instantaneous information from the XR server /video codec about the jitter or the network to do jitter prediction.

The following conclusion has been made in RAN1#110:

Conclusion

RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA.

Similarly, in RAN2, any enhancements based on availability of instantaneous jitter at the gNB should be checked with SA2 and SA4.

Hence, any enhancements based on availability of instantaneous jitter at the gNB should be checked with SA2 and SA4

Some solutions can achieve dynamic adaptation without the need for jitter awareness like using DCI signalling to increase or decrease the length of C-DRX active time. This allows the network to configure small C-DRX On-Duration and increase it when needed if the jitter is large and the traffic is arriving late. Hence, better power saving can be achieved at the UE.

- Netowrk dynamically adjusts the C-DRX active time per cycle (e.g. via DCI) to address the jitter and achieve UE power saving.

# Conclusion

In this contribution, we have presented several potential enhancements for the XR service that could be studied and explored further to enable better XR power saving on 5G.

We made the following proposals:

- RAN2 does not assume instantaneous jitter is available or predictable unless further inputs from SA.

- Prioritize semi-static solutions for the periodicity alignment between C-DRX and XR traffic.

- Re-use the 38.321 equation with some modification for the start of On-Duration timer .

- Prioritize the two stages C-DRX configurations to handle the jitter.

- Support multiple simultaneous DRX configurations to serve multiple XR data flows with different characteristics and different QoS requirements

- Netowrk dynamically adjusts the C-DRX active time per cycle (e.g. via DCI) to address the jitter and achieve UE power saving.

# References

- RP-213587, “New SID: Study on XR Enhancements for NR” Nokia (moderator), 3GPP TSG RAN#94e, Dec. 2021

- R1-2204028, “Discussion on XR specific power saving techniques” OPPO, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203585, “Discussion on XR specific power saving enhancements” vivo, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203638 “Discussion on power saving enhancements for XR” Ericsson, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2205054 “Power Saving Techniques for XR” Qualcomm, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203744 “Considerations on power saving techniques for XR” Sony, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203638 “Discussion on power saving enhancements for XR” Ericsson, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203131 ”Discussion on XR-specific power saving techniques” Huawei, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2204698 “On XR specific power saving techniques” MediaTek, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2203606 “Discussion on XR specific power saving techniques” ZTE, 3GPP TSG RAN WG1 #109-e May 2022.

- R1-2205412 “Final Moderator Summary on XR specific power saving techniques” Moderator (Qualcomm), 3GPP TSG RAN WG1 #109-e May 2022.

- S4aV220921 “Reply LS on further details on XR traffic” 3GPP TSG-WG SA4 VIDEO SWG call, E-meeting October 2022.

# Appendix

- Solution 1:

- Case 1:

  - Frame Rate = 30 fps

  - drx-Cycle = 32

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 4

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 32 | 1.33333 |
| 66.6667 | 64 | 2.66667 |
| 100 | 100 | 0 |
| 133.333 | 132 | 1.33333 |
| 166.667 | 164 | 2.66667 |
| 200 | 200 | 0 |
| 233.333 | 232 | 1.33333 |
| 266.667 | 264 | 2.66667 |
| 300 | 300 | 0 |
| 333.333 | 332 | 1.33333 |
| 366.667 | 364 | 2.66667 |
| 400 | 400 | 0 |
| 433.333 | 432 | 1.33333 |
| 466.667 | 464 | 2.66667 |
| 500 | 500 | 0 |
| 533.333 | 532 | 1.33333 |
| 566.667 | 564 | 2.66667 |
| 600 | 600 | 0 |
| 633.333 | 632 | 1.33333 |
| 666.667 | 664 | 2.66667 |
| 700 | 700 | 0 |
| 733.333 | 732 | 1.33333 |
| 766.667 | 764 | 2.66667 |
| 800 | 800 | 0 |
| 833.333 | 832 | 1.33333 |
| 866.667 | 864 | 2.66667 |
| 900 | 900 | 0 |
| 933.333 | 932 | 1.33333 |
| 966.667 | 964 | 2.66667 |
| 1000 | 1000 | 0 |
| 1033.33 | 1032 | 1.33333 |

- Case 2:

  - Frame Rate = 60 fps

  - drx-Cycle = 16

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 2

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 16.6667 | 16 | 0.66667 |
| 33.3333 | 32 | 1.33333 |
| 50 | 50 | 0 |
| 66.6667 | 66 | 0.66667 |
| 83.3333 | 82 | 1.33333 |
| 100 | 100 | 0 |
| 116.667 | 116 | 0.66667 |
| 133.333 | 132 | 1.33333 |
| 150 | 150 | 0 |
| 166.667 | 166 | 0.66667 |
| 183.333 | 182 | 1.33333 |
| 200 | 200 | 0 |
| 216.667 | 216 | 0.66667 |
| 233.333 | 232 | 1.33333 |
| 250 | 250 | 0 |
| 266.667 | 266 | 0.66667 |
| 283.333 | 282 | 1.33333 |
| 300 | 300 | 0 |
| 316.667 | 316 | 0.66667 |
| 333.333 | 332 | 1.33333 |
| 350 | 350 | 0 |
| 366.667 | 366 | 0.66667 |
| 383.333 | 382 | 1.33333 |
| 400 | 400 | 0 |
| 416.667 | 416 | 0.66667 |
| 433.333 | 432 | 1.33333 |
| 450 | 450 | 0 |
| 466.667 | 466 | 0.66667 |
| 483.333 | 482 | 1.33333 |
| 500 | 500 | 0 |
| 516.667 | 516 | 0.66667 |

- Case 3:

  - Frame Rate = 30 fps

  - drx-Cycle = 33

  - drx-CyclesAdjust = 3

  - drx-CyclesOffset = 4

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 33 | 0.33333 |
| 66.6667 | 66 | 0.66667 |
| 100 | 100 | 0 |
| 133.333 | 133 | 0.33333 |
| 166.667 | 166 | 0.66667 |
| 200 | 200 | 0 |
| 233.333 | 233 | 0.33333 |
| 266.667 | 266 | 0.66667 |
| 300 | 300 | 0 |
| 333.333 | 333 | 0.33333 |
| 366.667 | 366 | 0.66667 |
| 400 | 400 | 0 |
| 433.333 | 433 | 0.33333 |
| 466.667 | 466 | 0.66667 |
| 500 | 500 | 0 |
| 533.333 | 533 | 0.33333 |
| 566.667 | 566 | 0.66667 |
| 600 | 600 | 0 |
| 633.333 | 633 | 0.33333 |
| 666.667 | 666 | 0.66667 |
| 700 | 700 | 0 |
| 733.333 | 733 | 0.33333 |
| 766.667 | 766 | 0.66667 |
| 800 | 800 | 0 |
| 833.333 | 833 | 0.33333 |
| 866.667 | 866 | 0.66667 |
| 900 | 900 | 0 |
| 933.333 | 933 | 0.33333 |
| 966.667 | 966 | 0.66667 |
| 1000 | 1000 | 0 |
| 1033.33 | 1033 | 0.33333 |

- Solution 2:

  - Case 4:

    - Frame Rate = 30 fps

    - drx-Cycle = 32

    - On-Duration = 11ms

| Traffic Arrival | start On-Duration from the proposed formula | Residual Error |
| --- | --- | --- |
| 33.3333 | 32 | 1.33333333 |
| 66.6667 | 64 | 2.66666667 |
| 100 | 96 | 4 |
| 133.333 | 128 | 5.33333333 |
| 166.667 | 160 | 6.66666667 |
| 200 | 192 | 8 |
| 233.333 | 224 | 9.33333333 |
| 266.667 | 256 | 10.6666667 |
| 300 | 300 | 0 |
| 333.333 | 332 | 1.33333333 |
| 366.667 | 364 | 2.66666667 |
| 400 | 396 | 4 |
| 433.333 | 428 | 5.33333333 |
| 466.667 | 460 | 6.66666667 |
| 500 | 492 | 8 |
| 533.333 | 524 | 9.33333333 |
| 566.667 | 556 | 10.6666667 |
| 600 | 600 | 0 |
| 633.333 | 632 | 1.33333333 |
| 666.667 | 664 | 2.66666667 |
| 700 | 696 | 4 |
| 733.333 | 728 | 5.33333333 |
| 766.667 | 760 | 6.66666667 |
| 800 | 792 | 8 |
| 833.333 | 824 | 9.33333333 |
| 866.667 | 856 | 10.6666667 |
| 900 | 900 | 0 |
| 933.333 | 932 | 1.33333333 |
| 966.667 | 964 | 2.66666667 |
| 1000 | 996 | 4 |
| 1033.33 | 1028 | 5.33333333 |