3GPP TSG-RAN WG2 #120 meeting	R2-2212537

Toulouse France, 14th –18th November 2022

Source:		NEC

Agenda item:		8.5.2.3	PDU discard

Title:		Discussion on PDU discard for XR awareness

Document for:		Discussion and decision

# 1. Introduction

In RAN2#119bis-e meeting, agreements [1] on XR-awareness were made as below:

SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU). HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets). For UE transmitter, the PDCP discard should be performed per PDU set basis. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

In this contribution, we provide our views on PDU discard for XR awareness from RAN2 point of view.

# 2. Discussion

As captured in TR 23.700-60 clause 8 [2], SA2 has discussed PDU Set based QoS framework and agreed to deliver the following parameters from CN to RAN.

The following PDU Set QoS parameters are defined to support PDU Set handling: - PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window. - PDU Set Delay Budget. - Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

For user plane, PDU Set Information listed below are informed by UPF to RAN via GTP-U header.

The following PDU Set related information may be identified by UPF to support PDU Set based handling: - PDU Set Identifier. - Optional, Start PDU and End PDU of the PDU Set. - PDU SN within a PDU Set. - Optional, PDU Set Size. - PDU Set Importance.

  - 2.1 inter-PDU Set discard

Based on LS [4] replied from SA4 to SA2, discarding a PDU Set doesn’t necessarily result in discarding dependent PDU Sets.

- Furthermore, in motion-compensated predicted video decoding, some frames refer to other frames based on the video encoding configuration but also based on dynamic operational decisions. As a consequence, a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units.

Furthermore, according to the conclusions made by SA2 [2], dependency information between PDU Sets has been removed and won’t be sent from CN to RAN. Although PDU Set importance information may be helpful for inter-PDU Set based discard, it’s not accurate enough and may bring more complexity. Hence, without accurate dependency information between PDU Sets, we propose that RAN doesn’t consider inter-PDU Set related PDU discard. To align with the behavior of downlink, and for similar reasons, we also propose UE not to consider inter-PDU Set related PDU discard on uplink.

Proposal 1: RAN2 not to consider inter-PDU Set related PDU discard.

2.2 intra-PDU Set discard

Frame/slice may only be decoded in case all PDUs comprising the frame/slice (i.e., PDU Set) are successfully received. In this case, all PDUs in a PDU Set are needed by the application layer. If a PDU Set should be decoded/handled as a whole, all PDUs in a PDU Set should share the same delay deadline. If all PDUs are needed for a PDU Set and one PDU exceeds the deadline, PDU Set-based discard can be performed to save resources and unnecessary processing of PDUs. Furthermore, less important packets can be discarded when congestion happens to improve resource efficiency.

Since RAN2#119bis-e has agreed that SDAP maps each PDU in a PDU Set to a single PDCP SDU, legacy PDCP SDU discard can be reused for PDU discard. Currently, when the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. Moreover, the discardTimer is configured only for DRBs with per DRB granularity and in the transmitter, a new timer is started upon reception of an SDU from upper layer.

For uplink, it’s assumed that packets belonging to a PDU Set arrive at the PDCP transmitter at the same time, in this case, the legacy per-SDU discard timer can still be used for PDU Set based PDU discard. When the timer expires for a PDCP SDU, all PDUs of the corresponding PDU Set can be discarded.

Observation 1: For uplink, legacy per-SDU discard timer can be used to support PDU Set based PDU discard.

Since SA2 has captured their enhancements for supporting PDU Set in downlink in [2] including “PDU Set Importance” and “PDU Set Integrated Indication”. Therefore, we observe that following clarifications on RAN2#119bis-e meeting agreements are necessary:

- Whether it is beneficial to configure different discardTimer for different PDU Set Importance.

- Whether the PDU entity still discards all PDCP SDUs associated with the PDU Set when PDU Set Integrated Indication is false.

Observation 2: Clarification on whether it is beneficial to configure different discardTimer for different PDU Set importance is needed.

Observation 3: Clarification on whether the PDU entity still discards all PDCP SDUs associated with the PDU Set when PDU Set Integrated Indication is false.

For the first clarification, we see benefits to configure different PDU discard policies depending on PDU Set importance, for example, a longer timer for more important PDU Sets to allow such PDU Sets to have much more opportunity to be transmitted. When PDU Sets with different importance are mapped to different DRBs, current per DRB discardTimer configuration can be reused to configure individual discardTimer depending on PDU Set importance. On the other hand, when PDU Sets with different importance are mapped to a single DRB, multiple discardTimer configurations associated with different importance are necessary for configuring individual discardTimer depending on PDU Set importance. Considering how to map PDU Set to DRB is up to network implementation finally, it would be worth discussing further on enhancing multiple discardTimer configurations associated with different importance per DRB.

Proposal 2: RAN2 to discuss further on enhancing multiple discardTimer configurations associated with different importance per DRB.

Another issue we would like to raise is that whether current timer based PDCP SDU discard is sufficient for PDU Sets when PDU Set Integrated Indication is “true”. For example, when out-of-order delivery happens due to PDCP PDU delivery failure and t-reordering time expiring, it would be beneficial to inform such a situation to the transmitter to allow the transmitter to discard all remained PDUs associated with the PDU Set.

Observation 4: Current timer based PDCP SDU discard is insufficient for PDU Sets when PDU Set Integrated Indication is “true”.

Proposal 3: RAN2 to discuss out-of-delivery based PDCP SDU discard for PDU Sets when PDU Set Integrated Indication is “true”.

For downlink, efficiently discarding unnecessary PDUs in the PDCP receiver may save processing overhead, for example, PDUs belonging to a dropped PDU Set may be discarded without performing deciphering/integrity verification/header decompression, etc. Furthermore, useless packets should not be delivered to the upper layer, which may reduce the decoding work of the application layer. Unlike legacy per PDCP entity t-Reordering timer, PDU Set t-Reordering timer can be considered and all PDUs of a PDU Set share the same PDU Set t-Reordering timer. The timer is started upon the first reception of a PDCP PDU belonging to a PDU Set from lower layer. When the PDU Set t-Reordering expires for a PDU Set, all PDUs of the PDU Set can be discarded.

Proposal 4: For downlink, PDU Set t-Reordering timer can be considered to support PDU Set based discard.

# 3. Conclusion

In this paper, we provide our view on PDU discard for XR awareness, and have the following proposals:

Proposal 1: RAN2 not to consider inter-PDU Set related PDU discard.

Observation 1: For uplink, legacy per-SDU discard timer can be used to support PDU Set based PDU discard.

Observation 2: Clarification on whether it is beneficial to configure different discardTimer for different PDU Set importance is needed.

Observation 3: Clarification on whether the PDU entity still discards all PDCP SDUs associated with the PDU Set when PDU Set Integrated Indication is false.

Proposal 2: RAN2 to discuss further on enhancing multiple discardTimer configurations associated with different importance per DRB.

Observation 4: Current timer based PDCP SDU discard is insufficient for PDU Sets when PDU Set Integrated Indication is “true”.

Proposal 3: RAN2 to discuss out-of-delivery based PDCP SDU discard for PDU Sets when PDU Set Integrated Indication is “true”.

Proposal 4: For downlink, PDU Set t-Reordering timer can be considered to support PDU Set based discard.

# Reference

- R2-2210802, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE

- TR 23.700-60, Study on XR (Extended Reality) and media services, v1.2.0

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR

- S4-220505, LS Reply on QoS support with PDU Set granularity