3GPP TSG-RAN WG2 Meeting #120			     R2-2213755

Toulouse, France, 14 – 18 November, 2022

Agenda Item:	8.4.2.1.1 (NR_Mob_enh2-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on TA of candidate cells for LTM

Document for: 	Discussion and Decision

Introduction

This contribution discusses an issue related to TA of candidate cells to reduce HO interruption time.

Discussion

In LTM, it is required that a UE maintains a TA for a candidate cell before handover in order to reduce the handover latency. More specifically, the UE should obtain the TA for the candidate cell before receiving the LTM handover command from a source cell. Then, if the UE receives the LTM handover command from the source cell, the UE performs the handover to the candidate cell without performing the RA procedure to the candidate cell.

In order not to perform the RA procedure to the candidate cell after receiving the LTM handover command, the TA for the candidate cell should be obtained before receiving the LTM handover command. We think there are three options as shown below.

- Option 1. After candidate cell information is received, the UE performs a RA procedure to each candidate cell. When the UE receives the candidate cell information, the UE triggers a RA procedure on each candidate cell to obtain the TA.

- Option 2. Candidate cell information also includes TA information for each candidate cell. The UE applies a received TA for each candidate cell.

- Option 3. The network sends a (new) TAC MAC CE for a candidate cell before (or together with) sending LTM handover command. When the TAC MAC CE is received for a candidate cell, the UE applies the TA in the TAC MAC CE for the candidate cell.

There are pros and cons for each option from a general perspective.

- Option 1.

  - Pros: The network does not need to know the TA for the candidate cell. The UE can reuse legacy procedure, i.e. RA procedure, to obtain the TA.

  - Cons: If multiple candidate cells are configured, the UE should perform RA procedure multiple times. In addition, if TAT for candidate cell expires, the UE should perform RA procedure again for the candidate cell.

- Option 2.

  - Pros: No need to perform RA procedure to candidate cells.

  - Cons: The network has to know the TA for the candidate cell. Signalling overhead is increased because candidate cell information also includes TA information for each candidate cell. In addition, the network has to send candidate cell information again before TAT for candidate cell expires.

- Option 3.

  - Pros: The UE maintains only a TA for a candidate cell to which the network wants to move the UE. The TAC MAC CE can be used as LTM handover command.

  - Cons: The network has to know the TA for the candidate cell. A new TAC MAC CE may need to be introduced to indicate the TA for a target candidate cell.

On top of pros and cons above, as RAN2 also confirm to support LTM for inter-DU scenario, we need to check which options are proper for both intra-DU and inter-DU scenarios.

Given that the intra-DU scenario does not require the RLC re-establishment and the MAC reset, all options are feasible, but actually the option 1 may not be needed for the intra-DU scenario because all candidate cells belong to one DU which currently serves the UE and TA for all candidate cells can be achieved without RA procedure.

Observation 1. For intra-DU scenario, the option 1 may not be needed and the option 2 and 3 would be sufficient.

On the other hand, the inter-DU scenario would require the RLC re-establishment and the MAC reset due to the DU change and TA for candidate cells for inter-DU scenario may not be obtained without RA procedure, so the option 1 is needed, but the option 2/3 may not be feasible for the inter-DU scenario. In addition, since the inter-DU scenario requires a RA procedure to obtain TA for candidate cell on the not serving MAC entity, user plane impact such as CFRA on SCell and BWP handling on the not serving MAC entity which is associated with the target DU should be carefully reviewed.

Observation 2. For inter-DU scenario, the option 2 and 3 may not be feasible and the option 1 is needed, but user plain impact due to RA on the not serving MAC entity would be studied.

Of course, it would be the best to design the unified solution to cover both scenarios. However, considering the observation 1/2 and pros/cons above, we think that it is better to define a different solution for two scenarios, i.e., if RA procedure is not needed to obtain TA for candidate cell like intra-DU scenario, the option 2 or 3 should be selected, but if RA procedure is needed to obtain TA for candidate cell like inter-DU scenario, the option 1 should be selected. Actually all aspects mentioned above are considered and now discussed in RAN1. Considering that RAN2 impact can be different depending on RAN1 conclusions, it would be better to start TA related discussion in RAN2 after RAN1 made further details for TA management.

Observation 3. It may be infeasible to have the unified solution for both intra-DU and inter-DU scenarios for handling TA of candidate cells.

Observation 4. RAN2 impact can be different depending on RAN1 conclusions for TA management.

Proposal 1. For TA of candidate cells, RAN2 wait for RAN1 progress on TA management.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. For intra-DU scenario, the option 1 may not be needed and the option 2 and 3 would be sufficient.

Observation 2. For inter-DU scenario, the option 2 and 3 may not be feasible and the option 1 is needed, but user plain impact due to RA on the not serving MAC entity would be studied.

Observation 3. It may be infeasible to have the unified solution for both intra-DU and inter-DU scenarios for handling TA of candidate cells.

Observation 4. RAN2 impact can be different depending on RAN1 conclusions for TA management.

Proposal 1. For TA of candidate cells, RAN2 wait for RAN1 progress on TA management.