3GPP TSG-RAN WG2 Meeting #120	R2-2212939

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.16.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Potential impacts due to the use case specific aspects

SID:	FS_NR_AIML_air (Rel-18)

Document for:	Discussion and Decision

# 1	Introduction

In [1], RAN plenary approved the RAN1-led SI for studying the application of AI/ML techniques to the NR air interface. More specifically, the following use cases are in scope of the study item.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

In this contribution we provide some initial views on exploring the potential impact of the specific use cases and sub-use cases and the related AIML methods that are applicable to them. The document is structured accordingly.

# 2	CSI feedback enhancement

## 2.1	CSI compression

As part of the Rel-18 study item on AI/ML for air interface, one key use case for AI/ML is CSI feedback compression, which involves two-sided models with an encoder deployed on the UE side and a decoder on the gNB side. AI/ML-based CSI compression aims at finding a better quantized and efficient representation of the underlying DL channel than the legacy codebook-based mechanism can do. Inputs (and the associated outputs) like raw channels, eigenvectors, and precoders are being considered. In terms of model training strategy, currently two general categories of model training are being discussed: joint training and separate training [2].

The RAN1 agreements made in earlier meetings are copied here for reference:

Agreement [RAN1#109-e] For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. - At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement [RAN1#110] In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring: - Assistance signaling for UE’s data collection - Assistance signaling for gNB’s data collection - Delivery of the datasets.

Based on the above agreements for CSI feedback compression, the following aspects may have specification impact for both RAN1 and RAN2: on the aspects of model configuration, model performance monitoring, data collection for model training.

- Signalling to support field channel data collection

- Signalling to support model performance monitoring

- Signalling to support model selection/switching

- Signalling to support model configuration/activation/de-activation

Proposal 1: For AI/ML-based CSI compression, RAN2 should study the signalling support for data collection, performance monitoring, model activation/de-activation/selection/switching.

## 2.2	CSI prediction

Conclusion [RAN1#110] If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

CSI prediction is one of the sub use cases in RAN1 for future AI/ML applications. There is a general differentiation between one sided and two-sided channel prediction methods, but the focus for the time being is on one sided channel prediction and especially on UE sided prediction. In that case the UE can apply any type of channel prediction methods like physical layer based Kalman filtering, or certain AI/ML neural networks like LSTMs. Then, the main difference to conventional Type II CSI is the reporting of CSI for a future time instance instead for the latest current one.

From that perspective, the impact to RAN2 might be limited and, for example, related to the identification of the UE capabilities like the possible prediction horizon, the needed inference time, the required “observation time” of the radio channel required to infer a channel prediction, etc. The “observation time” is a relevant information for the gNB as it has to configure the related CSI RS to enable the UE sided channel estimation and it impacts the earliest scheduling opportunity for the given UE. Other information might be related to the achievable prediction accuracy and reliability.

With respect to minimizing the number of required ML models it is important to cover a large variety of scenarios, which leads then to varying channel prediction performance for different UE locations. For that purpose, we consider UE specific fine-tuning methods of one or few generalized AI/ML models, which again requires some form of gNB-UE collaboration to achieve high performance with low overhead, e.g., for CSI RSs.

Two-sided channel prediction methods are for the time being a research topic, promising similar or improved channel prediction performance with lower signaling overhead. These methods benefit from tight cooperation of UE and gNB sided AI/ML models and would correspondingly involve many RAN2 aspects but are currently out of scope.

In the future, more advanced channel prediction methods might benefit from assistance information like UE locations, SINR conditions, cell load and others, but such issues are so far out of scope as well.

Observation 1: The sub use case CSI prediction is so far limited to basic UE sided channel prediction methods. Based on expected performance gains this sub use case still has to be confirmed (by RAN1) to be part of the NR SI AI/ML for the air interface.

# 3	Beam management

Agreement [RAN1#109-e] For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations - BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams - BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams - FFS: details of BM-Case1 and BM-Case2 - FFS: other sub use cases Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

Agreement [RAN1#110] For the data collection for AI/ML model training (if supported), study the following aspects as a starting point for potential necessary specification impact: - Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported), Reference signals - Content/type of the collected data - Other aspect(s) is not precluded Agreement [RAN1#110] Regarding the model monitoring for BM-Case1 and BM-Case2, to investigate specification impacts from the following aspects - Performance metric(s) - Benchmark/reference for the performance comparison - Signaling/configuration/measurement/report for model monitoring, e.g., signaling aspects related to assistance information (if supported), Reference signals - Other aspect(s) is not precluded Agreement [RAN1#110] In order to facilitate the AI/ML model inference, study the following aspects as a starting point: - Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting - Enhanced or new signaling for measurement configuration/triggering - Signaling of assistance information (if applicable) - Other aspect(s) is not precluded Conclusion [RAN1#109-e] Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input: - Alt.1: Only L1-RSRP measurement based on Set B - Alt.2: L1-RSRP measurement based on Set B and assistance information - FFS: Assistance information. The following were mentioned by companions in the discussion: Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side. - Alt.3: CIR based on Set B - Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID - Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives - Note2: All the inputs are “nominal” and only for discussion purpose.

As part of the Rel-18 study item on AI/ML for air interface, one key use case for AI/ML is beam prediction for enhancing beam management performance and/or reducing the associated overhead due to CSI-RS resources and/or reporting overhead.  AI/ML-based beam prediction involves predicting the best beam(s) or predicting the ranking or other quantities (i.e. RSRP(s)) of all beams based on a limited set of measurements. Two variants of AI/ML-based beam prediction are being studied in RAN1: spatial-domain beam prediction (BM Case 1) and temporal-domain beam prediction (BM Case 2).  Both variants may require spec impact for RAN2.

Observation 2: AI/ML for beam management may require supporting up to two variants of beam prediction each potentially requiring additional signalling support: 1) AI/ML for spatial-domain beam prediction and 2) AI/ML for time-domain prediction.

## 3.1	Background for Beam Prediction

BM Case-1 - Spatial-domain DL beam prediction

In spatial-domain beam prediction, a limited set of beams are measured and reported by the UE, and the best beam for the UE is determined based on that limited set of measured beams. In spatial domain beam prediction, the limited set of measurements generally includes beam measurements that do not contain any historical information and therefore do not enable any tracking of the time dimension of the channel. For spatial-domain beam prediction, the beam prediction operation is predicting beam(s) belonging to a Beam Set A based on measurements performed on a Beam Set B (the definitions of Beam Set A and Beam Set B are in RAN1).

BM Case-2 - Temporal DL beam prediction

In time-domain beam prediction, the ML model can predict the best beam for a UE based on a set of limited measurements that includes historical information.  For example, the set of measurements could include a history of the best beam index selected by the UE with optional inclusion of the corresponding RSRP and/or UE position information.  One purpose for this prediction can be to lower the RS overhead by narrowing down a candidate set of best beam(s) for mobile UEs.  Another purpose for this prediction can be to increase the time period between the transmission of CSI-RS resource sets for beam refinement (e.g., CRI with RSRP feedback) for mobile UEs, which not only would decrease the CSI-RS overhead but would also reduce the reporting overhead since the time period between UE reports would also be increased.  For time-domain beam prediction, the beam prediction operation is predicting beam(s) belonging to a Beam Set A based on historic measurement results obtained on a Beam Set B (the definitions of Beam Set A and Beam Set B are in RAN1).

## 3.2	Impact of Beam Prediction

For both variants of AI/ML-based beam prediction described above, the following aspects may result in specification impact for RAN1 and RAN2: data collection for model training, model inference, and model monitoring.  In addition to these main aspects, RAN1 is studying the use of assistance information, which is any additional information that can be used to improve the overall performance of the AI/ML-based beam prediction. Examples of assistance information include but are not limited to Tx and/or Rx beam angle, UE direction information, positioning-related measurement, Tx and/or Rx beam shape information, and UE orientation information.

The potential spec impact (if any) of model training is being studied in RAN1 for the case where AI/ML model training is at the network side or at the UE side where the choice of offline versus on-line training is being considered. With respect to model training, the following aspects may require spec impact in terms of new signalling and/or configuration(s):

- Signalling/configuration/measurement/report for collecting training data, e.g., signalling aspects related to assistance information (if supported), and Reference signals.

- Content/type of the collected data

Observation 3: For model training, AI/ML for beam management impacts signalling and/or configuration of measurement reports for training data collection, e.g., signalling aspects related to any assistance information and reference signal configurations.

Proposal 2: For AI/ML based beam management, RAN2 assesses signalling impacts and/or configuration for measurement reports for training data collection, e.g., signalling aspects related to assistance information and reference signal configurations.

With respect to model inference, the following aspects may require signalling and/or configuration:

- Enhanced or new configurations/UE reporting/UE measurement, e.g., enhanced or new beam measurement and/or beam reporting

- Enhanced or new signalling for measurement configuration/triggering

- Signalling of assistance information (from the other side) if applicable (e.g., depending on where inference is happening)

Observation 4: For model inference, AI/ML for beam management impacts signalling and or configuration of UE reporting, UE measurements, e.g., new beam measurement and/or beam reporting, enhancing the signalling for measurement configurations and triggering of those measurements.

Proposal 3: For AI/ML based beam management, RAN2 assesses signalling impacts and or configuration of UE reporting, UE measurements, e.g., new beam measurement and/or beam reporting, enhancing signalling for measurement configurations and triggering of those measurements.

With respect to model monitoring, the following aspects may require signalling and/or configuration:

- measurement reports for data collection

- performance metrics

- benchmark or reference for the performance comparison

- model monitoring e.g., signalling aspects related to assistance information (if supported), Reference signals, wherein other aspects may not be precluded at this time.

- triggering possible model switching based on the collected measurements

Observation 5: For model monitoring, AI/ML for beam management may require measurement report extensions (or separate messages) for collecting performance metrics or benchmarks and signalling of AI/ML assistance information related to monitoring the AI/ML model.

Proposal 4: For AI/ML based beam management, RAN2 should study the need for measurement report extensions (or separate messages) for collecting performance metrics or benchmarks and signalling of AI/ML assistance information related to monitoring the AI/ML model.

# 4	Positioning accuracy enhancement

Agreement [RAN1#109-e] For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement

Agreement [RAN1#109-e] Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

Agreement [RAN1#110] Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded Agreement [RAN1#110] Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded Agreement [RAN1#110] Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity

Agreement [RAN1#110-bis-e] Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded Agreement [RAN1#110-bis-e] Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement [RAN1#110-bis-e] Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded Agreement [RAN1#110-bis-e] Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

## 4.1	Architecture Options for AI/ML-based Positioning

In 5G NR positioning, three main entities: LMF; gNB; and UE, and three main protocols: LPP; NRPPa; and RRC are involved which are also being discussed in RAN1 [3]. LMF is the entity that manages overall coordination, provides assistance information, and position estimation. The LMF configures positioning methods that trigger the gNB to configure the transmission of UL and DL resources for measurement by the gNB and UE, which are required for positioning a UE.

The position estimate computation may be made by the UE or by the LMF, which are referred to as the following types of positioning in 5G NR:

- UE-based: UE computes its own position

- LMF-based (UE-assisted or NG-RAN node assisted): UE or NG-RAN node provides measurements to the LMF, and LMF computes the position

During RAN1#110-bis, it was agreed to study 5 use cases for AI/ML positioning that map to legacy positioning method configurations. All of these only support one-sided models with the models residing on the UE, gNB, or the LMF, and indicate the relevant nodes as providing assistance or the location (e.g., UE-based, LMF-based, etc.). Use cases 1, 2a, and 2b could be considered first since they fall clearly under LPP and, therefore, in RAN2 scope, aside from potential NRPPa signalling to the gNB for the purpose of UE measurement configuration.

Proposal 5: RAN2 should study specification impacts of AI/ML architecture wherein the AI/ML model functionalities reside on the UE or the LMF: UE-based with UE-side model; UE-assisted with UE-side model; and UE-assisted with LMF-side model.

Architectural aspects such as which entity collects and maintains data, which entity or entities maintain, train, test and update models need to be decided before signalling solutions can be developed. The types of protocol extensions required will depend on the directionality of the required messaging, new types of triggers introduced, and the frequency, format, and size of new data to be transmitted over the network. Depending on the scenario, the specification impact may differ in complexity. For example, an LMF-side model that requests input for inference from the UE and obtains its own training data through its normal operation will be able to reuse much of the pre-existing signalling, while a UE-side model requiring training data from the LMF will require new signalling.

Observation 6: Architectural aspects such as which entity performs data collection and management, and which entity or entities are involved in AI/ML functionalities and LCM, need to be decided before any signalling solutions can be developed.

LMF may interact with a target UE in order to exchange location information applicable to UE-assisted and UE-based position methods and interacts with the NG-RAN in order to obtain location information or to provide assistance data information for broadcasting. For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the required QoS, UE positioning capabilities, and gNB positioning capabilities. The LMF may interact with the AMF to receive stored UE Positioning Capability from AMF, or to provide (updated) UE Positioning Capability to AMF.

The UE may make measurements of DL signals from NG-RAN and other sources such as E-UTRAN or RAT-independent techniques. The UE may also contain LCS applications or access an LCS application that may include measurement and calculation functions to determine the UE's position with or without network assistance. The UE may also report its position (independent of the NG-RAN transmissions).

The gNB may make measurements of radio signals for a target UE and provide measurement results for position estimation. A gNB may serve several TRPs, and may broadcast assistance data information, received from an LMF.

Besides LMF, a recent study on further enhancements to the 5GC LCS further considers involvement of the core network entity, Network Data Analytics Function (NWDAF) [3]. NWDAF can generate different analytics data, e.g., UE mobility analytics, and 5GC NF (e.g., AMF, PCF) can request analytics data from NWDAF for decision making. Similarly, there has been agreements in the study that NWDAF can interact with LCS. Several use cases according to [3] include the following:

- An ML model is trained using data like positioning estimate, positioning method used, assistance data, UE ID, time of the day etc. and corresponding accuracy. This model is then subsequently used by the LMF to determine if its location estimate meets the accuracy required and take appropriate action.

- A navigation service can take into account areas where location accuracy cannot be met when providing route guidance based on analytics from the NWDAF

- Location verification of the UE location provided via satellite access by leveraging NWDAF analytics in the 5GC

Observation 7: RAN2 may consider the signalling support needed for AI/ML enablement from other working groups (e.g. SA2) including RAN1.

In RAN1, the agreements consider either “UE” or “network” side, but do not further specify entities at the network side, such as gNB or LMF. RAN1 should make further progress or clarify which entity they refer to by “network”.

Proposal 6: RAN2 should wait for RAN1 to clarify the involved entities (e.g., UE, gNB, LMF) that are required per sub-use case or solution related to AI/ML-based positioning.

## 4.2	Procedures and Protocols for AI/ML-based Positioning

Similarities between the execution of legacy and AI/ML positioning can be used to minimize the complexity of introducing AI/ML positioning. In previous releases, the following have been successfully extended to introduce new positioning methods: capability information exchange; measurement configuration; measurement transmission (input for inference); position assistance output (model output); and position estimate output (model output). See Section 6.1 for more details on existing common procedures for positioning. New positioning methods will require Provide and Request IEs for the exchange of capabilities, location, and measurements.

Proposal 7: RAN2 should study reusable and extendable aspects of LPP and RRC for AI/ML-enabled positioning functionality instead of defining new interfaces and protocols when possible.

However, the effect of reusing messages from legacy positioning methods for new AI/ML-based counterparts should be analyzed as to avoid contaminating IEs associated with legacy positioning methods with AI/ML parameters.

Observation 8: Reuse of legacy positioning protocol messaging could result in their future contamination if AI/ML parameters are deemed necessary to be added.

Proposal 8: Extensions to LPP and RRC shall not interfere with the operation of legacy positioning methods.

There are aspects of AI/ML-based methods that are inherently incompatible with LPP and RRC in their current forms. Procedures such as model monitoring, model training, model validation and model configuration, and indications of preference or recommendations do not exist. Neither do pre-existing procedures exist that could be modified to support these functions.

Observation 9: Neither procedures such as model monitoring, training, validation and configuration, and indications of preference or recommendation, nor equivalents, exist in LPP and RRC.

Proposal 9: RAN2 should wait for the required signalling and protocol procedures in positioning enhancements related to AI/ML model life cycle management. RAN2 should coordinate with other working groups prior to working on protocol modifications to support them.

# 5	Conclusion

In this contribution we have provided some initial thoughts on exploring the potential impact of the specific use cases and sub-use cases and the related AIML methods that are applicable to them. The document dealt with the sub use cases for CSI feedback enhancements, beam management aspects and the positioning accuracy enhancements. Although the discussions actively continue in RAN1, RAN2 can aim to focus its contributions based on the following observations and proposals.

The observations are listed below:

CSI feedback enhancement:

Observation 1: The sub use case CSI prediction is so far limited to basic UE sided channel prediction methods. Based on expected performance gains this sub use case still has to be confirmed (by RAN1) to be part of the NR SI AI/ML for the air interface.

Beam management:

Observation 2: AI/ML for beam management may require supporting up to two variants of beam prediction each potentially requiring additional signalling support: 1) AI/ML for spatial-domain beam prediction and 2) AI/ML for time-domain prediction.

Observation 3: For model training, AI/ML for beam management impacts signalling and/or configuration of measurement reports for training data collection, e.g., signalling aspects related to any assistance information and reference signal configurations.

Observation 4: For model inference, AI/ML for beam management impacts signalling and or configuration of UE reporting, UE measurements, e.g., new beam measurement and/or beam reporting, enhancing the signalling for measurement configurations and triggering of those measurements.

Observation 5: For model monitoring, AI/ML for beam management may require measurement report extensions (or separate messages) for collecting performance metrics or benchmarks and signalling of AI/ML assistance information related to monitoring the AI/ML model.

Positioning enhancement:

Observation 6: Architectural aspects such as which entity performs data collection and management, and which entity or entities are involved in AI/ML functionalities and LCM, need to be decided before any signalling solutions can be developed.

Observation 7: RAN2 may consider the signalling support needed for AI/ML enablement from other working groups (e.g. SA2) including RAN1.

Observation 8: Reuse of legacy positioning protocol messaging could result in their future contamination if AI/ML parameters are deemed necessary to be added.

Observation 9: Neither procedures such as model monitoring, training, validation and configuration, and indications of preference or recommendation, nor equivalents, exist in LPP and RRC.

The proposals are listed below:

CSI feedback enhancement:

Proposal 1: For AI/ML-based CSI compression, RAN2 should study the signalling support for data collection, performance monitoring, model activation/de-activation/selection/switching.

Beam management:

Proposal 2: For AI/ML based beam management, RAN2 assesses signalling impacts and/or configuration for measurement reports for training data collection, e.g., signalling aspects related to assistance information and reference signal configurations.

Proposal 3: For AI/ML based beam management, RAN2 assesses signalling impacts and or configuration of UE reporting, UE measurements, e.g., new beam measurement and/or beam reporting, enhancing signalling for measurement configurations and triggering of those measurements.

Proposal 4: For AI/ML based beam management, RAN2 should study the need for measurement report extensions (or separate messages) for collecting performance metrics or benchmarks and signalling of AI/ML assistance information related to monitoring the AI/ML model.

Positioning enhancement:

Proposal 5: RAN2 should study specification impacts of AI/ML architecture wherein the AI/ML model functionalities reside on the UE or the LMF: UE-based with UE-side model; UE-assisted with UE-side model; and UE-assisted with LMF-side model.

Proposal 6: RAN2 should wait for RAN1 to clarify the involved entities (e.g., UE, gNB, LMF) that are required per sub-use case or solution related to AI/ML-based positioning.

Proposal 7: RAN2 should study reusable and extendable aspects of LPP and RRC for AI/ML-enabled positioning functionality instead of defining new interfaces and protocols when possible.

Proposal 8: RAN2 ensures that extensions to LPP and RRC shall not interfere with the operation of legacy positioning methods.

Proposal 9: RAN2 waits for the required signalling and protocol procedures in positioning enhancements related to AI/ML model life cycle management. RAN2 coordinates with other working groups prior to working on protocol modifications to support them.

# 6	Annex

## 6.1	Currently Supported Positioning Procedures

For reference, the following standard positioning procedures are currently supported:

- General LPP procedures [4]

  - Exchange of positioning capabilities;

  - Transfer of assistance data;

  - Transfer of location information (positioning measurements and/or position estimate);

  - Error handling;

  - Abort.

- General NRPPa procedures

  - UE associated procedure, i.e., transfer of information for a particular UE, including the procedures supporting the Positioning Information Transfer and E-CID Location Information Transfer functions;

  - Non-UE associated procedure, i.e., transfer of information applicable to the NG-RAN node and associated TRP, including the procedures supporting the OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, and Measurement Information Transfer functions.

  - Important note: An NRPPa transaction is only initiated by the server.

- General RRC procedures

  - Location Measurement Indication: used by the UE to request measurement gaps for OTDOA RSTD measurements, for subframe and slot timing detection for inter-RAT E-UTRA RSTD measurements, or for NR DL-PRS measurements.

  - UE Positioning Assistance Information: used by UE to report the UE Positioning Assistance Information for UL-TDOA. The UE reports the association between UL-SRS resources for positioning and the UE Tx TEG ID.

- Procedures for Broadcast of Assistance Data

- Procedures for On-demand PRS

- Procedures for Pre-configured Measurement Gap

- Procedures for Pre-configured PRS processing window

# 7	References

[1]	RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

[2]	3GPP RAN1 110 meeting, “Draft Report of 3GPP TSG RAN WG1 #110,” Aug. 2022.

[3] 3GPP TR 23.700-71, “Study on enhancement to the 5GC LoCation Services (LCS); Phase 3; (Release 18)”, Oct. 2022

[4] 3GPP TS 38.305, “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17)”, Sept. 2022