3GPP TSG-RAN WG2 Meeting #121		R2-220xxxx

Athens, Greece, 27th February – 3rd March, 2023

Source:	vivo

Title:	[AT121][014][PowSav] RLM/BFD relaxation for SCG deactivation (vivo)

Agenda Item:	6.1.3.1

Document for:	Discussion and Decision

- Introduction

The document summarizes the following offline discussion:

R2-2300308 Discussion on RLM/BFD relaxation for SCG deactivation vivo discussion Rel-17 NR_UE_pow_sav_enh-Core Proposal 1: RAN2 to discuss the below options to handle the RLM/BFD relaxation for SCG deactivation: - Option 1: Change RAN2 conclusion as: RLM/BFD relaxation and SCG deactivation with bfd-and-RLM configuration is true cannot be configured simultaneously. - Option 2: Keep the current situation, i.e. no change in RAN2, no change in RAN4. DISCUSSION - CATT support O2, think too late to change. - OPPO prefers to have consistency, prefer O1. - Ericsson agree we cannot change R4 req. think O2 can work ok, and that UE can have a configuration even though there are no new requirements for the deactivated case. - Offline 014 (vivo), converge the discussion

- Contact information

| Company | Name and email address |
| --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |
| Xiaomi | Liyanhua1@xiaomi.com |
| ZTE | Dong.fei@zte.com.cn |
| Qualcomm | Punyaslok Purkayastha (punyaslo@qti.qualcomm.com) |

- Discussion

According to the online spirit from Chair, we should try to conclude on option 2 (Keep the current situation, i.e. no change in RAN2, no change in RAN4) to see if there is any problem.

- Companies are invited to provide your views if they still have concern on Option 2, and provide why.

| Company’s name | Comments, if any |
| --- | --- |
| Xiaomi | No strong view. Even though option1 is more reasonable to solve the consistency. But that need we the modification to 331 and 37.340. Seems option2 can work and in that case, UE will report that UE is not relaxing RLM/BFD measurements when the SCG is de-activated. |
| ZTE | As we comment during the online discussion, we would clarify what is the UE behavior in the case of the good serving cell criteria is still configured but the SCG is deactivated. If we really do not want to change anything in the stage 3 specification, we need capture the conclusion in the chairman notes: RAN2 clarify that in the case of SCG deactivation and bfd-and-RLM is set to true, UE will perform the RLM and BFD according to the requirements for SCG deactivation as specified in TS 38.133 no matter whether goodServingCellEvaluationBFD-r17 and goodServingCellEvaluationRLM-r17 is configured for SCG. |

During online and offline discussion, it seems most companies understand option 2 (Keep the current situation, i.e. no change in RAN2, no change in RAN4), should not change any specification in RAN2.

- Companies are invited to provide your views if anything need to be change in RAN2 specification.

| Company’s name | Comments, if any |
| --- | --- |
| Xiaomi | Yes. After talking to RAN4 people, they did not have relaxed RLM/BFD requirements applying for SCG deactivated cells is based on the reason that when the SCG is de-activated the configured measCyclePSCell (160, 256, 320, 512, 640, 1024 or 1280 ms) is already longer thant 80ms. So we have sympathy for R2-2301201(RLM and BFD relaxation when SCG is deactivated Ericsson) with some change that measCyclePSCell is mandatory in case network simultaneously configures the UE to perform radio link monitoring on the SCG and beam failure detection on the SCG while SCG is deactivated. Conditional Presence; Explanation. <TEXT OMITTED>. SCG; This field is present in the measConfig associated with the deactivated SCG when network simultaneously configures the UE to perform radio link monitoring on the SCG and beam failure detection on the SCG . It is absent in the measConfig associated with the MCG. Otherwise it is optionally present, Need M.. <TEXT OMITTED>. |
| ZTE | No strong view. We also have a same sympathies with the intention of the tdoc R2-2301201. But we do not like introducing the need-M parameter in the MO configuration, it means if we want to delete the configuration we need to release a whole MO which may reset the releated measurement. We suggest it can rely on gNB implementation to guarantee the measCyclePSCell is always present for SCG deactivation. |
| Qualcomm | We think we should have Ericsson’s clarification as in the Tdoc R2-2301201 which clarifies that measCyclePSCell is present when SCG is deactivated. |

In R2-2300308, company think if we go option 2, we should inform RAN4 our condition, but expect No response.

- Companies are invited to provide your views whether need to reply LS to RAN4:

- Option 1: Yes, just inform them our conclusion.

- Option 2: No.

| Company’s name | Yes, No | Comments, if any |
| --- | --- | --- |
| Xiaomi | No | I guess it is not needed. Because that will not change anything they have decided. People can inform their RAN4 people. LS is not needed. |
| ZTE | No |  |
| Qualcomm | No |  |

- Conclusion

This contribution summarizes the offline discussion: Report of [AT121][014][PowSav] RLM/BFD relaxation for SCG deactivation (vivo), and achieves the following proposals:

- Reference