3GPP TSG-RAN2 Meeting 121                                                R2-2302149

Athens, Greece, February-March, 2023

Agenda Item:	6.5.3

Source:	OPPO

Title:	Summary of [AT121][408][Relay] Wording for Rel-17 relay UP CRs (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the following offline discussion

- [AT121][408][Relay] Wording for Rel-17 relay UP CRs (OPPO)

Scope: Confirm the details of the agreeable CRs on Rel-17 relay user plane.

Intended outcome: Agreeable CRs

Deadline: Wednesday 2023-03-01 1900 EET

# Discussion

## Change 1 of R2-2301123

The intention of change-1 in R2-2301123 has been agreed, and the detailed wording is to be checked

- Change 1 of R2-2301123 is taken into discussion [408] for wording; change 2 is not pursued.

## 4.5 Configurations The configuration of the SRAP entity for U2N Remote UE includes: - Mapping from a radio bearer to egress PC5 Relay RLC channel via RRC; - The local identity via RRC. The configuration of the SRAP entity for U2N Relay UE includes: - The local identity for each U2N Remote UE via RRC; - Mapping from a radio bearer to egress Uu Relay RLC channel for each U2N Remote UE via RRC; - Mapping from a radio bearer to egress PC5 Relay RLC channel for each U2N Remote UE via RRC.

Rapp understand in the current spec, it is clear that remote UE would only rely on bearer-ID while relay UE would rely on both UE-ID and bearer-ID for the mapping, to maintain this information, but also to align the wording, potential rewording can be

The configuration of the SRAP entity for U2N Remote UE includes: - Mapping from a radio bearer identified by BEARER ID field to egress PC5 Relay RLC channel via RRC; - The local identity via RRC. The configuration of the SRAP entity for U2N Relay UE includes: - The local identity for each U2N Remote UE via RRC; - Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress Uu Relay RLC channel for each U2N Remote UE via RRC; - Mapping from a radio bearer identified by UE ID field and BEARER ID field to egress PC5 Relay RLC channel for each U2N Remote UE via RRC.

The following question is to check companies view on change 1 of R2-2301123.

Q1: Which do you prefer on the wording to reflect the intention of change 1 in R2-2301123?

Option-1: Original wording in change 1 of R2-2301123;

Option-2: Reworded proposed as above.

| Company | Option1/2 | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| OPPO | 2 | In this way, we do not miss any info, but also ensure the text alignment. |
| Nokia | 2 | Seems clearer. |
| - ZTE | - 1 | - I understand the point OPPO said, we also thought about the alternative way for the change when we prepare the CR, but we are wondering whether it is appropriate to mention how to identify the RB using the SRAP field names. Anyway the bearer mapping is specified in RRC spec and not use the SRAP fields. |
| vivo | 2 | Agree with the Rapporteur. |
| Sharp | 2 |  |
| Futurewei | 2 |  |
| Xiaomi | 2 |  |
| Samsung | - 2 | - Agree with OPPO and Nokia. |
| ASUSTeK | 2 |  |
| CATT | - 2 |  |
| Huawei, HiSilicon | - 2 |  |
| Apple | - 1 | - The change proposed by ZTE is more clear because UE ID and BEARER ID terms are not used in RRC |
| MediaTek | - 2 |  |
| LG | - 2 |  |
| InterDigital | - 2 |  |

[Rapp summary]

15 companies replied this question, 13 companies prefer option 2 and 2 companies prefer option 1, Rapp suggest to go for majority view on this issue.

- Recommendation1	For change 1 of R2-2301123, it is agreed as “Mapping from a radio bearer identified by UE ID field and BEARER ID field to…”.

## Change 3 of R2-2301123

The intention of change-3 in R2-2301123 has been agreed, and the detailed wording is to be checked

Proposal 2	RAN2 to agree on the intention of R2-2301123 (change-3), R2-2301176, R2-2301351 (change-1), and wording is to be checked in CR discussion.

- Changes listed in P2 are agreed in intention, with wording to be finalised in discussion [408].

- 5.3.3.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - if there is an entry in sl-SRAP-ConfigRelay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity for BEARER ID plus 1):

The following question is to check companies view on change 3 of R2-2301123.

Q2: Do you agree with the current wording of change 3 in R2-2301123?

| Company | Agree/Disagree | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| Nokia | Disagree | and for DRB, the DRB identity is BEARER ID plus 1 |
| ZTE | agree | For UL data transfer, remote UE determines BEARER ID corresponding to DRB identity minus 1 for DRB. When Relay UE determines egress Uu RLC channel by matching bearer mapping of the RB identity based on BEARER ID, for DRB, the DRB identity is BEARER ID plus 1. It shall be clarified in the spec. |
| Sharp | Agree |  |
| Xiaomi | Agree |  |
| - Samsung | - Agree with intention but… | - Perhaps a better way to make this clarification is to place it in 5.3.3.1 UE ID field and BEARER ID field determination? |
| CATT | Disagree | We can agree with Nokia’s modification. |
| Apple | Agree |  |
| MediaTek |  | Agree with Nokia. |
| LG |  | Agree with Nokia. |
| InterDigital | Agree | Agree with Nokia |

[Rapp summary]

10 companies replied, 9 companies are fine with the rewording by Nokia, so Rapp suggest we go for Nokia’s rewording for this change.

- Recommendation2	For change 3 of R2-2301123, it is agreed as “and for DRB, the DRB identity is BEARER ID plus 1”.

## Change of R2-2301176

The intention of change in R2-2301176 has been agreed, and the detailed wording is to be checked

Proposal 2	RAN2 to agree on the intention of R2-2301123 (change-3), R2-2301176, R2-2301351 (change-1), and wording is to be checked in CR discussion.

- Changes listed in P2 are agreed in intention, with wording to be finalised in discussion [408].

- 5.4 Handling of unknown, unforeseen, and erroneous protocol data For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall: - discard the received SRAP Data PDU. For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall: - discard the received SRAP Data PDU. When the U2N Remote UE or the U2N Relay UE receives a SRAP Control PDU or a SRAP PDU with invalid values, the SRAP entity shall: - - discard the received SRAP PDU.

Rapp understand besides invalid value, the reserved value can also be included, in the similar manner as for PDCP / RLC spec.

<In 38.322> - 5.6 Handling of unknown, unforeseen and erroneous protocol data - 5.6.1 Reception of PDU with reserved or invalid values When an RLC entity receives an RLC PDU that contains reserved or invalid values, the RLC entity shall: - discard the received RLC PDU. <In 38.323> - 5.10 Handling of unknown, unforeseen, and erroneous protocol data When a PDCP PDU that contains reserved or invalid values is received, the receiving PDCP entity shall: - discard the received PDU.

So, the following rewording is suggested.

- 5.4 Handling of unknown, unforeseen, and erroneous protocol data For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall: - discard the received SRAP Data PDU. For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall: - discard the received SRAP Data PDU. When the U2N Remote UE or the U2N Relay UE receives a SRAP Control PDU or a SRAP PDU with invalid or reserved values, the SRAP entity shall: - discard the received SRAP PDU.

The following question is to check companies view on change of R2-2301176.

Q3: Which do you prefer on the wording to reflect the intention of change 1 in R2-2301176?

Option-1: Original wording in R2-2301176;

Option-2: Reworded proposed as above.

| Company | Option1/2 | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| OPPO | 2 | So to align with other UP spec. |
| Nokia | 2 with changes | When the U2N Remote UE or the U2N Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall: - discard the received SRAP PDU. SRAP Control PDU is SRAP PDU. If the intention was to cover both of SRAP Control PDU and SRAP Data PDU, it would be better to say SRAP PDU. |
| ZTE | 2 | Fine with Nokia’s changes. |
| vivo | 2 | Alignment is preferable. |
| Sharp | 2 |  |
| Futurewei | Neither at this point. | We are not sure what “reserved values” are referring to here. If they are referring to the “R” bits in the SRAP header, TS 38.351 has already described how to treat the “R” bit, i.e., “Reserved bits shall be ignored by the receiver.” We also suggest changing “SRAP Control PDU” to “SRAP Control PDU (for this release)”, because we have defined the SRAP Control PDU format, except that it is not used in this release. So, this error handling applies to this release (A future release may remove it if the use of SRAP Control PDU is introduced). |
| Xiaomi | 2 |  |
| - Samsung | - 2 but with changes | - This is how this issue is handled in the BAP spec: “when a BAP Control PDU that contains reserved or invalid values is received the BAP entity shall: - discard the received BAP PDU.” - We feel we should also stick to SRAP Control PDU rather than include SRAP Data PDU. It is not clear for a Data PDU what invalid values would cover anyway. This is why we have the preceding two paragraph, which deal with invalid data for the Data PDU case. Here we can focus on Control PDU case. |
| ASUSTeK | 2 | We prefer the text proposal as suggested by Nokia. |
| CATT | 2 with changes | Nokia’s change is better. |
| - Huawei, HiSilicon | - Both are ok to us | - As proponent, we think the original wording is clear, because the change is based on the existing wording in 38.351, i.e. “in this release D/C value shall not be set to 1 as no SRAP Control PDU is defined, and R bit shall not be set to 1.”. and the intention of “invalid value” is to cover R bit as well. But if companies want to mention reserved bits explicitly, it should be fine. Nokia’s version is ok to us as well. |
| - Apple | - No strong view |  |
| - MediaTek | - 2 |  |
| - LG | - No strong view |  |
| - InterDigital | - 2 |  |

[Rapp summary]

15 companies replied,14 companies are fine with Option-2, serval companies express the preference on Nokia’s rewording, the rewording suggests use “SRAP PDU” to include both data PDU and control PDU which is correct and future proof from Rapp understanding, so Rapp suggest we go for Nokia’s rewording for this change.

- Recommendation3	For change of R2-2301176, it is agreed as “When the U2N Remote UE or the U2N Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall: -discard the received SRAP PDU.”.

## Change 1 of R2-2301351

The intention of change in R2-2301176 has been agreed, and the detailed wording is to be checked

Proposal 2	RAN2 to agree on the intention of R2-2301123 (change-3), R2-2301176, R2-2301351 (change-1), and wording is to be checked in CR discussion.

- Changes listed in P2 are agreed in intention, with wording to be finalised in discussion [408].

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE: - - For UL data packet not corresponding to SRB0 (i.e., received from SL-RLC0 as specified in TS 38.331 [3]), the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface. <omit>

The following question is to check companies view on change 1 of R2-2301351.

Q4: Do you agree with change 1 in R2-2301351(with changing draftCR to CR)?

| Company | Agree/Disagree | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| Nokia | Agree (Proponent) |  |
| ZTE | comments | the sentence is not only for UL data packet, but also include DL data packet delivery. The first half of sentence is about DL while the latter part is for UL. |
| vivo | Comments | In “For UL data packet”, “UL” needs to be removed since the description of following sentence includes both UL and DL. |
| Futurewei | Yes with comment | Since the expression style has changed, it is unclear whether “i.e., …” applies to “corresponding to SRB0” or “not corresponding to SRB0”. Suggest the following to make it clearer: “For UL data packet not corresponding to SRB0 (i.e., packet not received from SL-RLC0 as specified in TS 38.331 [3]) …” |
| Xiaomi | Comments | Remove UL and agree with Futurewei’s change. |
| Samsung | See comments | Agree with ZTE – the meaning appears to change if this change is agreed. On reflection we prefer the original wording, which is not incorrect since SRB0 only refers to the UL case (operation of receiving part of SRAP entity on PC5). Or perhaps the following clarification could wok: The receiving part on the SRAP entity of Uu interface delivers DL SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface. The receiving part on the SRAP entity of PC5 interface delivers UL SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface, except for UL data packet for SRB0 (i.e., received from SL-RLC0 as specified in TS 38.331 [3]). |
| CATT | Comments | Same view as Xiaomi |
| Apple |  | Agree with ZTE. |
| MediaTek |  | Agree with ZTE. |
| LG |  | Agree with ZTE. |
| InterDigital |  | Agree with Futurewei |

[Rapp summary]

11 companies replied, 10 companies express the concerns on the “the sentence is not only for UL data packet, but also include DL data packet delivery.” Which is valid from Rapp understanding, so it is suggested that change 1 of R2-2301351 is not agreed this time.

- Recommendation4	For change 1 of R2-2301351, it is not pursued.

## Change of R2-2301329

The intention of change in R2-2301329 has been agreed, and the detailed wording is to be checked

- R2-2301329 is agreed in intention, wording to be checked in discussion [408].

- 5.2.2.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received): - - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3]; - else if there is an entry in sl-SRAP-ConfigRelay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received): - - Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

During the online session, an error in the cover page has been identified: in the summary of change, the change is for 38.351 not 38.331, and this cover page should be updated.

The following question is to check companies view on change of R2-2301329.

Q5: Do you agree with change in R2-2301329 with cover page updated?

| Company | Agree/Disagree | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| Nokia | Agree |  |
| ZTE | yes |  |
| Sharp | Agree |  |
| Xiaomi | Agree |  |
| Samsung | Agree |  |
| ASUSTeK | Agree |  |
| CATT | Agree |  |
| Apple | Agree |  |
| MediaTek | Agree |  |
| LG | Agree |  |
| InterDigital | Agree |  |

[Rapp summary]

11 companies replied, all companies agree with the change.

- Recommendation5	For change of R2-2301329, it is agreed with cover page updated.

## Change of R2-2301527/R2-2301548

The intention of change in R2-2301527/R2-2301548 have been agreed, and the detailed wording is to be checked

- R2-2301527/R2-2301548 are agreed in intention, wording to be checked in discussion [408].

Solution in R2-2301527

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall: - if the transmitting PDCP entity is associated with one RLC entity: - submit the PDCP PDU to the associated RLC entity; - else, if the transmitting PDCP entity is associated with at least two RLC entities: - if the PDCP duplication is activated for the RB: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication; - else: - submit the PDCP Control PDU to the primary RLC entity; - else (i.e. the PDCP duplication is deactivated for the RB or the RB is a DAPS bearer): - if the split secondary RLC entity is configured; and - if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold: - submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity; - else, if the transmitting PDCP entity is associated with the DAPS bearer: - if the uplink data switching has not been requested: - submit the PDCP PDU to the RLC entity associated with the source cell; - else: - if the PDCP PDU is a PDCP Data PDU: - submit the PDCP Data PDU to the RLC entity associated with the target cell; - else: - if the PDCP Control PDU is associated with source cell: - submit the PDCP Control PDU to the RLC entity associated with the source cell; - else: - submit the PDCP Control PDU to the RLC entity associated with the target cell; - else: - submit the PDCP PDU to the primary RLC entity; NOTE 2: If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity. - else, if the transmitting PDCP entity is associated with the SRAP entity: - submit the PDCP PDU to the SRAP entity.

Solution in R2-2301548

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall: - if the transmitting PDCP entity is associated with one RLC entity: - if the transmitting PDCP entity is associated with an SRAP entity: - submit the PDCP PDU to the associated SRAP entity; - else: - submit the PDCP PDU to the associated RLC entity;

New merged-solution from the source companies (AsusTek and Nokia)

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall: - if the transmitting PDCP entity is associated with an SRAP entity: - submit the PDCP PDU to the associated SRAP entity; - else, if the transmitting PDCP entity is associated with one RLC entity: - submit the PDCP PDU to the associated RLC entity; - else, if the transmitting PDCP entity is associated with at least two RLC entities: - if the PDCP duplication is activated for the RB: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication; - else: - submit the PDCP Control PDU to the primary RLC entity;

For the solution in R2-2301527, the PDCP PDU is submitted to the SRAP entity if the PDCP is not associated to RLC entity and it is associated to an SRAP entity;

For the solution in R2-2301548, the PDCP PDU is submitted to SRAP entity if the PDCP is associated to a single RLC entity and if the PDCP is associated to an SRAP entity;

For the new-merged solution, the PDCP PDU is submitted to SRAP entity if the PDCP is associated to an SRAP entity.

It is suggested RAN2 to discuss the following options for this issue:

Option1: Solution from R2-2301527;

Option2: Solution from R2-2301548;

Option3: The merged solution from the source companies;

The following question is to check companies’ view on this issue

Q6: Which do you prefer on the solution for this PDCP-SRAP mapping?

Option1: Solution from R2-2301527;

Option2: Solution from R2-2301548;

Option3: The merged solution from the source companies;

| Company | Option1/2/3 | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| OPPO | 3 | Thanks for the coordination effort by proponents. |
| Nokia | 3 | As an RB with SRAP is also with RLC below SRAP, it may be confusing whether this RB is also considered to be associated with RLC. |
| ZTE | 3 |  |
| vivo | 3 |  |
| Sharp | 3 |  |
| Futurewei | 1 (with similar wording as in 3, such as “an” and “associated”) or 3 | We think Options 1 and 3 are essentially the same (except that 1 can adopt the similar wording as in 3). They both can be easily extended to cover MP relaying later. But extending option 2 to cover MP relaying wound not be as clean as options 1/3. |
| Xiaomi | 3 | Option 3 can be more forward compatible to cover multipath. |
| Samsung | 3 |  |
| ASUSTeK | 3 |  |
| CATT | 3 |  |
| Huawei, HiSilicon | 2 | We slightly prefer option#2, which only changes the case with one RLC leg. The reason is that we will introduce MP in Rel-18, which has two legs, with one associated to SRAP, then the current change in 3 needs to be split into one RLC case and two RLC case anyway. |
| MediaTek | 3 |  |
| LG | 3 |  |
| InterDigital | 3 |  |

[Rapp summary]

14 companies replied, 13 companies prefer Option-3, 1 companie prefers Option-2, so Rapp suggest we go for majority view on this issue.

- Recommendation6	For change of R2-2301527/R2-2301548, agree the change as ‘if the transmitting PDCP entity is associated with an SRAP entity: submit the PDCP PDU to the associated SRAP entity; else…’.

## Change of R2-2301483

The intention of change in R2-2301483 has been agreed, and the detailed wording is to be checked

- R2-2301483 is agreed in intention, wording to be checked in discussion [408].

NOTE: If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair. NOTE Y: If this TB is associated to broadcast and the associated LCID is equal to 58, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

During the online session, some suggestions on the rewording have been given, i.e., to merge the discovery message filtering with the existing NOTE. So the following rewording on this change is suggested

NOTE: If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, or if this TB is associated to broadcast and the associated LCID is equal to 58, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair.

Q7: Which do you prefer?

Option1: Solution from R2-2301483;

Option2: the reworded version as above;

| Company | Option1/2 | Comments (if disagree with the current wording, please provide the suggested rewording) |
| --- | --- | --- |
| OPPO | 2 | The spec should be as brief as possible. |
| Nokia | 2 | It should be in normative text. The rewarded text is clearer and sufficient. |
| ZTE |  | Follow the majority. |
| vivo | 2 |  |
| Sharp | 2 |  |
| Futurewei | Not 2 as is | Option 2, as is, could be mis-interpreted as the following: “If A, or if B and C”, C being “the DST field ...”, while the intention is “If A or B, and if C” [Rapp]: thanks for the comments, would the following rewording clearer(also add ‘if’ for C)? NOTE: If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, or if this TB is associated to broadcast and the associated LCID is equal to 58, and if the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair. |
| Xiaomi | 2 |  |
| Samsung | 2 | Samsung |
| ASUSTeK | 2 |  |
| CATT | 2 |  |
| Huawei, HiSilicon | 1, but can follow majority | As proponent, we think to have a separate NOTE should be clearer, because the existing one is for unicast, but now we are discussing broadcast. But if companies prefer to merge it into the existing NOTE, we can accept. |
| Apple | 1 | We think a separate note is required. Because the LCID ==58 is only for discovery case, and as L2 ID(s) for discovery is separated from non-discovery case, so we think the note can be improved by adding “NR sidleink discovery” as below: NOTE Y: If this TB is associated to broadcast and the associated LCID is equal to 58, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) for NR Sidelink discovery of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. [Rapp]: thanks for the comments, is the concern here same with Futurewei, if it is, please see the reply above. |
| MediaTek | 2 |  |
| LG | 2 |  |
| InterDigital | 2 |  |

[Rapp summary]

15 companies replied, 11 companies prefer Option-2 (merge the 2 NOTEs), 2 companies (with Huawei can follow majority view) prefer Option-1 (use 2 split NOTEs), 1 company follow majority view, and 1 company propose the rewording suggestion for Option-2. So Rapp suggest we go for majority view on the merged-NOTE way and further rewording the NOTE to make it clear.

NOTE:	If this TB is associated to unicast and this TB is the first TB of a logical channel which associated LCID is equal to 0 or 1, or if this TB is associated to broadcast and the associated LCID is equal to 58, and if the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, deliver the decoded MAC PDU to the disassembly and demultiplexing entity. Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair.

- Recommendation7	For Change of R2-2301483, agree the CR as adding “or if this TB is associated to broadcast and the associated LCID is equal to 58, and if” in the existing NOTE.

# Conclusion

We have the following proposals:

- Recommendation1	For change 1 of R2-2301123, it is agreed as “Mapping from a radio bearer identified by UE ID field and BEARER ID field to…”.

- Recommendation2	For change 3 of R2-2301123, it is agreed as “and for DRB, the DRB identity is BEARER ID plus 1”.

- Recommendation3	For change of R2-2301176, it is agreed as “When the U2N Remote UE or the U2N Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall: -discard the received SRAP PDU.”.

- Recommendation4	For change 1 of R2-2301351, it is not pursued.

- Recommendation5	For change of R2-2301329, it is agreed with cover page updated.

- Recommendation6	For change of R2-2301527/R2-2301548, agree the change as the PDCP PDU is submitted to SRAP entity if the PDCP is associated to an SRAP entity without checking on PDCP-RLC association.

- Recommendation7	For Change of R2-2301483, agree the CR as adding “or if this TB is associated to broadcast and the associated LCID is equal to 58, and if” in the existing NOTE.

# Reference

- R2-2301123	Corrections on SRAP for SL relay	ZTE Corporation, Sanechips	CR	Rel-17	38.351

- R2-2301176	Correction to error handling in SRAP	Huawei, HiSilicon	CR	Rel-17	38.351

- R2-2301329	Correction on SRAP for L2 U2N Relay	Philips International B.V.	CR	Rel-17	38.351

- R2-2301351	SRAP layer corrections	Nokia, Nokia Shanghai Bell	draftCR	Rel-17	38.351

- R2-2301483	Correction on discovery message filtering	Huawei, HiSilicon	CR	Rel-17	38.321

- R2-2301527	Clarification on PDCP for L2 U2N Relay	ASUSTeK	CR	Rel-17	38.323

- R2-2301548	PDCP PDU delivery to SRAP for sidelink relay	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.323