3GPP TSG-RAN WG2 Meeting #121						  R2-2301854

Athens, Greece, February 27 – March 3, 2023

Agenda item:	8.3.2

Source:			MediaTek Inc.

Title:	Further discussion on Cell DTX/DRX

Document for:	Discussion and Decision

- Introduction

By the following objective of the NES WID [1]:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

Considering the total TU allocated in RAN2 for Rel-18 NES is 6, and Cell DTX/DRX is just a topic of time domain technique, we think RAN2 needs to do further down-scoping focusing on the simpler and/or efficient solution. This paper is to discuss the further considerations on the details of Cell DTX/DRX technique.

- Discussion

  - DTX/DRX configuration and activation control

During SI phase, RAN2 had agreed that Cell DTX and Cell DRX modes can be configured per serving cell and operated separately, or simultaneously [2]. Therefore, it is natural to reuse the existing SI mechanism to deliver the cell-wise configurations of Cell DTX/DRX in a grouping-like way. (We think the ODSI is one of UE-specific RRC signaling.)

- RAN2 to agree adding Rel-18 NES SIB-X and the System Information procedures are used to broadcast and update the Cell DTX and/or Cell DRX configurations.

So does the activation and deactivation control.

- RAN2 to agree using DCI to activate or deactivate the Cell DTX and/or Cell DRX. The grouping mechanism is left to RAN1.

  - Alignment considerations

No matter Cell DTX runs with or without the legacy UEs, the baseline shall be the network-based solution: the network configures Cell DTX so that its active duration can cover the Active Time of each UE C-DRX. If there is a need to align each C-DRX pattern among multiple UEs, it is recommended to align UE C-DRX offset [4] in a cell-specific way. We believe that is the underlying fundament of having a regular opportunity of Cell DTX.

Figure 2-1: C-DRX start offset alignment among multiple UEs

- RAN2 to agree a NES-specific start offset is used in aligning C-DRX pattern among the UEs.

In addition to the existing RRCReconfiguration procedure reusing for the UEs, it is beneficial to have a more efficient way in delivering the offset to the NES-capable UEs.

- RAN2 to agree including the NES-specific C-DRX start offset in the Rel-18 NES SIB-X.

  - Active time adaptation (impact on UE behavior)

The presence of the legacy UEs is a key factor in affecting higher layer procedures and the NES gain of Cell DTX/DRX. Whenever uplink data is available in the transmission buffer, the legacy UE can send SR irrespective of sleep state so that the Active Time is prolonged until inactivity timer expired. In this regard, the opportunity of Cell DTX is still an uncertain period even the Cell DTX and UE C-DRX are aligned already, and Cell DRX shall never be activated to avoid uplink transmission failure. Therefore, we have different considerations (not an exhaustive list) for two scenarios:

Scenario 1: Cell DTX with the legacy UEs

Consideration 1.1: Cell DRX shall not be activated simultaneously.

Consideration 1.2: Active time of Cell DTX shall be extended in the same way as behaving in C-DRX.

Consideration 1.3: No grouping mechanism to align Cell DTX and the C-DRX pattern of the legacy UEs. (preparation time to enter Cell DTX may be longer somehow)

Consideration 1.4: Suboptimal NES gain.

Scenario 2: Cell DTX without the legacy UEs

Consideration 2.1: Cell DRX can be activated simultaneously.

Consideration 2.2: Active time of Cell DTX could be semi-static or even static if no harm to UE QoS is guaranteed.

Consideration 2.3: Having grouping mechanism to align Cell DTX and UE C-DRX pattern, and to control the activation/deactivation.

Consideration 2.4: Optimal NES gain but poor service coverage for the legacy UEs.

Figure 2-2: Cell DTX with the legacy UEs

- RAN2 to agree following Working Assumptions:
WA1 (for Cell DTX with legacy UEs): Cell DTX/DRX shall be backward compatible with the C-DRX mechanism.
WA2 (for Cell DTX without the legacy UEs): Cell DTX/DRX could have further UE behavior restriction on top of the C-DRX mechanism.

Focusing on a single UE behavior at any point in time, for WA1, we propose:

- For Cell DTX with the legacy UEs, whenever PDCCH indicates a new transmission for either downlink or uplink, the active time of Cell DTX is extended by “drx-InactivityTime”.

For WA2, we propose the baseline is to allow data retransmission to minimize the impact.

- For Cell DTX without the legacy UEs, only the retransmission is allowed during the inactive time of Cell DTX (new data transmission, e.g., SR, is prohibited, or delayed to the next occasion).

  - Detailed configuration parameters

As per illustration in the Figure 2-2, the gNB Tx active time is restricted by Cell DTX so that the scheduling load spreading by the start offset is suppressed. There is a trade-off between UE capacity and scheduling load during active time of Cell DTX so that the range and value candidate of ON duration needs further study and shall be determined by RAN1 decision. Then the periodicity can be studied and determined accordingly.

- There is a trade-off between UE capacity and scheduling load during active time of Cell DTX.

- RAN2 waits for RAN1 decision to start ASN.1 level working on detailed configuration parameters.

  - Overall procedure

Our understanding and proposals so far are conceptually captured in the Figure 2-3. The yellow-highlighted parts are newly introduced in Rel-18 NES. RAN2 can discuss the overall procedure for Cell DTX/DRX to be captured in the stage 2 specification.

Figure 2-3: Overall procedure of Cell DTX/DRX

- RAN2 to discuss the overall procedure of Cell DTX/DRX to be captured in the stage 2 specification.

- Summary

In previous sections we made the following observations and proposals:

Proposal 1	RAN2 to agree adding Rel-18 NES SIB-X and the System Information procedures are used to broadcast and update the Cell DTX and/or Cell DRX configurations.

Proposal 2	RAN2 to agree using DCI to activate or deactivate the Cell DTX and/or Cell DRX. The grouping mechanism is left to RAN1.

Proposal 3	RAN2 to agree a NES-specific start offset is used in aligning C-DRX pattern among the UEs.

Proposal 4	RAN2 to agree including the NES-specific C-DRX start offset in the Rel-18 NES SIB-X.

Proposal 5	RAN2 to agree following Working Assumptions:
WA1 (for Cell DTX with legacy UEs): Cell DTX/DRX shall be backward compatible with the C-DRX mechanism.
WA2 (for Cell DTX without the legacy UEs): Cell DTX/DRX could have further UE behavior restriction on top of the C-DRX mechanism.

Proposal 6	For Cell DTX with the legacy UEs, whenever PDCCH indicates a new transmission for either downlink or uplink, the active time of Cell DTX is extended by “drx-InactivityTime”.

Proposal 7	For Cell DTX without the legacy UEs, only the retransmission is allowed during the inactive time of Cell DTX (new data transmission, e.g., SR, is prohibited, or delayed to the next occasion).

Observation 1	There is a trade-off between UE capacity and scheduling load during active time of Cell DTX.

Proposal 8	RAN2 waits for RAN1 decision to start ASN.1 level working on detailed configuration parameters.

Proposal 9	RAN2 to discuss the overall procedure of Cell DTX/DRX to be captured in the stage 2 specification.

- References

- RP-223540		New WID: Network energy savings for NR

- R2-2213352	Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

- TR 38.864 V18.0.0 (2022-12)

- R2-2212840	Recommendations for DTX DRX mechanism