3GPP TSG-RAN WG2 Meeting #122	R2-2304729

Incheon, South Korea, May 22 - May 26, 2023 	Revision of R2-2303202

Agenda Item:	7.11.3

Source:	MediaTek Inc.

Title:	Discuss on shared processing for broadcast and unicast reception

Document for:	Discussion, Decision

Introduction

In RAN2#121 meeting, several agreements were made on UE capability and assistance information for shared processing for MBS broadcast and unicast reception. In this contribution, we further discuss the granularity for UE capability and other additional information for the coexistence of broadcast and unicast reception.

Discussion

Granularity of non-serving cell broadcast reception capability

In RAN2#121, RAN2 agreed to introduce a capability for MBS broadcast reception from a non-serving cell

- Indicate the capability of receiving MBS broadcast from a non-serving cell. FFS whether the granularity is at FeatureSetDownlink or FeatureSetDownlinkPerCC level.

UE with shared processing capabilities can indicate which specific component carrier supports multiple timing advance capabilities for CA, i.e. the capability for non-serving cell broadcast reception. This finer granularity enables the UE to provide more precise information, allowing the serving gNB to configure new component carriers that support simultaneous unicast and broadcast reception. Therefore, we propose indicating the granularity of non-serving cell broadcast reception capability per FeatureSetDownlinkPerCC.

Proposal 1: The granularity of the capability for non-serving cell broadcast reception is FeatureSetDownlinkPerCC

Additional information for MII

In case unicast and broadcast services are provided from different operators, the number of component carriers (CCs) supporting unicast reception may be unclear to the serving gNB. Even if the UE reports its capability to receive broadcasts from non-serving cells, the serving gNB remains unaware of other cells' broadcast configurations, and therefore does not know the UE's remaining capabilities. This lack of information could affect legacy unicast or multicast reception from the Pcell.

One typical scenario is that assuming UE has total N component carriers (CCs) capability and all CCs are used for unicast reception at first. Then, twos CC are used for MBS broadcast reception from non-serving cell. The Pcell may not know the capability change that the CCs can be used for unicast transmission is changed to N-2, and this may lead to the conflict between broadcast and unicast service.  If such capability changes could be reported to the Pcell, the network could release the CC capacity used for broadcast reception and adjust the CCs used for unicast reception accordingly.

In last meetings, several information (i.e., frequency, SCS, bandwidth) were agreed for MII signalling as assistance information. To also consider the NR characteristics, we propose to also includes the number of component carriers used for broadcast reception in non-serving cell to avoid UE capability exceeding its limits.

Proposal 2: The number of component carriers used for broadcast reception in non-serving cell can be signalled in MBSInterestIndication.

For the broadcast reception via Scell, the SIB20 is indicated to UE by dedicated signalling (which needs extra interaction between different cells). To facilitate broadcast reception from non-serving cell, it is proposed to have the similar procedure as Scell. Since this may require extra interaction between cells from different operators, the transfer of SIB20 from non-serving cell can be up to network implementation.

Proposal 3: If supported by the network, the SIB20 of non-serving cell can be delivered to UE by dedicated signalling.

UE limitations on maximum data rate could impact legacy reception (unicast or multicast) in the Pcell when a UE is receiving broadcast services from scell/non-serving cells. Therefore, MII may also encompass configurations and resources used by the UE for broadcast reception, including the common frequency resource (CFR) configuration, MIMO layer, modulation order, etc. Further details can be checked with RAN1 if needed.

Proposal 4: RAN2 to discuss whether to report the following configurations in MII for maximum data rate calculation. Further details can be checked with RAN1 if needed.

- CFR configuration

- MIMO layer

- Modulation order

- Supported band combination

Scenarios for enhanced MII

As we agreed, the enhanced MII at least reports the broadcast frequency, subcarrier spacing, and bandwidth for broadcast services. It may be further enhanced if additional information is needed. For intra-PLMN cases, the information is already known by the network (as long as the MII is received). Therefore, the enhanced MII is not necessary.

Proposal 5: The enhanced MII is not needed for the intra-PLMN case.

For inter-PLMN cases, whether extra information is needed depends on whether serving gNB knows the configuration for the reported broadcast service from another operator. In the last meeting, RAN2 also discuss whether network can control the MII type by adding some information in SIB1. Meanwhile, UE can also report the enhanced MII in advance. Therefore, the following three options for enhanced MII reporting can be discussed together:

Option 1: For inter-PLMN broadcast reception, UE send enhanced MII to the network in the beginning.

Option 2: UE reports legacy MII (Rel-17 MII) to network in the beginning. If network think it is necessary, it indicates UE to send enhanced MII. Then UE reports the enhanced MII to network.

Option 3: A new IE is introduced in SIB1 to control whether UE send legacy MII or enhanced MII in the beginning.

Compared with option1, option2 reduces signalling overhead with the consequence of more signalling interaction and latency. We think the drawbacks outweigh the benefit. Option 3 may be a viable way to regulate UE's reporting of MII, but it may be complex as the network may have various information related to different broadcast services from other PLMNs. Thus, the IE which indicates MII type may need to be distinct for each broadcast service, even from other operators.

Based on the above discussion, we proposed:

Proposal 6: For UE receiving broadcast from non-serving cell, the enhanced MII is reported in the beginning. (i.e., no IE is introduced to control whether enhanced MII or legacy MII is reported)

Conclusion

In this contribution, the following proposals are made:

Proposal 1: The granularity of the capability for non-serving cell broadcast reception is FeatureSetDownlinkPerCC

Proposal 2: The number of component carriers used for broadcast reception in non-serving cell can be signalled in MBSInterestIndication.

Proposal 3: If supported by the network, the SIB20 of non-serving cell can be delivered to UE by dedicated signalling.

Proposal 4: RAN2 to discuss whether to report the following configurations in MII for maximum data rate calculation. Further details can be checked with RAN1 if needed.

- CFR configuration

- MIMO layer

- Modulation order

- Supported band combination

Proposal 5: The enhanced MII is not needed for the intra-PLMN case.

Proposal 6: For UE receiving broadcast from non-serving cell, the enhanced MII is reported in the beginning. (i.e., no IE is introduced to control whether enhanced MII or legacy MII is reported)

References

R2_121 MBS session notes (Dawid) 2023-03-03 EOM_rm

R2_119bis-e_MBS session notes (Dawid)_2022-10-19 EOM_rm