3GPP TSG-RAN WG2 #122 	R2-2304825

Incheon, Korea, 22nd – 26th May, 2023

Title:		Discussion on NCR-MT capability

Source:	Huawei, HiSilicon

Agenda item:	7.1.3

Document for:	Discussion

- Introduction

During the last RAN2 meeting, following agreement was achieved for the NCR-Fwd access beam management when the NCR-MT is released to the INACTIVE state [1]:

- When NCR-MT is released to RRC_INACTIVE state (NCR-Fwd is forwarding), the periodic beam indication configuration (if configured and not removed) shall be applied.

This agreement was based on another agreement that NCR-Fwd can continue working when NCR-MT is released to RRC_INACTIVE.

In this paper, we will discuss whether to introduce a capability for this.

- Discussion

In RAN2#121 meeting, RAN2 agreed:

- After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB.

At that time, RAN2 didn’t consider the beam management issue for backhaul link, and assumed that there was no additional requirement for NCR-MT in RRC_INACTIVE, compared to NCR-MT in RRC_CONNECTED.

However, according the discussion in [4], different from NCR-MT in RRC_CONNECTED where the backhaul link beam is indicated by MAC CE, when NCR-MT in RRC_INACTIVE, there would be new backhaul link beam management procedure to make NCR-Fwd running, i.e. the backhaul link beam may change following the best C-link beam.

Observation 1: When RAN2 agreed that NCR-Fwd can be ON after NCR-MT enters RRC_INACTIVE, RAN2 didn’t assume that there would be additional requirement to NCR implementation compared to NCR-MT in RRC_CONNECTED.

Observation 2: When NCR-MT is in RRC_INACTIVE, there will be additional requirement to NCR in terms of backhaul beam management.

In this case, RAN2 may consider to introduce two different implementations for NCR devices in:

- Implementation 1: The NCR-Fwd can be running when NCR-MT is in RRC_INACTIVE, i.e. NCR needs to be able to support backhaul beam management when NCR-MT is in RRC_INACTIVE (if a solution is agreed for this).

- Implementation 2: The NCR-Fwd is OFF when NCR-MT is in RRC_INACTIVE, NCR doesn’t need to support backhaul beam management when NCR-MT is in RRC_INACTIVE.

Of course, if RAN2 agrees to revert the agreement and agree to not support NCR-Fwd working when NCR-MT is in RRC_INACTIVE, there would be only Implementation 2 possible. Otherwise, we think both the implementations can be allowed.

Proposal 1: RAN2 is requested to discuss the support of two different NCR implementations:

- Implementation 1: The NCR-Fwd can be running when NCR-MT is in RRC_INACTIVE, i.e. NCR needs to be able to support backhaul beam management when NCR-MT is in RRC_INACTIVE (if a solution is agreed for this).

- Implementation 2: The NCR-Fwd is OFF when NCR-MT is in RRC_INACTIVE, NCR doesn’t need to support backhaul beam management when NCR-MT is in RRC_INACTIVE.

The UE capability signalling can be introduced for the gNB to be aware of the NCR implemented capability.

Proposal 2: In case NCR-Fwd can continue work when NCR-MT is in RRC_INACTIVE, RAN2 to introduce a new capability of NCR-MT for this.

3.	Conclusion

This contribution mainly discusses NCR capability to support NCR-Fwd running when NCR-MT in RRC_INACTIVE. The following observations and proposals are provided:

Observation 1: When RAN2 agreed that NCR-Fwd can be ON after NCR-MT enters RRC_INACTIVE, RAN2 didn’t assume that there would be additional requirement to NCR implementation compared to NCR-MT in RRC_CONNECTED.

Observation 2: When NCR-MT is in RRC_INACTIVE, there will be additional requirement to NCR in terms of backhaul beam management.

Proposal 1: RAN2 is requested to discuss the support of two different NCR implementations:

- Implementation 1: The NCR-Fwd can be running when NCR-MT is in RRC_INACTIVE, i.e. NCR needs to be able to support backhaul beam management when NCR-MT is in RRC_INACTIVE (if a solution is agreed for this).

- Implementation 2: The NCR-Fwd is OFF when NCR-MT is in RRC_INACTIVE, NCR doesn’t need to support backhaul beam management when NCR-MT is in RRC_INACTIVE.

Proposal 2: In case NCR-Fwd can continue work when NCR-MT is in RRC_INACTIVE, RAN2 to introduce a new capability of NCR-MT for this.

4.	References

- Chairs Notes, RAN2-121bis-NCR-(Sasha)-2023-04-26-1345, Online.

- Chairs Notes, RAN1 #112, Athens, Greece.

- Chairs Notes, RAN1 #111, Toulouse, France.

- R2-2304824, Discussion on the remaining CP issues for NCR, Huawei, HiSilicon.