3GPP TSG RAN WG2 #122 	                                                                     R2-2304901

Incheon, KR, 22nd – 26th May, 2023

Source:	CATT, CEPRI

Title:	Discussion on enhanced eDRX in RRC_INACTIVE

Agenda Item:	7.19.2

Document for:	Discussion and Decision

Introduction

One objective of the WID Enhanced support of reduced capability NR devices is [1]:

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement

Some agreements have been achieved and some open issues are left, in last meeting. In this contribution, we give some discussion on the open issues left in last meeting.

Discussion

For the open issue left based on the discussion of last meeting:

UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_IDLE eDRX. TBD if it must also support Rel-17 RRC_INACTIVE eDRX. A cell can allow Rel-18 INACTIVE eDRX, only if eDRX-AllowedIdle is configured. TBD if it must also configure Rel-17 RRC_INACTIVE eDRX.

Rel-17 RRC_INACTIVE eDRX is an optional feature for any UE (including RedCap and non-RedCap UEs), and the following bit is introduced to indicate whether the UE support Rel-17 RRC_INACTIVE eDRX:

| extendedDRX-CycleInactive-r17 Indicates whether UE supports the extended DRX in RRC_INACTIVE with values of 256, 512 and 1024 radio frames as specified in TS 38.331 [9]. The UE may indicate support for extended DRX in RRC_INACTIVE only if it supports extended DRX in RRC_IDLE. | UE | No | No | No |
| --- | --- | --- | --- | --- |

And in last meeting, the following agreement was achieved:

Introduce an optional UE capability with signalling for Rel-18 enhanced eDRX in RRC_INACTIVE.

That is, for Rel-18 enhanced eDRX in RRC_INACTIVE, a separate capability bit will also be introduced. So it is possible that, the UE supports only one of or both of Rel-17 RRC_INACTIVE eDRX and Rel-18 enhanced eDRX in RRC_INACTIVE.

But we think that:

- R18 INACTIVE eDRX is an enhancement of R17 INACTIVE eDRX, it is a little strange that an UE has the power saving requirement based on Rel-18 enhanced eDRX in RRC_INACTIVE, but has no power saving requirement based on Rel-17 RRC_INAVTIVE eDRX.

- If both R18 INACTIVE eDRX and R17 INACTIVE eDRX are supported by UE, the UE can fall back to R17 INACTIVE eDRX if the UE moves into a cell not supporting R18 INACTIVE eDRX, but supporting R17 INACTIVE eDRX, which is beneficial for the UE power saving.

- Supporting both R18 INACTIVE eDRX and R17 INACTIVE eDRX has very little impact on the UE cost and/or complexity, compared to supporting only R18 INACTIVE eDRX

- If UE supporting Rel-18 enhanced eDRX must also support Rel-17 RRC_INACTIVE eDRX, the UE can only report Rel-18 enhanced eDRX capability, Rel-17 RRC_INACTIVE eDRX capability bit can be saved.

So we think that, UE supporting Rel-18 enhanced eDRX must also support Rel-17 RRC_INACTIVE eDRX.

However, for the second open issue of cell indication, it is more about network deployment and/or network upgradation. For example, a cell not supporting Rel-17 RRC_INACTIVE eDRX can be upgraded to support Rel-18 enhanced eDRX directly. So it is not necessary that a cell supporting Rel-18 INACTIVE eDRX must also support Rel-17 RRC_INACTIVE eDRX.

Based on the analysis, we have the following proposal:

Proposal 1：UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_INACTIVE eDRX.

Proposal 2：A cell can allow Rel-18 INACTIVE eDRX, without having to configure Rel-17 RRC_INACTIVE eDRX.

For the following working assumption:

Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

We think it is a valid scenario that: an UE is configured with both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX, and then it moves into a cell not supporting Rel-18 enhanced INACTIVE eDRX. From the network point of view, the network should have the flexibility to configure both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX to UE if the UE indicates that it supports both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX, and this has little network complexity impact. We also think the network and the UE should have same and clear understanding on the UE behavior in the given scenario above, to avoid paging transmission waste or erroneous judgement of the UE state. It makes sense that the UE falls back to use Rel-17 INACTIVE eDRX in the given scenario, from the view of UE power saving requirement.

Proposal 3：Confirm the following working assumption:

UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

Conclusion

In this contribution, we give some discussion on the objective of enhanced eDRX in RRC_INACTIVE(>10.24s), based on the agreements and open issues in last meeting. The following proposals are given:

Proposal 1：UE can support Rel-18 enhanced eDRX, only if it supports Rel-17 RRC_INACTIVE eDRX.

Proposal 2：A cell can allow Rel-18 INACTIVE eDRX, without having to configure Rel-17 RRC_INACTIVE eDRX.

Proposal 3：Confirm the following working assumption:

UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

Reference

- RP-223544 Revised WID on Enhanced support of reduced capability NR devices