3GPP TSG-RAN WG2 Meeting #122                               R2-2304968

Incheon, KR, May 22 – 26, 2023

Source:        CATT

Title:	       Enhancements for SFN wrap-around

Agenda Item:	7.5.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will discuss the pros and cons of the various options for SFN wrap-around issue.

- Discussion

The current DRX formulas to start drx-onDurationTimer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle: [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle).

Hence, DRX on-duration start time is based on SFN and subframe number. The range of SFN is 0-1023 while the range of subframe number is 0-9. Hence, [(SFN × 10) + subframe number] is repeated every 10,240ms, i.e. every time SFN=0 again. However, the periodicities of XR traffic are not factors of 10240ms. With the above formulas, the time when the SFN value wraps around does not match the traffic arrival time.

According to contributions in last RAN2#121bis-e meeting, possible solutions to address the SFN wrap around issue are:

Alt. 1 E-SFN (extended SFN) is introduced in the C-DRX formulas.

E-SFN can be considered as an SFN system information. It is broadcast over the network, and incremented when legacy SFN wraps around. The maximum value, i.e., the wraparound for E-SFN, can be selected by the network such that, at the E-SFN wraparound, the DRX cycles will not overlap. The network can indicate the maximum value of E-SFN to the UE by RRC signalling (e.g., dedicated or broadcast).

Because the E-SFN is broadcast by the network, the UE can always read the E-SFN value from the system information and ensure to be in sync with the network with respect to the DRX on-durations.

At the same time, because of the following formula, DRX cycles can support non-integer cycles, i.e. rational numbers:

A modulo B = A – [ B × floor (A / B) ]					(Eq1)

With the use of E-SFN, the C-DRX formula can be adapted to support non-integer DRX cycles (using the floor function) as below:

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset			(Eq2)

where n = [(E-SFN × 10240) + (SFN × 10) + subframe number]

floor [n – [drx-ShortCycle × floor (n / drx-ShortCycle) ] ] =

floor [drx-StartOffset – [drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle) ] ]	(Eq3)

where n = [(E-SFN × 10240) + (SFN × 10) + subframe number]

As can be seen from Eq2 and Eq3, alt1 supports non-integer DRX cycles, therefore the solution can solve both the non-integer DRX cycle issue and the SFN wraparound issue.

Observation1: E-SFN can support non-integer (rational number) DRX cycle.

Alt. 2 H-SFN is reused to extend range of SFN.

H-SFN is already broadcasted in SIB1 for eDRX. The current DRX formula for long DRX cycle becomes [1][2]:

[(SFN + 1024* H-SFN) × 10+ subframe number] mod (drx-LongCycle) = drx-StartOffset	(Eq4).

Alt 2 reuses the existing H-SFN parameter which introduces less specification impact. However, there could still be a H-SFN wraparound issue, e.g., UE may be configured DRX when the value of H-SFN is near to the maximum value of H-SFN. Or it can be resolved via network implementation, i.e. the network needs to avoid configuring DRX if there may be H-SFN wraparound for the traffic, considering the H-SFN wraparound is a rare case.

Alt 2 introduces H-SFN into the DRX formulas, and it has low specification impacts.

Observation2: H-SFN has low impact on the current specifications.

The two alternatives are similar. Considering the specification impact, we think alt 2 with network implementation avoiding the H-SFN wraparound issue is feasible to address SFN wrap around issue.

Proposal1: To address SFN wrap around issue, RAN2 to discuss the introduction of H-SFN in the DRX formula.

- Conclusion

According to the analysis in section 2, it is proposed:

Observation1: E-SFN can support non-integer (rational number) DRX cycle.

Observation2: H-SFN has low impact on the current specifications.

Proposal1: To address SFN wrap around issue, RAN2 to discuss the introduction of H-SFN in the DRX formula.

- References

[1]. R2-2300324	Discussion on DRX Enhancements for XR Power Saving,	vivo, discussion

[2]. R2-2301372	C-DRX enhancements for XR	MediaTek Inc.	discussion